3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206549

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, Samsung

Title:	KI#2 conclusion proposal

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract: Principles for conclusion proposal in terms of KI#2

# 1. Introduction/Discussion

It is proposed for solution conclusion proposal for KI#2.

There are 3 solutions (sol#11, sol#14, sol#15) addressing key issue #2:

-	Sol#11 proposes to introduce new "group status event" monitoring event for NEF event exposure service, the NEF is responsible to transform the AF subscription to the "group status event" to subscription to monitoring event(s) for each UE group member: e.g. PDU Session status, Registration state, UE Presence in the Area Of Interest, the NEF then triggers the monitoring event for each individual UE using existing mechanism. The NEF collects the event reporting for each individual UE group member and sends aggregated report to AF as notification to "group status event" monitoring event.

-	Sol#14 assumes that AF treats "group status event" monitoring event as existing monitoring events such as UE Reachability, PDU Session Status and Number of UEs in an area, and then AF uses existing NEF event exposure service to subscribe to notification for those existing monitoring events for a single UE or a group UEs, so the AF later can associate the reports for the related existing monitoring events as "group status event" monitoring event reporting. However, this solution does not cover the case where the configuration of a monitoring event subscription is performed directly at NFs, i.e. not via UDM.

-	Sol#15 proposes that the AF can map "group status event" monitoring event to monitoring event for changes on the members of the group, and the NEF/UDM parameter provisioning service is enhanced to allow UDM/NEF to notify the changes for the 5G VN group membership to the given Notification Target Address. Other existing monitoring events can be supported using existing Nnef_EventExposure service.

Sol#14 and sol#15 rely on the AF capability that associates the reports for different monitoring events as "group status event" monitoring event reporting, this brings extra complexities on the AF design. To simplify AF, it is proposed to enhance the NEF function to support "group status event" monitoring event.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-74 V0.3.0.

# * * * * First change * * * *

## 8.2	Key Issue #2: Enhance group status event reporting

The following principle is proposed for normative work:

-	This solution applies for 5G VN groups as well as “non-5GVN” groups configured using O&M.

-	Information to reflect the changes of group status can comprise group member list stored in UDM changes. This is reported by the 5GC via notification mechanism.

-	The existing event notification (as described in Table 4.15.3.1-1 in TS 23.502 [3]) can be reused to collect the following events for a group of UEs (identified by an External Group ID) e.g. Registration state, UE Reachability, PDU Session status, UE Presence in the Area Of Interest, Number of UEs in an area.

-

# * * * * End of changes * * * *