3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206553

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#4, solution evaluations

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract: Principles for solution evaluations for key issue 4.

# 1. Introduction/Discussion

It is proposed to capture the solution evaluations for key issue 4.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-74 V0.3.0.

# * * * * First change * * * *

## 7.4	Key Issue #4: Multiple SMFs for VN group communication

There are 6 solutions (sol#3, sol#4, sol#5, sol#16, sol#19, and sol#20) for this KI. The main aspects includes support of SMF redundancy for reliability of the 5G VN group communication and which architectural enhancements, if any, are needed to enable the support of multiple SMFs to serve a 5G VN group.

The evaluation of key issue #4 separates into two aspects and uses the principles as below table:

Table 7.X-1: Evaluation of KI#4 related principles

| Objectives | Principles | Impacts | Pros/Cons | Solution |
| --- | --- | --- | --- | --- |
| Serving SMF selection | All the SMF serving the same 5G VN group belongs to a single SMF set | already supported | Pros: Cons: | sol#3, |
|  | the SMF serving the same 5G VN group can belong to different SMF sets |  |  | sol16 |
|  | All the SMF serving the same 5G VN group can belong to different SMF sets | NRF or GSMF: supports SMF discovery using serving VN ID. SMF: Update/Register serving VN ID to NRF GSMF. | Pros: Cons: | sol#4, sol#5, sol#19, sol#20, sol16 |
|  | Introduce the Single-SMF indicator as part of the group data in group subscription data so the SMF can be informed that whether this 5G VN group needs multiple SMFs | AF/NEF/UDM/UDR: support Single-SMF indicator when defining the 5G VN group. SMF: handle the Single-SMF indicator in the received 5G VN group data. | Pros: Cons: | sol#19 |
| Connection of PSA UPFs | N19 tunnels between UPFs controlled by SMFs in a single SMF set are set up. N6-based forwarding is applied between UPFs controlled by SMFs in different SMF sets. Local switching at I-UPF as instructed by A-SMF is applied. | SMF: Implementation dependent signaling can be used between SMF(s) that are part of a SMF set e.g. based on an implementation choice SMF(s) within the set can select one SMF to control the N19 configuration. UPF: Support of VPN solution towards N6, e.g. to act as Provider Edge router in IP/E VPN solution based on IETF RFCs. | Pros: Cons: | sol#3, sol#16 |
|  | N19 tunnels between UPFs controlled by SMFs in either the same SMF set or different SMF sets are set up. | SMF: two SMFs interact with each other to establish N19 tunnels directly or indirectly via GSMF/relay-SMF. UPF: two UPFs exchange data via N19 or via relay-UPF. | Pros: Cons: | sol#4, sol#5 sol#19, sol#20 |

To support SMF redundancy for reliability of the 5G VN group communication, the SMFs serving a 5G VN group can belong to the same SMF Set (e.g. solution #3) or different SMF Sets (e.g. solution #4, #5, #16, #19, #20)... For example, for a 5G VN group over a wide area (e.g. nationwide), the network operator can flexibly deploy one or more SMF Sets for each region within the service area of the 5G VN group.

To support traffic forwarding between the UPFs controlled by different SMFs serving a 5G VN group, either the N6-based forwarding or N19-based forwarding is used.

-	If N6-based forwarding is used, how to transmit traffic over N6 depends on configuration and is out of 5GC control. For example, solution #16 proposes to use Native N6 or N6 Overlay (IP/E-VPN) and it ensures the correct connectivity between UPFs via proper configuration between all UPFs supporting the DNN/S-NSSAI of a 5G VN group. However, the configuration workload will become heavy if the network operator deploys many UPFs for the DNN/S-NSSAI of a 5G VN group, and each time a new UPF is deployed or a UPF is released, it needs to re-configure the connectivity between UPFs. This could be alleviated by notifications from SMF to an AF responsible of the management of the N6 connectivity for a VN grouup

-	If N19-based forwarding is used, the 5GC is in charge of the N19 tunnel establishment and routing rules over the N19 tunnel. The 5GC can dynamically control the N19 connectivity for a newly joined UPF or released UPFs for a 5G VN group, as well as the routing rules over the N19 connectivity. For example, solution #4, #5 and #20 propose to enable SMF discovery and selection, exchange of N19 tunnel and UE address between SMFs, as well as the UPF N19 connectivity topology management.

Solutions (4, 5, 19 and 20) requiring inter (anchor) SMF communication induce a huge 3GPP specification effort as well as a lot of signaling:

- SMF: The SMF retrieves a peer SMF information via the GSMF (NRF) via two ways:

a)	the SMF subscribes to the GSMF (NRF) for the 5G VN group change events and receives a notification including other SMF's identifier information (but also N19 information and information on UE IP/MAC address routing) to the subscribed SMF when events occurs;

b)	the SMF can query the SMF information of a specific group member UE from the GSMF (NRF).

- Based on the information retrieved about peer SMF(s) the SMF needs to be able to:

-	Establish a 5GVNSM session with each other SMF(s) involved in the 5G VN group.

-	Use this 5GVNSM session with each other SMF(s) involved in the 5G VN group to establish N19 tunnels as instructed by the GSMF.

-	When a SMF no more serves any member of a 5G VN group, this SMF removes N19 tunnel(s) between UPFs controlled by itself and other SMFs. And then this SMF releases the 5GVNSM sessions with other SMF(s).

-	Receive instructions from GSMF about which N19 tunnels need to be established.

-	Configure UPF(s) for traffic routing for such N19 tunnels.

-	Handle UPF reports about unknown destination UE address.

-	If the SMF determines that an unknown destination UE address is not served by it, then the SMF needs to query or subscribe to the GSMF to obtain the target SMF serving this destination UE address and needs to contact the target SMF to obtain the N19 tunnel information of the UPF serving the destination UE address.

Conversely solution 3 coupled with solution 16 rely on existing mechanisms (N6 VPN(s), N16a communication between SMF and I-SMF with possibly some improvements) while supporting 5G VN groups spread over a huge country as I-SMF and local PSA UPF serving members of a 5G VN group may be spread all over a huge country while being controlled by more centralized SMF. Solution 16 ensures that, in this case, traffic may be routed over N6 between remote PSA UPF(s).

There is no advantage of using N19 instead of N6 data path between remote UPF(s): N6 VPN(s) and N19 GTP-u tunnels would likely use the same transport capabilities.

It has to be noted that the normative specification effort that would be required by Solutions (4, 5, 19 and 20) relying on inter (anchor) SMF communication would be equivalent to that of ETSUN (was 5 TU see S2-1813348) and is thus an order of magnitude higher than what is foreseen in the SA Approved SID: 0.25 TU

| WT#2.1 | 0.75 | 0.25 | No | self-contained |
| --- | --- | --- | --- | --- |

# * * * * End of changes * * * *