3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206561r04

Elbonia, Aug 17 – 26, 2022

Source:	Huawei, HiSilicon

Title:	KI#3: Evaluation for redundant traffic steering

Document for:	Approval

Agenda Item:	9.3

Work Item / Release:	FS_ATSSS_Ph3 / Rel-18

Abstract: This paper proposes evaluation on solutions for KI#3.

# 1. Introduction/Discussion

In this paper, evaluation part is proposed for the KI#3.

# 2. Text Proposal

It is proposed to capture the following changes into the TR 23.700-53.

# * * * * First change * * * *

## 7.x	Evaluation for KI #3: Support of redundant traffic steering

### 7.x.1	Considerations on Duplication Criteria

In order to support redundant traffic steering, there are solution #3.1, 3.2, 3.3, 3.4, 3.5 and 3.6 proposed to define the Redundant Steering Mode (RSM). If this new steering mode is applied by UE and UPF, the traffic is transmitted via 3GPP and non-3GPP accesses in a redundant way.

Considering redundant transmission is costly, which consumes double resources compared with normal transmission without duplication, duplication criteria may be defined to indicate when RSM should be initiated. In other words, predefined conditions or criteria, as proposed by quite a few solutions, such as solution 3.1, 3.2, 3.3 and 3.4, can be defined as trigger points for initiating redundant transmission. The duplication criteria may not always be needed as, in some cases, applications may always require redundant transmission without criteria for activation / deactivation.

One of the major benefits of redundant transmission is to reduce packet loss rate, therefore Packet loss rate should be taken as one of the criteria.Except packet loss rate, other criteria proposed by solution 3.3 and/or solution 3.4 to trigger redundant transmission, include Round Trip Time (RTT), jitter, percentage of duplicated traffic and load on an access network.

Another benefit of redundant transmission is to reduce the RTT. If the RTT via both accesses cannot satisfy the traffic requirement, the redundant transmission can reduce the RTT because the receiver always selects the packet that arrives faster. Note that if the RTT over 3GPP access is smaller than the RTT over non-3GPP access, this does not mean that all packets arrive faster via 3GPP access. Some percentage of packets will arrive faster via non-3GPP and since the receiver always selects the packet that arrives first, this means that the RTT due to redundant transmission will be smaller than the RTT over 3GPP access and than the RTT over non-3GPP access. However, if both accesses exceed the threshold, but the RTT differs a lot between the two accesses, duplication will not help much. A few packets may arrive earlier via the slow access (depending on the jitter) but the max delay may not decrease in any significant way. Redundant steering based on RTT threshold may thus not be better than using Lowest-Delay steering mode but consumes more resources.

Some of the solutions (#3.3 and #3.4) propose that the core network can decide how much of the traffic is duplicated on either of the accesses and provides this information in ATSSS rules to the UE and to the UPF. The amount of traffic that is duplicated on the secondary access is defined by a percentage value between 0% and 100%. This can be used to limit the number of duplicated packets to the ones seen as important for UE and UPF. The concrete benefits of duplication based on percentages regarding the given service requirements are however unclear. Furthermore, the data rate is already limited by the GBR and MBR values, and a duplication would maximally consume twice that amount. PCF thus already has control over the steering modes and bit rates.

Editor's note:	It is FFS whether traffic duplication based on percentage values is progressed in Rel-18.

# * * * * End of changes * * * *