SA WG2 Meeting #S2-152E	S2-2206586

16 – 20 August 2022, Electronic, Elbonia	(revision of S2-220xxxx)

Source:	ZTE

Title:	KI#2, solution#7 update

Document for:	Approval

Agenda Item:	9.6

Work Item / Release:	FS_DetNet / Rel-18

Abstract of the contribution: This paper proposes to update solution#7 on Detnet configuration mapping to the KI#2.

1. Proposal

It is proposed to update the solution#7 in the TR 23.700-46 "Study on 5GS DetNet interworking"

## 6.7	Solution #7 for Key Issue #2: Detnet configuration mapping to 5GS parameters

### 6.7.1	Introduction

This solution is for Key Issue #2, Provisioning DetNet configuration from the DetNet controller to 5GS.

-	Which parameters provided by the DetNet controller should be mapped into which 5G parameters.

-	How the 5GS finds the PDU Sessions corresponding to the given DetNet configuration.

-	What mechanisms are used in 5GS to configure the system according to the configuration provided by the DetNet controller.

This solution follow the assumption in clause 4.

-	Existing 3GPP routing mechanisms can be re-used for DetNet; no new routing function in the 3GPP system is to be defined.

-	The existing filtering mechanisms can be re-used in the UE and in the UPF to identify the traffic for QoS differentiation.

-	IP based DetNet traffic is carried in PDU Sessions of IP type.

-	The mapping functionality for DetNet is realized in the TSCTSF.

-	The solutions should reuse the functionality of the TSC framework defined in Release 17 where applicable.

-	The solutions shall not have any 5G AN and UE impacts

### 6.7.2	Functional Description

In this solution, the TSCTSF receives the configuration from Detnet controller and map it to service requirement and TSCAC.

Figure 6.7.2-1: Parameters mapping

The Detnet flow-related parameters is defined in RFC 9016 [4], and expressed in draft-ietf-detnet-yang [5].

The service requirement needed for PCF is defined in clause 5.6.2.7 (MediaComponent) and clause 5.6.2.8 (MediaSubComponent) of TS 29.514 [15]. The TSCAC is defined in TS 23.501 [12].

The table 6.7.2-1 show the mapping relation between Detnet configuration defined in clause 5 of RFC 9016 [4] and 5G QoS parameters (service requirement and TSCAC).

Table 6.7.2-1: Mapping between DetNet parameters and 5GS parameters

| DetNet parameters |  | 5G Qos parameters |  | Note |
| --- | --- | --- | --- | --- |
| DetNet flows attributes |  |  |  |  |
| clause 5.1 | DnFlowID |  | N/A |  |
| clause 5.2 | DnPayloadType |  | N/A | IP |
| clause 5.3 | DnFlowFormat |  | N/A | IP |
|  | DnFlowSpecification |  |  | Applied for both south and north interface |
|  | a. SourceIpAddress |  |  |  |
|  | b. DestinationIpAddress |  |  |  |
| clause 5.4 | c. IPv6FlowLabel | TS 29.514 [15] | MediaSubComponent |  |
|  | d. Dscp |  |  |  |
|  | e. Protocol |  |  |  |
|  | f. SourcePort |  |  |  |
|  | g. DestinationPort |  |  |  |
|  | h. IPSecSpi |  |  |  |
|  | Traffic Specification of the DetNet Flow |  |  |  |
|  | Interval |  | Periodicity in TSCAC |  |
| Clause 5.5 | MaxPacketsPerInterval |  | Used for calculate the maxTscBurstSize |  |
|  | MaxPayloadSize |  | Used for calculate the maxTscBurstSize |  |
|  | MinPayloadSize |  | ? |  |
|  | MinPacketsPerInterval |  | ? |  |
| Clause 5.6 | DnFlowEndpoints |  | ? | End to End |
| Clause 5.7 | DnFlowRank | TS 29.514 [15] | tscPrioLevel |  |
| Clause 5.8 | DnFlowStatus | TS 29.514 [15] | fStatus |  |
| DetNet flows requirement attributes |  |  |  |  |
| Clause 5.9 | DnFlowRequirements |  |  |  |
|  | MinBandwidth |  | GFBR |  |
|  | MaxLatency |  | Use this parameters to determine the 5GS node latency Note 3 | End to End |
|  | MaxLatencyVariation |  | ? |  |
|  | MaxLoss | TS 29.514 [15] | maxPacketLossRateDl maxPacketLossRateUl |  |
|  | MaxConsecutiveLossTolerance | TS 23.501 [12] | Survival time |  |
|  | MaxMisordering |  | ? |  |
| Clause 5.10 | DnFlowBiDir |  | ? |  |
| NOTE 1: "?" show how to map is unclear NOTE 2: "End to End" means this value is not the parameters for Detnet node NOTE 3: TSCTSF use MaxLatency and configuration to determine the 5GS detnet node latency. |  |  |  |  |

The TSCTSF uses the incoming and outgoing interfaces in the draft-ietf-detnet-yang [5] to determine the related PDU Session (s) and Flow direction. If the incoming and outgoing interface are related the UE side, the TSCTSF separate it to two PDU Sessions, one for UL and one for DL.

NOTE:	For the IP type traffic, the incoming interface is optional. If there is no incoming interface for UL traffic, the TSCTSF determine the PDU session according to configuration, topology information learned from KI#1 and source IP address in the DnFlowSpecification.

Editor's note:	How the map the end to end Detnet parameters to 5GS parameters is FFS.

Editor's note:	Which parameters in the table are carried in draft-ietf-detnet-yang [5] to Detnet node is FFS.

### 6.7.3	Procedures

The procedure in clauses 4.15.6 and 4.15.6.6a of TS 23.502 [13] are re-used with the following enhancement:

-	The Detnet controller send the Detnet configuration to TSCTSF.

NOTE:	In this solution, there is no NEF between TSCTSF and Detnet controller..

### 6.7.4	Impacts on existing entities and interfaces

TSCTSF:

-	mapping DetNet flow configuration to 5GS QoS requirements and TSC parameters.

Editor's note:	Additional impacts are FFS.