3GPP TSG-SA WG2 Meeting #152-e	S2-2206662r09

# E-Meeting, 17th – 26th Aug 2022

Title:	[draft]LS Out on RAN dependency of FS_eNS_Ph3

Response to:	None

Release:	Rel-18

Work Item:	FS_eNS_Ph3

Source:	SA2

To:	RAN2, RAN3

Cc:	None

Contact Person:

Name:	Zhu Jinguo

E-mail Address:	zhu.jinguo@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2207524

1. Overall Description:

SA2 starts the Rel-18 study FS_eNS_Ph3 study in TR 23.700-41(link). This study includes 6 key issues and associated solutions so far. SA2 is now evaluating all solutions and try to conclude the study at Oct meeting. Since some solutions have RAN dependency, SA2 would like to ask RAN2 and RAN3 on their views on the following aspects:

For Key Issue #3:  Network Slice Area of Service for services not mapping to existing TAs boundaries, and Temporary network slices, SA2 has following questions:

2.	Whether NG-RAN can broadcast one or more Secondary TAIs (up to a number RAN2 agrees, we note that for NTN is already possible to broadcast TWO TACs) via an updated SIB or new SIB, and report them to the CN and between gNBs as per existing Tracking Area related information exchange procedures but with indication they are secondary. The additional TAIs are associated with specific S-NSSAI(s) like the existing TAs and will be treated by UEs supporting secondary TAs as a normal Tracking area from RM standpoint (as described in solution#9)

3.	Whether the NG-RAN can be configured with a slice availability on a per-cell basis and

  - inform AMF and other gNBs in NGAP messages (as described in solution#11 and others)

  - Whether in Constrained Service Area the network slice is still supported but since no dedicated resources are allocated for the network slice the SLA of the network slice is not guaranteed.(as described in S2-2206563).

4.	The NG-RAN receives in solution 29 (but conceivably this would be needed for similar solutions) the partially allowed S-NSSAIs in addition to the Allowed NSSAI. Can the NG-RAN in principle trigger handover procedure to a supporting TAI of the partially allowed S-NSSAIs when it is possible to do so? this can happen while in connected mode or when the UE is engaged in transition from Idle to connected mode. The reason is to enable the support of the maximum number of S-NSSAIs in the Allowed and partly allowed S-NSSAIs lists.

5.	Whether the NG-RAN can support extended Mobility Restriction List (that is extended with per S-NSSAI and per cell restrictions (as described in S2-2207524)

2. Actions:

To RAN2 and RAN3 group.

ACTION: 	SA2 kindly asks RAN2 and RAN3 to provide feedback on the aspects above before SA2#153e Oct meeting

3. Date of Next SA2 Meetings:

3GPP TSG SA2#153e		10-14,Oct, 2022	E-Meeting

3GPP TSG SA2#154e		14-18, Nov,2022	E-Meeting