3GPP SA WG2 Meeting #152E	S2-2206679r07

e-meeting, August 17-26, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#4 Evaluation and Conclusion

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_PH3/ Rel-18

Abstract of the contribution: This paper proposes analysis and conclusion for KI #4 in 23.700-81.

Discussion

This contribution proposes Overall Evaluation and Conclusions for aspects of KI #4. Seven KI#4 Solutions are proposed, which may be grouped as follows:

- DCCF Relocation (solutions 12 and 44) – to support e.g. when UE moves outside the serving area of a DCCF.

- Managing Impact of storing data in NFp during muting (Solutions 41 and 45) – to support NFp when it can no longer store / buffer notifications during event muting.

- Storage and Retrieval of Trained ML models to/from ADRF (Solutions 42 and 43) – to support model storage in ADRF, which currently stores Data and Analytics.

- ADRF / NWDAF Data Storage Management (Solution 46) – Manages storage of data in ADRF/NWDAF according to NFc preference or provisioned Policy.

- Sharing models between NWDAFs – Allows NWDAFs to directly share models.

Group 3 - Storage and Retrieval of Trained ML models is pending possible resolution of ENs in Solution 42, and similarly Group 5 (Solution 47) is pending resolution of ENs. For the other groups, solution comparisons (where applicable), observations and KI conclusions are proposed.

Proposal

It is proposed to add the following to TR 23.700-81.

*** Start of changes (all new text) ***

8.x	Conclusion for KI#4

- For DCCF Relocation, Data consumer or central DCCF selects the target DCCF. MFAF is assumed co-located with DCCF, therefore no separate relocation procedure defined.

Editor’s note:	whether target DCCF needs to get the data subscription information from the source DCCF is FFS.

- For Managing Impact of storing data in NFp during muting, Solutions #41 and #45 are adopted for normative work.

- For ADRF / NWDAF Data Storage Management, ADRF is configured with operator policies for data storage as defined in Solution#46.

- For storing ML models in ADRF, MTLF can store ML model in ADRF based on MTLF policy.

Editor’s Note: It is for FFS whether Model provisioning service shall only be provided by MTLF.

Editor’s Note: Detailed procedures, service operations and corresponding parameters on how MTLF stores the ML model in ADRF are FFS.

*** End of changes ***