SA WG2 Meeting #152E eMeeting	S2-2206695r01

Elbonia, Aug. 17 – 26, 2022

Source:	Oracle, Toyota, NTT DOCOMO, OPPO, Huawei, HiSilicon

Title:	TR 23.700-80: KI#5 – Solution#10 Update for BDT Extensions

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys / Rel-18

Abstract of the contribution: This contribution is to address the outstanding ENs in the Solution#10 BDT Extensions for KI#5. It also provides some update to the procedure for applying BDT policy to an existing PDU session.

# Background

The intent of this paper is to address the two outstanding ENs in Solution#10 to support KI#5.

- The first EN is about whether it is feasible for the BDT solution to be extended to support packet delay variance parameter

Editor’s note: Whether and How to monitor and report the Variance of Packet Delay for UL/DL of all UEs is FFS.

Observation: The Variance of Packet Delay for UL/DL of all UEs can be used to assist FL operations, e.g., FL AF may be able to choose a time window with smaller delay variance of all UEs to guarantee the efficiency of FL model training. Therefore, it is necessary to enhance the BDT policy for the support of the Variance of Packet Delay for UL/DL of all UEs in the FL group. As proposed in clause 6.10.2.1 of Solution #10, the Network Performance analytics of NWDAF could be updated to support the new analytics outputs (including the variance of the packet delay) to enhance the BDT policy. As described in clause 5.33.3 of TS 23.501 [3], the NWDAF can obtain the QoS monitoring result (including packet delay measurement) for UL/DL of per UE in the FL group, so the NWDAF may derive packet delay statistics or prediction for UL/DL of per UE in the FL group. Furthermore, it is not difficult for the NWDAF to derive the variance of packet delay statistics or prediction for UL/DL of all UEs in the FL group.

Conclusion: By enhancing the Network Performance analytics of NWDAF, the monitoring and the reporting of the Variance of Packet Delay of UL/DL of all UEs can be supported.

- The second EN is about how the proposed BDT extensions can support also general AI/ML traffic transmission.

Editor's note: This solution is not only suitable for non-real-time AI/ML traffic transmission, but also suitable for general AI/ML traffic transmission. How this solution can be extended to support general AI/ML traffic transmission is FFS.

Observation: When interpreting the meaning of the “general AI/ML traffic” in the context of this study, the 3GPP TS 22.261 which defines the stage-1 requirements of this study is referred. In 3GPP TS 22.261, clause 6.40.1, three types of Application AI/ML operations were considered when defining the stage-1 requirements, and they are: - AI/ML operation splitting between AI/ML endpoints - AI/ML model/data distribution and sharing over 5G system - Distributed/Federated Learning over 5G system Among the above three operation types, type b) above could involve a specific UE or a group of UEs, where as type c) above will be target to a group of UEs. When the application AI/ML transmits intermediate data, local training data, inference results or model performance data via the support of 5GS for a specific UE, the AF could leverage the Nnef_AFsessionWithQoS_Create request as described in TS 23.502, clause 5.15.6.6 to include specific QoS parameters for the AF session for the given UE. Likewise, if the AF needs to update the QoS request for the AF session for a specific UE, AF could leverage Nnef_AFsessionWithQoS_Update request as described in TS 23.502, clause 5.15.6.6a to specify the corresponding Flow Description(s) and QoS parameter(s) for the QoS update. However, for the case of operation b) and c) which involves a group of UEs that could consume a high volume of network resources, then, it is very inefficient to have the AF to provision individual transport policy for individual UE one-by-one. It is necessary that, the AF needs to coordinate with 5GS in advance to reserve/plan for such kind of transport. As the existing BDT procedure as described in TS 23.502, clause 4.16.7 is designed to handle the group signalling communication and policy configuration and reconfiguration, therefore, the existing BDT procedures Nnef_BDTPNegotiation_Create/Update with the QoS extension as proposed by this Solution#10 is the most efficient and more appropriate mechanism to support such planned and event driven AF session that involves a group of UEs when it is compared to other existing AF Session mechanisms.

Conclusion: When the application AI/ML transmits intermediate data, local training data, inference results or model performance data via the support of 5GS for a specific UE, the AF could leverage the Nnef_AFsessionWithQoS_Create/Update request for the AF session for the given UE. When the application AI/ML operations involves a group of UEs that could consume a high volume of network resources, the existing BDT procedures Nnef_BDTPNegotiation_Create/Update as described in TS 23.502, clause 4.16.7 with the extension as proposed by this Solution#10 is the most efficient and more appropriate mechanism to support the planned and event driven AF session for application AI/ML operation types b) and c) above when it is compared to other existing AF Session mechanisms. Furthermore, when applying the BDT policy to an existing PDU session (e.g. BDT policy update to an existing PDU session which may or may not be part of the existing BDT’s PDU sessions), then either Npcf_PolicyAuthorization_Create/Update or Nnef_ApplyPolicy_Create/Update, respectively can be used to update the BDT policy for an existing PDU session.

In addition to address the ENs, this PCR also provides some update to the clarify the existing BDT procedures on how to apply, update or remove BDT policy for a PDU session to be part of the BDT managed by the AF.

# Proposals

***** Start of Change *****

## 6.10	Solution #10: Background Data Transfer Extensions to support planned and event driven AI/ML traffic transport

### 6.10.1	Description

This KI is to address KI#5: 5GC Enhancement to enable Application AI/ML Traffic Transport.

In 3GPP TS 22.261, clause 6.40.1, three types of Application AI/ML operations were considered when defining the stage-1 requirements, and they are:

- AI/ML operation splitting between AI/ML endpoints

- AI/ML model/data distribution and sharing over 5G system

- Distributed/Federated Learning over 5G system

Among the above three operation types, type b) above could involve a specific UE or a group of UEs, where as type c) above will be target to a group of UEs that would consume a high volume of network resources, it is very inefficient to have the AF to provision and to update individual transport policy for UE one-by-one when supporting the group transmission.  It would be beneficial if the AF can request prior to the transmission whether there will be QoS resources to support the 3 operations above and negotiated with 5GS when the transmission can take place.,

Network capacity planning is a major challenge for any mobile operator. The AI/ML data transfer can be non GBR type  (e.g. "please start the transmission for this model at midnight") or  GBR type that requires some resource reservation to aim to achieve the transmission requirements. (e.g. "the AF will start the transmission for this model 10 minutes later and  aims to complete the transmission in 1 second, this will mean that some 5QI, GBR or PDB requirements are set for this type of trafic"). In order to enable the MNOs to assist ASPs to transport their Application AI/ML traffic in a more controlled time-table to ensure the availability of network resources, this solution proposes to have a prior negotiation between the AF and the 5GS of the type of traffic that will be transmitted as  existing Background Data Transfer (BDT) mechanism to support planned and event driven application AI/ML traffic.

NOTE 1:	Occurrences of the name BDT throughout this solution (e.g. procedure names) may be replaced by a more suitable name (e.g. Application AI/ML Data Transfer (AAMDT)), which may better reflect the enhancements proposed by this solution. This is to be discussed before conclusions are reached.

NOTE 2:	The updates to the TSs indicated in this paper are for information purpose only.

As described in clause 6.1.2.4 of TS 23.503 [5], today BDT supports two different ways of applying data transfer policies, One is for a future PDU session and another one is for an existing PDU session. Even though the initial intent of BDT policy negotiation is to support non-time-critical data during off-hour, given the ability of BDT of allowing negotiation in advance of the desired data transfer time-table and location, data volume and data transfer policy (e.g. bit rate for one UE or a group of UEs, charging keys, etc.), including the ability to update it based on changing network conditions (e.g. triggered by NWDAF notifications) one possibility is to extend  the following existing BDT mechanisms, see NOTE 1 to support the  Application AI/ML group data transfer (e.g. FL data transfer) that meets the operational behaviours as described in clause 6.40.2 of TS 22.261 [2]:

-	the in-time data transfer,

-	the event driven (e.g. responding to the threshold reporting of "Network Performance" from NWDAF for the area of interest and time window, UE's location etc.) data transfer, and

-	the dynamic policy adaptation data transfer

According to clause 7.10 of TS 22.261 [2], there are KPIs defined for the QoS performance for the data transfer to support split AI/ML inference, AI/ML model download and FL learning. In order to leverage the existing BDT mechanisms as described above to support the planned and event driven Application AI/ML traffic, the BDT Transfer Policy needs to be extended to support additional QoS parameters beyond the traffic bit rate and volume that have been defined. The following additional QoS parameters are proposed by this solution to be added to the BDT Transfer Policy to support Application AI/ML traffic:

-	Packet Delay Budget for UL/DL per UE;

-	Packet Error Rate for UL/DL per UE;

-	Packet Loss Rate for UL/DL per UE;

-	Guaranteed Bitrate for UL/DL per UE.

Editor's note:	Whether and How to monitor and report the Variance of Packet Delay for UL/DL of all UEs is FFS.

NOTE 3:	The QoS parameters for uplink and downlink may be asymmetrical based on the KPIs that were defined in clause 7.10 of TS 22.261 [2].

In addition, this solution also proposes to merge with Solution#14 which addresses minimum guarantee of the QoS performance among the selected FL members when they are participating in the FL operation. When supporting the FL operation as described in TR 22.874 [8], [P.R.7.4-002] and related description, one fundamental important consideration is to eliminate bias - allowing diverse users and devices to participate and bring to the learning task diversity of input data, as the users will have different attributes. It is important not to merely focus on the 'best performing devices' in the federation and drop the rest. It may increase the performance in terms of time to iterate the synchronous federated learning task to drop stragglers, but this will reduce the diversity of the data set and introduce bias. This phenomenon as described is referred as "flock".

It is important to strike a right balance between performance and diversity when proceeding with the FL operation. The AF may not necessarily select the best performance UEs, but certain minimum performance should be guaranteed for the selected UEs in order to sustain the effectiveness of the FL operation. 5G system should avoid risking selected UE performance for participating in the federated learning process that are caused by an inappropriate communication resource allocation. The mobile network, at least, should guarantee the minimum required QoS for the selected UEs to ensure the adequate performance for the FL operation. In the context of this solution, Flock QoS refers to a set of QoS parameters that specifies the minimum QoS performance for a group of UEs.

Also as defined in the clause 6.1.2.4 of TS 23.503 [5], the PCF may determine the candidate list of BDT policies based on the analytics on "Network Performance" from NWDAF. In order to assist PCF to determine these additional QoS parameters to the BDT policy, it is necessary to enhance the "Network Performance" analytics by NWDAF. So this solution also proposes to add some new output parameters into the Network Performance analytics as defined in the Table 6.6.3-1 and Table 6.6.3-2 of TS 23.288 [6], such as:

-	Average packet delay;

-	Average packet error rate;

-	Average packet loss rate;

-	Average bitrate.

Further details of the proposal are described in the following Solutions clause.

### 6.10.2	Solutions

The intent of this solution is to extend the existing BDT data transfer mechanism to support planned and event driven Application AI/ML data transfer.

The solution describes the update to the BDT transfer policy in clause 6.1.2.4 of TS 23.503 [5]. This update includes added QoS parameters support and potentially update the Network Performance analytics by NWDAF in TS 23.288 [6] to support the additional QoS parameters. It also updates the existing BDT negotiation procedures for future PDU session as defined in clause 4.16.7.2 of TS 23.502 [4] to include the additional QoS parameters support, and in addition, it specifies how AIML AF discovers its serving NEF as well as the need for the NEF to authenticate/authorize the AIML AF. Finally, this solution presents the procedures on how BDT can be applied to the existing or future PDU session.

In addition, this solution also proposes to merge with Solution#14 which addresses minimum guarantee of the QoS performance among the selected FL members when they are participating in the FL operation.

#### 6.10.2.1	Additional QoS Parameters for BDT Transfer Policy to support Application AI/ML data transfer

The current BDT data transfer policy does not support the QoS parameters other than the maximum aggregated bitrate. In order to meet the KPIs that have been defined in TS 22.261 [2], clause 7.10 to support the QoS performance for the data transfer for various Application AI/ML traffic, this solution proposed to enhance the BDT transfer policy for the support of the following QoS parameters that have been indicated in the KPIs:

-	Packet Delay Budget for UL/DL per UE

-	Packet Error Rate for UL/DL per UE

-	Packet Loss Rate for UL/DL per UE;

-	Guaranteed Bitrate for UL/DL per UE;

This solution proposes to update the BDT policy descriptions in clause 6.1.2.4 of TS 23.503 [5] and the Network Performance analytics by NWDAF in TS 23.288 [6] to include the support of the above QoS parameters. The proposed update to clause 6.1.2.4 of TS 23.503 [5] and the Network Performance analytics by NWDAF in TS 23.288 [6] is shown in "bold italic" below.

6.1.2.4 Negotiation for future background data transfer (TS 23.503 [5]) The AF may contact the PCF via the NEF (and Npcf_BDTPolicyControl_Create service operation) to request a time window and related conditions for future background data transfer (BDT). NOTE 1: The NEF may contact any PCF in the operator network. The AF request shall contain an ASP identifier, the volume of data to be transferred per UE, the expected amount of UEs, the desired time window, the External Group Identifier and optionally, Network Area Information, MAC address or IP 3-tuple to identify the Application server, request for notification. The AF provides as Network Area Information either a geographical area (e.g. a civic address or shapes), or an area of interest that includes a list of TAs or list of NG-RAN nodes and/or a list of cell identifiers. When the AF provides a geographical area, then the NEF maps it based on local configuration into of a short list of TAs and/or NG-RAN nodes and/or cells identifiers that is provided to the PCF. In addition the AF may provide latency and reliability requirements for UL/DL per UE. NOTE 2: The latency and reliability requirements provided by the AF should be explicit enough such that the PCF will be able to map them to their corresponding Packet Delay Budget (PDB) for UL/DL per UE, Packet Error Rate(PER) for UL/DL per UE, for UL/DL per UE. The PCF will ensure that network resources area available right before the time of the BDT actual data transmission in the following two ways: 1. The PCF may interact with the NWDAF as described in TS 23.502 [4] 4.16.7.3, and upon receiving a notification about degradation of the network, may re-negotiate the background data transfer policy with the AF/NEF if needed. The subscription with the NWDAF can be such that the PCF will have visibility into the available network resources right until the moment of actual BDT transmission. The NEF may map the ASP identifier based on local configuration to a DNN and S-NSSAI that is in addition provided to the PCF. The MAC address or IP 3-tuple to identify the Application server may be provided by the AF or may be locally configured at the PCF and it is used for the generation of a URSP rule for the application as well as a PCC rule for the application traffic. The request for notification is an indication that the ASP accepts that the BDT policy can be re-negotiated using the BDT warning notification procedure described in clause 4.16.7.3 of TS 23.502 [4]. NOTE 3: A 3rd party application server is typically not able to provide any specific network area information and if so, the AF request is for the whole operator network. The PCF shall first retrieve all existing BDT policies stored for any ASP from the UDR. The PCF may retrieve analytics on "Network Performance" from NWDAF following the procedure and services described in TS 23.288 [6]. Afterwards, the PCF shall determine, based on the information provided by the AF, the analytics on "Network Performance" if available and other available information (e.g. network policy and existing BDT policies) one or more BDT policies. The PCF may be configured to map the ASP identifier to a target DNN and S-NSSAI if the NEF did not provide the DNN, S-NSSAI to the PCF. A BDT policy consists of a recommended time window for the background data transfer, a reference to a charging rate for this time window, optionally a maximum aggregated bitrate (indicating that the charging according to the referenced charging rate is only applicable for the aggregated traffic of all involved UEs that stays below this value) and optionally QoS parameters of Packet Delay Budget (PDB) for UL/DL per UE, Packet Error Rate(PER) for UL/DL per UE, Packet Loss Rate for UL/DL per UE, Guaranteed Bitrate for UL/DL per UE etc. to be equally used by any UE in the BDT group, and Variance of Packet Delay for UL/DL of all UEs in the BDT group. Finally, the PCF shall provide the candidate list of BDT policies to the AF via NEF together with the Background Data Transfer Reference ID. If the AF received more than one BDT policy, the AF shall select one of them and inform the PCF about the selected BDT policy. NOTE 4: The maximum aggregated bitrate (optionally provided in a BDT policy) is not enforced in the network. The operator may apply offline CDRs processing (e.g. combining the accounted volume of the involved UEs for the time window) to determine whether the maximum aggregated bitrate for the set of UEs was exceeded by the ASP and charge the excess traffic differently. NOTE 5: It is assumed that the 3rd party application server is configured to understand the reference to a charging rate based on the agreement with the operator. : : :

6.6.3 Output Analytics (TS 23.288 [6]) The NWDAF shall be able to provide both statistics and predictions on Network Performance. Network performance statistics are defined in Table 6.6.3-1. Table 6.6.3-1: Network performance statistics Information; Description. List of network performance information (1..max); Observed statistics during the Analytics target period. > Area subset; List of TAs or Cell IDs within the requested area of interest as defined in clause 6.6.1. > Analytics target period subset; Time window within the requested Analytics target period as defined in clause 6.6.1.. > gNB status information (NOTE 1); Average ratio of gNBs that have been up and running during the entire Analytics target period in the area subset. > gNB resource usage (NOTE 1); Average usage of assigned resources (CPU, memory, disk). > Number of UEs (NOTE 1); Average number of UEs observed in the area subset. > Communication performance (NOTE 1); Average ratio of successful setup of PDU Sessions. > Mobility performance (NOTE 1); Average ratio of successful handover. > UE group ID or UE ID; Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [3] clause 5.9.7, or SUPI.. >> Packet delay for UL/DL; Average of the packet delay and variance of the packet delay (in the case of a UE group) for UL/DL. >> Packet error rate for UL/DL; Average of the packet error rate and variance of the packet error rate (in the case of a UE group) for UL/DL. >> Packet loss rate for UL/DL; Average of the packet loss rate and variance of the packet loss rate (in the case of a UE group) for UL/DL. >> Guaranteed Bitrate for UL/DL; Average of the guaranteed bitrate and variance of the guaranteed bitrate (in the case of a UE group) for UL/DL. NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset".. Network performance predictions are defined in Table 6.6.3-2. Table 6.6.3-2: Network performance predictions Information; Description. List of network performance information (1..max); Predicted analytics during the Analytics target period. > Area subset; List of TAs or Cell IDs within the requested area of interest as defined in clause 6.6.1. > Analytics target period subset; Time window within the requested Analytics target period as defined in clause 6.6.1.. > gNB status information (NOTE 1); Average ratio of gNBs that will be up and running during the entire Analytics target period in the area subset. > gNB resource usage (NOTE 1); Average usage of assigned resources (CPU, memory, disk) (average, peak). > Number of UEs (NOTE 1); Average number of UEs predicted in the area subset. > Communication performance (NOTE 1); Average ratio of successful setup of PDU Sessions. > Mobility performance (NOTE 1); Average ratio of successful handover. > UE group ID or UE ID; Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [3] clause 5.9.7, or SUPI.. >> Packet delay for UL/DL; Average of the packet delay and variance of the packet delay (in the case of a UE group) for UL/DL. >> Packet error rate for UL/DL; Average of the packet error rate and variance of the packet error rate (in the case of a UE group) for UL/DL. >> Packet loss rate for UL/DL; Average of the packet loss rate and variance of the packet loss rate (in the case of a UE group) for UL/DL. >> Guaranteed Bitrate for UL/DL; Average of the guaranteed bitrate and variance of the guaranteed bitrate (in the case of a UE group) for UL/DL. > Confidence; Confidence of this prediction. NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset".. ….

#### 6.10.2.2	BDT Negotiation for future PDU session to support Application AI/ML data transfer

This clause presents further clarifications on how to leverage the existing BDT procedures for negotiation of future background data transfer as specified in clause 4.16.7.2 of TS 23.502 [4] to support the Application AI/ML data transfer.

Prior to the establishment of PDU session to support the BDT within a specified time-window, AF initiates requests to its serving NEF to (re)negotiate BDT policy (as described in clause 6.1.2.4 of TS 23.503 [5]) that might have been pre-provisioned or earlier negotiated and stored in the UDR.

Figure 6.10.2.2-1: BDT Policy Negotiation for future PDU session to support Application AI/ML data transfer

The procedures as shown in Figure 6.10.2.2-1 above is based on the procedures specified in TS 23.502 [4], clause 4.16.7.2 to support negotiation for future background data transfer. The following descriptions focus on the changes to the existing procedures:

Prior to the transport of the Application AI/ML data within the specified time-window, the AF negotiates with the 5G Core for the BDT policies that apply to its given Application AI/ML data transfer corresponding to the authorized 3rd party (i.e. AF), the AF is required to discover its serving NEF, if it has not done so, using the mechanism as described in clause 6.3.14 of TS 23.501 [3].

In step 1a, in addition to the list of parameters for Nnef_BDTNegotiation_Create Request from AF as described in clause 4.16.7.2 of TS 23.502 [4], the AI/ML AF should include the added latency and reliability requirements as described in clause 6.10.2.1.

Furthermore, if "flock" scenario applies to the group of selected FL UEs who are to be participated in this BDT session, the BDT policy automatically takes into account the Flock QoS policy into consideration by specifying the minimum guaranteed QoS requirements, via proper QoS attributes, e.g. Packet Delay Budget.

NOTE 1:	Flock QoS is determined by an AF based on AF's internal logic (policy). For example, the AF can determine the flock QoS based on the UEs and the network input data and the FL trigger conditions.

In step 1b, the NEF may authenticate the AF and authorize the BDT request from the AF. If the given AF is not authenticated and authorized, the NEF will reject AF's request through the Nnef_BDTNegotiation_Create Response, and the following steps are skipped.

In step 2, in addition to the list of parameters for Npcf_BDTPolicyControl_Create Request from AF as described in clause 4.16.7.2 of TS 23.502 [4], the NEF should include the added latency and reliability requirements exactly as received from the AF.

In step 4, PDB and PER etc. QoS parameters may also be provided by UDR as part of the BDT policies to the PCF. This may happen if the PCF (or another PCF) had received from the AF/NEF in a previous BDT negotiation, latency and reliability requirements, mapped it to the corresponding PDB and PER etc. QoS parameters, and stored it in the UDR.

In Steps 5, 6 the PCF maps the additional latency and reliability requirements to PDB and PER etc. QoS parameters. As described in current step 5, the PCF may interact with the NWDAF and request analytic on the Network Performance analytics information for the Desired time window and the Network Area Information as defined in TS 23.288 [6]. the PCF may determine the PDR, PER etc. QoS parameters in the BDT policy based on the Network Performance analytics from NWDAF.

After the BDT policy negotiation and before the PDU session is established to transport the Application AI/ML data, if the AF decides to select an alternative BDT policy (e.g. data rate reduction, relaxing delay constraints of planned and event driven traffic etc.) which may include the Flock QoS policy consideration with the minimum guaranteed QoS requirements as the result of certain AI/ML operation decision, the AF may trigger step 8 to update the corresponding PCF via the support of NEF for the new selected BDT policy.

NOTE 2:	It is not in the scope of this solution to specify the event or consideration that triggers the AF decision to update the BDT policy.

In step 12, the additional QoS parameters may be added to the UDR by the PCF as part of the updated BDT policies.

#### 6.10.2.3	BDT Activation of BDT Policy or Updating/Removing BDT Policy for a given Application AI/ML data transfer to a PDU Session

This clause describes two ways to apply the negotiated BDT policy for a given Application AI/ML data transfer to the PDU Session(s) for given UE(s) - i.e. (a) activating the negotiated BDT policy as described in clause 6.10.2.2 above to a PDU session; and (b) adding an PDU session of the target UE to the BDT by applying the BDT policy.

For the activation of the previously negotiated BDT policy to a PDU session to support the Application AI/ML data transfer, before the start of the BDT transfer window, the AF triggers the UDR via NEF by actuating Nnef_ApplyPolicy_Create/Update service, if the AF is untrusted, to notify the PCF via Npcf_PolicyAuthorization_Create/Update as described in clause 4.15.6.8 of TS 23.502 [4]; otherwise, AF actuates Npcf_PolicyAuthorization_Create/Update directly to PCF. The AF discover the PCF (may be via NEF) via BSF by invoking Nbsf_Management_Discovery.  In both cases, the PCF will then leverage the negotiated BDT policy to generate the PCC rules to support the establishment of the future PDU session(s). For further details on how to apply the previously negotiated BDT policy can be referred to clause 6.1.2.4 of TS 23.503 [5] and clause 4.15.6.8 of TS 23.502 [4].

If the PDU session has been established and the AF would like to apply/update/delete the BDT policy to such existing PDU session for individual UE to support the Application AI/ML data transfer, then the AF invokes the Npcf_PolicyAuthorization_Create/Update service directly to PCF (via the N5 interface). The AF may discover the PCF via BSF by invoking Nbsf_Management_Discovery.   The AF also provides the Background Data Transfer Reference ID together with the AF session information to the PCF that serves the PDU session. The PCF authorizes the AF request. If the PCF determines that the AF can't be authorized, it rejects the AF request. Once the PCF authorizes the AF request, and if the AF request is to apply or to update the PDU session with the BDT policy, the PCF retrieves the corresponding BDT policy from UDR to derives the PCC rule for the BDT according to the transfer policy which may include the Flock QoS policy consideration with the minimum guaranteed QoS requirements. In addition, the AF may specify the individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 different than the BDT policy.  The PCF then updates the SMF with corresponding new PCC rule(s) with PCF initiated SM Policy Association Modification procedures as described in TS 23.502 [4], clause 4.16.5.2. Further details on how AF applies the Background Data Transfer Policy to an existing session are provided in clause 6.1.2.4 of TS 23.503 [5].

If the AF would like to remove the BDT policy for a PDU session of a given UE, AF invokes the Npcf_PolicyAuthorization_Delete service directly to PCF. The AF also provides the Background Data Transfer Reference ID together with the AF session information to the PCF that serves the PDU session. The PCF authorizes the AF request as described above before removing the PDU session from the BDT

Figure 6.10.2.3-1: Applying/Updating/Removing BDT policy to an PDU session to support Application AI/ML data transfer

1.	AF decides to apply/update/delete the BDT policy of a given application AI/ML data transfer for a PDU session, then the AF will, at the time the data transfer is about to start, provide, for the target UE, the Background Data Transfer Reference ID together with the AF session information to the PCF that serves the PDU session (via the N5 interface). Note that AF may need to discover the UE’s serving PCF via NEF with the support of BSF.  This Npcf_PolicyAuthorization_Create/Update/Delete service as described in clause 5.2.5.3.2 of TS 23.502 [4] is to authorise an AF request and to create policies as requested by the authorized AF for the PDU Session to which the AF session is bound. More specifically, the added latency and reliability requirements as described in clause 6.10.2.1 should be included in the AI/ML AF request.

NOTE:	It is not the scope of this solution to specify the event(s) or consideration(s) that triggers the AF decision to apply the BDT policy to an existing PDU session.

2.	The PCF authorizes the AF request.

3.	If the PCF determines that the AF can't be authorized, it rejects the AF request by including the rejection in Npcf_PolicyAuthorization_Create/Update/Delete response. All the subsequent steps below are skipped.

4-5. Once the PCF authorizes the AF request, if the AF request is to create or to update the BDT policy for the given PDU session, the PCF retrieves the corresponding BDT policy from UDR to derives the PCC rule(s) for the BDT according to the transfer policy that has been pre-provisioned which may include the Flock QoS policy consideration with the minimum QoS requirements.  In addition, the AF may specify an individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 to replace the BDT policy for the given PDU session.

6.	The PCF updates the SMF with corresponding new PCC rule(s) with PCF initiated SM Policy Association Modification procedures as described in clause 4.16.5.2 of TS 23.502 [4].

Further details on how PCC rules are applied to either future or an existing PDU session are provided in clause 6.1.2.4 of TS 23.503 [5].

***** Next Change *****

### 6.10.3	Impacts on services, entities and interfaces

AIML AF shall be able to provide latency and reliability requirements in BDT negotiation request.

AIML AF determines and provisions the flock QoS for the federated group i.e. minimum QoS requirement for the group of selected UEs who participate in the given FL operation.

NWDAF shall be able to provide Network Performance analytics including additional parameters such as average packet delay, average packet error rate, average packet loss rate, average bitrate, during the BDT policy negotiation procedure.

PCF shall be able to consider such information from the AIML AF and/or Network Performance analytics from NWDAF for the generation of PCC rules. Specifically, the PCF shall be able to map the above information to QoS parameters, which include Packet Delay Budget (PDB) for UL/DL per UE or/and Packet Error Rate (PER) for UL/DL per UE or/and Packet Loss Rate (PLR) for UL/DL per UE or/and Guaranteed Bitrate (GBR) for UL/DL per UE or/and Variance of Packet Delay for UL/DL of all UEs (on top of the original BDT related PCC rules), and store/retrieve them in/from the UDR as part of the BDT transfer policy.

PCF shall be available to re-evaluate conditions for the AIML/BDT data transmission at any time (prior to the actual transmission) based on the outcome on the enhanced Network Performance analytics of NWDAF, which supports the additional QoS parameters (e.g. packet delay, packet error rate, packet loss rate, guaranteed bitrate), the PCF shall be able to re-negotiate the background data transfer policy with the AF/NEF.

NEF shall be able to consider latency and reliability requirements received from the AF in BDT negotiation request as part of authorizing the AF's permission to submit the BDT request.

***** End of Change *****