SA WG2 Meeting #S2-152E	S2-2206713r01

17 - 26 August, 2022, E-Meeting

Source: 	InterDigital

Title: 	KI#5: Evaluation of Solutions for KI#5 and conclusion

Document for: 	Agreement

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: The contribution analyses the solutions on KI#4 and proposes the conclusion principles for KI#5.

# 1	Discussion

Solution #7, Solution #11, Solution #13, Solution #15, Solution #18, Solution #27, Solution #28, Solution #35, Solution #36, and Solution #37 are for Key Issue #5. For solutions that address multiple Key Issues, only the part that’s related to KI#5 will be discussed.

This paper proposes text for the Key Issue #5 evaluation and conclusion sections.

# 2	Proposal

It is proposed to agree the proposed text for inclusion in TR 23.700-08.

*** BEGIN CHANGES ***

# 7	Evaluation

Editor's note:	This clause provides an evaluation of the solutions.

## 7.x Evaluation of Solutions for Key Issue 5: Enabling access to localized services via a specific hosting network

Table 7.x-1: Summary of solutions for KI#5

| Solution #7 | The solution provides the high level entities and procedures for providing access to localized services, accessing localized/home services, and charging information for the home network operator. |
| --- | --- |
| Solution #11 | Solution is reusing existing roaming architecture and LADN mechanism. The UE uses the local breakout session for accessing specific services from hosting network and home routed session for accessing services from the home network. Only PNI-NPN as Hosting Network is considered. |
| Solution #13 | The UE will be configured with the N3IWF address of the home network, wherein for accessing the home network services. The UE uses the hosting network as an underlay network. |
| Solution #15 | The solution focuses more on KI#4. |
| Solution #18 | Solution proposes to use the existing mechanism i.e., Steering of Roaming to allow the service provider (PLMN or 3rd Party service provider) to update the UE with required information on availability of hosting networks along with the localized services they are providing. Steering of Roaming information container would carry the "operator defined hosting network selector list", which will assist the UE to switch to hosting network for obtaining localized services. |
| Solution #27 | Solution is reusing N3IWF overlay architecture and LADN mechanism, i.e., 5G System architecture with access to SNPN using credentials from Credentials Holder and 5G System architecture accessing overlay network (home network) via underlay network (hosting network). The solution is built on top of solution #11. |
| Solution #28 | The UE (User) obtains the time restricted credentials from the LSP via the home network and uses that information for selection of hosting network as well as to access localized services. N3IWF node used to access home network services. |
| Solution #35 | The home network provides URSP which maps local services to its S-NSSAI and DNN. The registration of the S-NSSAI related to local services causes the cell/network reselection. |
| Solution #36 | In one of the options presented, home network and UE coordinate, wherein home network would either use the UE location to get information about target suitable hosting network performance information via target hosting network (e.g., SNPN) NEF or home network would instruct the UE to measure and report the signal reception status of the potential target hosting network(s). Based on the information received from the UE, the home network would send SoR (Steering of Roaming) instructions to steer the UE toward the target hosting network (e.g., SNPN). |
| Solution #37 | The solution defines a new network function LSF (Localized Service Function) in the home network and hosting network that specifically handles information related to localized services. The new network function will provide information to AMF about all available localized services at the UE location and this information about localized services will be shared with the UE via NAS signaling. |

In several solutions (e.g., #18, #36, etc.) , existing SoR mechanism with some enhancement is proposed to be used for the home network to steer the UE to the hosting network. This approach addresses the KI well and has limited impacts on the UE and network.

In the solutions, the typical triggers for steering the UE to a specific hosting network are:

- The time and location of UE has met the validity condition of a local service (e.g., #18).

- The performance of the hosting networks (e.g., #36).

All these triggers can be supported.

For the aspect of enabling access to both home network services and local services. Several solutions propose to use existing overlay/underlay architecture to access the target network through N3IWF. This approach addresses the KI well and has limited impacts on the UE and network.

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

## 8.x	Conclusions for Key Issue #5: Enabling access to localized services via a specific hosting network

The following conclusions are agreed for normative work:

- The home network determines whether to steer a UE towards a hosting network based on:

  - Whether the time and UE location meets the validity condition of a local service.

  - The performance of the hosting network.

- The existing overlay/underlay architecture is reused for the UE to access both home network services via N3IWF and localized services, where the localised services provided by local network are accessible directly via underlay connection

Editor’s note: the scenario of accessing to localized services via N3GPP network is for further consideration

*** END CHANGES ***