SA WG2 Meeting #SA2-152E 	S2-2206772r03

17 – 26 August, 2022 Electronic meeting

Source:	China Mobile

Title:	New Solution: Supporting ECN in 5GS for XR/media Enhancements

Document for:	Approval

Agenda Item: 	9.19

Work Item / Release:	FS_XRM

Abstract of the contribution: This document proposes a solution update for Sol#43

- Discussion

- Proposal

It is proposed to capture the following aspects in TR 23.700-60.

# * * * * Start of change * * * *

6.X	Solution #X: Supporting ECN mark for L4S in 5GS for XR/media Enhancements

6.X.1	Key Issue mapping

The solution addresses Key Issue #3: 5GS information exposure for XR/media Enhancements.

6.X.2	Description

The ECN mark for L4S technology has been describe in Sol#41,#43 and#46. There are some differences between the three solutions but the main idea is the same that introducing ECN bits in the 5GS to indicate the node congestion happen.

While some aspects should be further clarified.

1) PCF enhancement

When AF provides QoS parameters requirement for specific XR/media traffic, the request for utilizing ECN mark for L4S can also be included. AF may also provide ECN marking for L4S instruction information e.g. support for adapting data rate based on ECN marking for L4S, desired data rate for application to perform properly while ensuring Quality of experience for end user. PCF can generate corresponding policy based on these instruction information, and send the policy to RAN or UPF to help the ECN marking.

2) UE capability indication

UE should have the capability to support L4S mechanism. UE is considered as an endpoint for the data transmission path, receiver or sender. For downlink traffic, if the NG-RAN adds the ECN marking for L4S, the UE as a receiver is expected to generate a congestion notification in TCP layer and sends it out as the uplink data to application server. For uplink traffic, if the NG-RAN adds the ECN marking for L4S, the UE as a sender is expected perform congestion response based on the received congestion notification.

However, L4S traffic cannot be detect via user plane. ECT(1) in the user plane data header can be used to indicate either ECN or L4S is supported. Based on ECT(1) from a sender (UE or Server) supporting ECN but not supporting L4S, the UPF and RAN mis-detects the traffic as L4S and RAN marks CE for L4S when a lighter congestion happens. The sender supporting ECN but not L4S will misunderstand the received L4S CE (“lighter congestion”) as ECN CE (“heavier congestion”) and take an aggressive congestion response. This over reaction negatively impacts on the UE throughput. Therefore, during PDU session establishment, the UE notifies the network the UE supports L4S mechanism. Based on the UE capability notification, the ECN mark for L4S is performed.

3) 5G Mobility consideration

During NG Handover, if the target NG-RAN is no longer congested, it can stop marking ECN for L4S and the application may start to gradually increase the data rate.

6.X.3	Impacts on services, entities and interfaces

AF:

-	Provide ECN marking for L4S instruction information e.g. support for adapting data rate based on ECN marking for L4S, desired data rate for application to perform properly while ensuring Quality of experience for end user

PCF:

-	Generate corresponding policy based on AF instruction information, and send the policy to SMF and RAN or UPF to help the ECN marking for L4S.

UE :

-	Supports L4S mechanism.

-    Support to notify the network the UE supports L4S mechanism

# * * * *end of change * * * *