3GPP SA WG2 Meeting #152E	S2-2206773r02

Elbonia, Aug. 17 – 26, 2022

Source:	Google Inc.

Title:	Sol#39 Update: Clarification of User consent for Event/Analytics Exposure

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys / Rel-18

Abstract of the contribution: This paper proposes to update Solution#39 with clarify user consent checking procedure.

# Discussion

This solution proposes to update Solution#39 with clarification on user consent checking procedure and address the following Editor’s notes:

- Editor's note:	The user consent checking for the UE related 5GC information or analytics exposure to 3rd AF needs to cooperate with SA3.

- Editor's note:	Whether the enhancement of the current procedures for Analytics Exposure or Event Exposure is FFS.

# Proposal

It is proposed to include the following updates for solution#39 in TR 23.700-80.

****** Start of changes ******

## 6.39	Solution #39: 5GC information assistance to FL members' selection

### 6.39.1	Description

This solution addresses Key Issue #7 "5GS Assistance to Federated Learning Operation" on assistance to selection of UEs for FL operation.

This solution is based on the principles as follows:

-	The FL group management is under the control of the AF; and

-	The 5GC NF just provides some network information to help the AF to select and manage the group of UEs which will be part of FL operation; and

-	How the AF selects and manages the group of UEs which will be part of FL operation based on the network information is out of 3GPP scope.

To request the network information from the 5GC NF, the AF may provide an Area of Interest and/or a UE group ID to reduce the signalling of network information exposure. Take the FL assisted Word Query Suggestion Model as an example, the AF for Gboard on Android (i.e. the Google Keyboard) may provide TAC (Type Allocation Code) as an External UE group ID to 5GC NF to request the UE's network information if the UE is an Android terminal. Furthermore, there are hundreds of millions of Android UEs in the 5G network, so to save the signalling of network information exposure, the AF may also provide an AoI to 5GC NF to request the UE's network information if the UE is located in the AoI or about to enter the AoI.

The assistance network information which can be provided by 5GC NFs to assist FL members' selection by AF is shown in Table 6.39.1-1.

Table 6.39.1-1: Information which can be provided by 5GC NFs for FL members' selection

| Network Information | 5GC NF | Description | Actions of AF (examples) |
| --- | --- | --- | --- |
| UE locations (1..max) | AMF | UE positions | AF only selects the UEs which are located in the requested AoI to join the FL group. |
| >UE location |  | TA or cells that the UE enters (NOTE 1) |  |
| >Timestamp |  | A time stamp when the AMF detects the UE enters this location |  |
| CM state | AMF | The NAS signalling Connection state of the UE with the AMF, including CM-IDLE state and CM-CONNECTED state | AF only selects the UEs which are in CM-CONNECTED state to join the FL group. |
| UE Reachability | AMF | Whether the UE is reachable | When the number of the connected UEs is not enough, AF may select the UEs which are in IDLE state but are reachable to join the FL group. |
| UE Mobility | NWDAF | Statistics or predictions on UE Mobility. | AF only selects the UEs which are about to enter (UE mobility predictions) the requested AoI to join the FL group. |
| UE Abnormal behaviour | NWDAF | Statistics or predictions on UE Abnormal behaviour. | AF does not select the UEs with abnormal behaviour to join the FL group. |
| NOTE 1: UE location includes either the last known location or the current location, under the conditions defined in Table 4.15.3.1-1 in TS 23.502 [4]. |  |  |  |

To subscribe to the UE locations, CM state and UE Reachability from AMF, the Event IDs "Location Report", "Connectivity state changes", "UE reachability status" as defined in clause 5.2.2.3.1 of TS 23.502 [4] are used respectively. Instead of the "Location Report", the "UE moving in or out of a subscribed "Area Of Interest"" event can also be used to monitor if a UE is located in the requested AoI.

To subscribe to the UE Mobility analytics and Abnormal behaviour related network data analytics from NWDAF, the Analytics ID "UE Mobility" as defined in clause 6.7.2 of TS 23.288 [6] and "Abnormal behaviour" as defined in clause 6.7.5 of TS 23.288 [6] are used respectively.

NOTE:	Which network information is requested by the AF depends on the FL member selection criteria at the AF.

Editor's note:	Whether the solution can be extended to provide generic 5G Core assistance for UE selection for Application AI/ML operation, and then be applicable to other key issues, is FFS.

### 6.39.2	Procedures

Figure 6.39.2: Procedure for 5GS assistance to FL members' selection

The AF ensures that the UEs associated to the UE group ID have application users consent in place and then follows the following procedure.

1a-5a.	The procedure in clause 6.1.1.2 of TS 23.288 [6] for UE-related analytics subscribe by AFs via NEF are re-used.

1b-5b.	The procedure in clause 4.15.3.2.3 of TS 23.502 [4] for UE-related event notifications subscribe by AFs via NEF are re-used.

After having notification from the 5G network, the AF selects AI/ML user members for FL operation. Then the AF initiates the Nnef_AFSessionWithQoS_Create/Notify/Update/Revoke procedure in TS23.502 [4] clause 4.15.6.6 for selected UEs for AI/ML operation.

In step 1a or step 1b, AF can include user consent indication and the UE filtering policy (e.g. UE group ID, Expected number of UE for reporting, validity conditions of areas of interest and time) in the subscription service operation, and NWDAF or AMF can derive the notifications based on the UE filtering policy.

With user consent indication and the UE filtering policy (e.g. UE group ID), the NEF checks with UDM for the user consent of the requested Analytics and Events Exposure Notifications before proceed step 3a and 2b. The NEF only indicates the list of UEs with granted user consent to NWDAF or UDM. The NEF also subscribes to the UDM with user consent change notification. If user consent changes for the UE, the NEF notifies the AF indicating the UE ID with proper cause value.

In step 3a and step 2b, the user consent indication is indicated to NWDAF and UDM, respectively. With the user consent indication and target list of UEs for Analytics and Event Exposure, the UDM does not check user consent again. If target list of UEs is not indicated, i.e. for any UEs, the user consent indication triggers requested NF, e.g. NWDAF, UDM, to check user consent of UEs that meet the validity conditions, e.g. Area of Interest, time.

Alternatively, the AF can check user consent before proceeding step 1a-5a and step 1b-5b by subscribing to a new Event notification of user consent change by using the procedure in clause 4.15.3.2.3 of TS 23.502 [4] for UE-related event notifications.

Editor's note:	Whether the enhancement of the current procedures for Analytics Exposure or Event Exposure is FFS.

Editor's note:	The user consent checking for the UE related 5GC information or analytics exposure to 3rd AF needs to cooperate with SA3.

Editor's note:	It is FFS whether the 5G network that exposes the network information needs to be aware of the purpose of the FL operation of the application.

### 6.39.3	Impacts on services, entities and interfaces

AF:

-	Support to subscribe the Event IDs "Location Report", "Connectivity state changes", "UE reachability status", and/or "UE moving in or out of a subscribed "Area Of Interest"" by AMF, via NEF.

-	Support to subscribe the Analytics IDs "UE Mobility" and "Abnormal behaviour" by NWDAF, via NEF.

-	Including the UE filtering policy in the subscription service operation

-	Including user selection indication for AnalyticsExposure_Subscribe/EventExposure Subscribe

NEF:

- Support to check user consent if receiving user selection indication in AnalyticsExposure_Subscribe or EventExposure Subscribe from AF

- Support to subscribe the new Event ID “user consent changes” from UDM for Event Exposure.

- Support to notify the AF with the UE ID and the proper cause of notification when user consent of a UE is changed.

### ****** End of changes ******