3GPP TSG-WG SA2 Meeting #152E meeting	S2-2206783r03

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Lenovo

Title:	KI #4, New Sol: Discovery and selection of Hosting network based on URSP rules and configuration information

Document for:	Approval

Agenda Item:	9.4

Work Item / Release:	FS_eNPN_ph2/Rel-18

Abstract of the contribution: This solution proposes to use the URSP rules to configure the UE to perform Hosting network discovery and selection for Localized service(s). This solution also supports manual Hosting network selection.

# 1. Discussion

According to the SA1 requirements from TA 22.261, a mechanism is required to provision the UE with information to:

-	discover, select and access suitable hosting network for localized services with possible validity conditions (e.g., time and location constrains); and

-	allow the UE, which is already registered in a network (PLMN or NPN), to discover a suitable hosting network (i.e. in order to use the localized services provided via the hosting network) when the hosting network and/or localized services become available.

# 2. Proposal

It is proposed to add the following new key issue to 3GPP TR 23.700-08.

* * * First Change * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | X |  |  |  |  |  |
| 2 |  | X |  |  |  |  |
| 3 |  | X |  |  |  |  |
| 4 |  | X |  |  |  |  |
| 5 |  | X |  |  |  |  |
| 6 |  | X |  |  |  |  |
| 7 |  |  | X | X | X | X |
| 8 |  |  |  |  |  | X |
| 9 |  |  |  |  |  | X |
| 10 |  |  |  | X |  | X |
| 11 |  |  |  |  | X |  |
| 12 |  |  |  | X |  |  |
| 13 |  |  |  | X | X |  |
| 14 |  |  |  | X |  |  |
| 15 |  |  |  | X | X |  |
| 16 |  | X |  |  |  |  |
| 17 |  |  |  |  |  | X |
| 18 |  |  |  |  | X |  |
| XX |  |  |  | X | X |  |

* * * Second Change * * * *  ALL text is new

## 6.X	Solution #XX: Hosting network selection based on information for hosting network selection and triggered by URSP

### 6.X.1	Introduction

This solution targets KI#4 and KI#5.

This solution applies to both manual and automatic Hosting network selection.

### 6.X.2	Functional Description

#### 6.X.2.1	General

This solution is based on the following main principles:

-	the SoR-AF in the Home network configures the UE with information for Hosting network selection (i.e. Localized Service information). The Localized Service information can be used for automatic or for manual Hosting network selection.

-	upon evaluation of the URSP rules for traffic from a Local service application, the RSD of a URSP rule (which is matching rule to the Localized Service) is enhanced with a component which triggers the UE to trigger an automatic Hosting NW discovery and selection. The enhanced URSP's RSD can be used to offload the UE's traffic to a Hosting network (as described in clause 6.X.2.3) or to register to Hosting Network as overlay network (as described in clause 6.X.2.4). Such URSP rule triggered network selection can be described as service-based Hosting network selection.

#### 6.X.2.2	Configuration of Localized Service information in the UE

It is assumed that a Localized Service Provider (LSP) and a Hosting network has negotiated which Localized service(s) are allowed to be used in the Hosting network (e.g. identified by an application ID). Further, the LSP and the Hosting network can negotiate the validity conditions when and where the Localized service(s) will be offered by the Hosting network, e.g. time period and location constrains.

The UE stores subscription ID, credentials and configuration from the Home network and may store subscription ID, credentials and configuration (e.g. list of preferred Hosting networks) from the Hosting network.

The LSP can perform service provisioning to the Home network by including the Localized service ID, target Hosting NW ID, validity criteria and further information.

The Home network identifies the UEs which may use the Localized service(s). The Home network, e.g.

the SoR-AF in the Home network configures the UE with Localized Service information (i.e. information for Hosting network selection), which may include a list of hosting network IDs in priority order, and which may be associated with the Hosting network subscriber profile (i.e. subscription ID and credentials). In addition, the information for Hosting network selection may include network selection criteria when and/or where the Hosting network is considered as available, e.g. time period (start time and end time), location information. For example, a list of hosting network IDs may be used:

-	during automatic Hosting NW selection triggered by e.g. network selection criteria or by matching URSP rule (refer to clause 6.X.2.3), or

-	during manual Hosting NW discovery and selection where the list of Hosting network IDs is used to filter the discovered available networks to be displayed to the user.

#### 6.X.2.3	Automatic Hosting network selection

The automatic Hosting network selection may be triggered (a) by the network selection criteria from the Localized Service information or (b) by the RSD part of the URSP rule in case of URSP rules evaluation.

For the purpose of case (b), the Home network's PCF creates URSP rule(s) for the localized service(s) which are identified in the Traffic Descriptor (TD) part of the URSP rule. The Route Selection Descriptor (RSD) part of the URSP rule includes a component "hosting network offload", which means that the UE should attempt to select and register with a Hosting network to transmit data for this application. The RSD is associated with Route Selection Validation Criteria including the validity conditions (e.g., time and location constrains) as negotiated with the hosting network. If a Localized application in the UE wants to send traffic, the matching URSP rule would trigger the UE to perform automatic Hosting NW discovery and selection that uses the list of Hosting network IDs associated with Hosting network subscriber ID and credentials (as described in clause 6.X.2.2).

#### 6.X.2.4	Hosting network discovery and selection as overlay network

In addition, or optionally to the above solution, the UE can simultaneously use the Home network service(s) via the Home network and the Localized service(s) via the Hosting network, whereby the Hosting network is used as an overlay network and the Home network as underlay network. For this purpose, the UE stores subscription ID and credentials from the Home network and subscription ID and credentials from the Hosting network.

The UE stores configuration in the corresponding subscription profile for the Hosting network which may include Hosting network operator controlled prioritized lists of preferred SNPNs/GINs or ANDSP policies. When the UE performs Hosting network selection procedure, the UE applies the configuration associated with the Hosting network.

The Home network (or the underlay network in general) configures the UE with URSP rules, which indicate that the traffic of an application or certain Traffic Descriptor should be send via an overlay network ID. A new RSD component is introduced in the URSP rules of the underlay network as shown in Figure 6.X.2.4-2. This allows the UE to determine (a) what traffic should be sent via an overlay network and (b) the identity of the overlay network that should be used. The UE then attempts the registration procedure with the overlay network via a PDU Session established in the underlay network.

Figure 6.X.2.4-1: Example deployment of localized services A, B and C

The figure 6.X.2.4-1 shows an example deployment of localized services A, B and C and the corresponding hosting networks "a" and "b" which provide connectivity to the localized services.

When the UE detects traffic that matches the URSP rule, the UE establishes a PDU Session in the underlay network based on the RSD (if such PDU Session is not already established). Subsequently, the UE performs Hosting network selection and registration procedure (e.g. using the configured ANDSP policies from the Hosting network). The UE establishes connectivity to the overlay network (i.e. the hosting network) via the PDU Session of the underlay network. The UE sends the Localized application traffic to the Hosting network via a PDU Session established in the Hosting network. The Figure 6.X.2.4-2 shows an example of the URSP rule from the underlay network (i.e. HPLMN) which matches the localized service. The PDU Session establishment in the underlay network uses the RSD of the URSP rule of the underlay network. The UE establishes a PDU session in the overlay network (i.e. hosting network) by using existing mechanisms (e.g., by applying URSP rules from the overlay network, or by requesting a default PDU session).

Figure 6.X.2.4-2: Example for the URSP rule from the underlay network (i.e. HPLMN)

### 6.X.3	Procedures

The Figure 6.X.3-1 shows the procedure how the UE is configured with information to use Localized services via the Hosting network. The procedure includes the configuration of the UE:

-	for automatic Hosting network selection: with URSP rules including an RSD component for "Hosting NW offload"; and

-	for manual Hosting network selection: with "information for Hosting network selection" (which includes a list of Hosting network IDs associated with selection criteria).

Figure 6.X.3-1:  Procedure for UE configuration to use Localized services

The detailed description of the steps is provided as follows:

0.	The Localized Service Provider (LSP) and a Hosting network (HtNW) (e.g. OAM system or OSS) negotiate which Localized service(s) should be deployed in the used in the Hosting network. It is also negotiated how Localized service is identified (e.g. identified by an application ID) and which are the Localized service's validity conditions (time period, location). The Hosting network propagates the service deployment in the corresponding network functions (e.g. UDM/UDR, etc.).

1.	The UE is registered with the Home network (or any roaming partner). The UE uses connectivity to services provided by the Home network, and in addition, the application App1 of the UE may connect to the LSP for App1 and the user/application may request a service subscription or service usage.

2.	The LSP for App1 acts as an Application Function (AF) and sends a request for service provisioning to the Home network. The service provisioning request may include: GPSI, AF ID, Service Information (target/serving Hosting NW ID, service availability conditions (e.g. time period, location)). The Hosting NW ID identifies the network where the Localized service for App1 is supported.

The LSP can use an existing service exposure operation, e.g. service specific parameter provisioning, or application guidance for URSP determination. After NEF/UDR authorizes the request, the content is forwarded to the UDR.

3.	The UDR may store this information in the related service subscription data and/or in the subscription data of the UEs corresponding to the list of GPSI(s). The UDM/UDR translates the GPSI to the UE subscriber identity (SUPI) and also the location information to the HPLMN topological location information (e.g. Tracking Areas).

(automatic network selection)

4a.	The UDR may determine to trigger a service availability update procedure based on different triggers, e.g. the service availability update from the LSP may trigger this step.  The UDR may send service availability update request to the corresponding PCF containing: the Application ID (e.g. App1), serving Hosting NW ID (where the service may be locally available) and the availability conditions like time period and location as described in step 2.

4b.	The H-PCF determines that the UEs has to be updated for the routing of App1 traffic. The H-PCF may generate a new URSP rule for App1 that can include a new route selection description (RSD) component called "Hosting NW offload". There are 2 options how this new component can be implemented:

-	The RSD component indicates a network ID to be used to select a Hosting NW via a direct access for the application traffic.

-	The RSD component indicates to use offload via a Hosting network, where a list of Hosting network identifiers is configured in a separate list according to step 5a and 5b.

Further, the H-PCF can ser RSD validation criteria corresponding to the time period and location received for the application App1 in step 2.

4c.	The H-PCF initiates a signalling procedure to update the URSP policy in the UE.

5a.	The UDM/UDR (or by interrogating with a SOR-AF) may determine that the UE needs to be provided with "information for Hosting network selection". The "information for Hosting network selection" may include a list of Hosting NW IDs, where each Hosting NW ID may be associated with selection criteria like time period, location and supported Localized service ID. For this purpose, a list called "Operator controlled Hosting Network selector with Access technology" can be sent to the UE.

In addition, if manual Hosting network selection is to be allowed, the "information for Hosting network selection" may also include information how a Hosting network ID maps to a human readable name.

5b.	The UDM can trigger the UE parameter update procedure in order to provide to the UE the "information for Hosting network selection" (e.g. the list of Hosting NW IDs). If the UE stores subscriber ID and credentials for the Hosting network, the UE associates the received configuration information with this subscriber ID and credentials.

6.	(Automatic NW selection) When an application associated with the Localized service (App1) wants to send traffic and there is a matching URSP rule with valid RSD (according to time period and location) and the RSD component indicates "Hosting NW offload", the UE internally triggers a Hosting network discovery procedure. The UE uses the UE configuration information for Hosting network selection (i.e. the list of Hosting network IDs in priority order as configured in step 5b or the network ID in the “Hosting NW offload” component of the URSP rule as in 4b) to discover available Hosting networks.

7.	(Manual NW selection) When the user requests Hosting network selection, the UE uses the "information for Hosting network selection" and the selection criteria (from step 5b) for each Hosting NW ID in order to perform search of the available Hosting networks. The UE considers the Hosting network selection criteria when and/or where the Hosting network is available. The UE displays to the user only the human readable names of the Hosting networks which are available and which fulfil the selection criteria.

### 6.X.4	Impacts on services, entities, and interfaces

UE impacts:

-	Ability to receive and maintain URSP rule with RSD component indicating "Hosting NW offload" and/or "overlay network";

-	Ability to receive and store "information for Hosting network selection" (aka Localized Service information) which is used for:

(a)	network selection when an application requests connectivity and matching URSP rule with valid RSD with "Hosting NW offload" component is applicable.

(b) for manual Hosting network selection.

(c) switching of UE access mode.

H-PCF impacts:

-	Ability to create URSP rule including an RSD with a component "Hosting NW offload" and Route Selection Validation Criteria including the validity conditions for the localized service.

-	Ability to create URSP rule including an RSD with a component "overlay network".

UDM / SoR-AF impacts:

-	Ability to create "information for Hosting network selection" (aka Localized Service information), which may include a list of Hosting network IDs in priority order, and optionally, associated network selection criteria when and/or where the Hosting network is considered as available.