SA WG2 Meeting #S2-152e(e-meeting)	S2-2206865r07

17 - 26 August 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Qualcomm Inc.

Title:	Conclusion proposal for KI#1

Document for:	Approval

Agenda Item:	9.8

Work Item / Release:	FS_RedCap_Ph2/Rel-18

Abstract of the contribution: Introduction of conclusion to support CN handling of MT data/signalling during long eDRX in RRC Inactive.

This paper proposes an interim conclusion for FS_RedCap_Ph2.

There are essentially two categories of proposed solutions in clause 6 of TR 23.700-68:

-	Category 1: Solutions that rely on buffering at the 5GC

-	Category 2: Solutions that rely on buffering at the NG-RAN

CN based solutions in category 1 are mainly for SA2 to decide. Aspects in category 2 requiring extended NG-RAN buffering require RAN support and can only be introduced together with the RAN groups.

# 2. Proposal

This paper proposes the following updates to TR 23.700-68 clause 8.

* Start of change *

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

## 8.1	Conclusions for Key Issue #1: Enabling long eDRX cycle support in RRC_INACTIVE state

The following aspects are concluded as principles for the normative work:

-	It is agreed to support MT data signalling handling in CN when the UE is unreachable due to long extended DRX in RRC inactive.

-	The eNB determines if and when extended CN buffering is needed.

-	For the MT data/signalling handling in CN during unreachable time approach, the gNB sends an indication to CN and provides unreachability information (e.g., eDRX values negotiated between UE and gNB for RRC_INACTIVE state) to CN (when UE is entering RRC_INACTIVE state with long eDRX or when receiving a MT traffic targeted for the UE). If CN handles the MT data, CN applies the HLCOM functions based on gNB provided unreachability information (as described in solution #6).

-	If the eNB has indicated UE unreachability to the CN, the gNB notifies also CN with RRC State transition to RRC_CONNECTED from MT data/signalling delivery (as described in solution #6).

- 	RAN paging triggered by CN in RRC_INACTIVE state may be escalated to entire Registration Area if RAN paging in the RNA failed.

NOTE: further synch with RAN WGs is needed.

* End of changes *