SA WG2 Meeting #S2-152e(e-meeting)	S2-2206865r08

17 - 26 August 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Qualcomm Inc.

Title:	Conclusion proposal for KI#1

Document for:	Approval

Agenda Item:	9.8

Work Item / Release:	FS_RedCap_Ph2/Rel-18

Abstract of the contribution: Introduction of conclusion to support CN handling of MT data/signalling during long eDRX in RRC Inactive.

This paper proposes an interim conclusion for FS_RedCap_Ph2.

There are essentially two categories of proposed solutions in clause 6 of TR 23.700-68:

-	Category 1: Solutions that rely on buffering at the 5GC

-	Category 2: Solutions that rely on buffering at the NG-RAN

CN based solutions in category 1 are mainly for SA2 to decide. Aspects in category 2 requiring extended NG-RAN buffering require RAN support and can only be introduced together with the RAN groups.

# 2. Proposal

This paper proposes the following updates to TR 23.700-68 clause 8.

* Start of change *

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

## 8.1	Conclusions for Key Issue #1: Enabling long eDRX cycle support in RRC_INACTIVE state

The following aspects are concluded as principles for the normative work:

-	It is agreed to support MT data signalling handling in CN when the UE is unreachable due to long extended DRX in RRC inactive.

-	The gNB sends an indication to the CN and provides unreachability information (e.g., eDRX values negotiated between UE and gNB for RRC_INACTIVE state) when UE is entering or has entered RRC_INACTIVE state with long eDRX. When this occurs the CN applies the HLCOM functions based on gNB provided unreachability information (as described in solution #6).

-	If the gNB has indicated the UE has entered RRC_INACTIVE to the CN, the gNB also notifies the CN about the RRC State transition back to RRC_CONNECTED (as described in solution #6).

NOTE 1: 	If the indication of transition to RRC_INACTIVE is delayed by the gNB then the CN cannot apply HLCOM functionality and other NFs will not be aware of the UE reachability, including AFs which may for example continue to send data for the UE. This data might then be discarded and not delivered to the UE.

NOTE 2:	Further coordination with RAN WGs may be needed during the normative phase.

* End of changes *