SA WG2 Meeting #S2-152e(e-meeting)	S2-2206865r06

17 - 26 August 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Qualcomm Inc.

Title:	Conclusion proposal for KI#1

Document for:	Approval

Agenda Item:	9.8

Work Item / Release:	FS_RedCap_Ph2/Rel-18

Abstract of the contribution: Introduction of conclusion to support CN handling of MT data/signalling during long eDRX in RRC Inactive.

This paper proposes an interim conclusion for FS_RedCap_Ph2.

There are essentially two categories of proposed solutions in clause 6 of TR 23.700-68:

-	Category 1: Solutions that rely on buffering at the 5GC

-	Category 2: Solutions that rely on buffering at the NG-RAN

CN based solutions in category 1 are mainly for SA2 to decide. Aspects in category 2 requiring extended NG-RAN buffering require RAN support and can only be introduced together with the RAN groups.

# 2. Proposal

This paper proposes the following updates to TR 23.700-68 clause 8.

* Start of change *

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

## 8.1	Conclusions for Key Issue #1: Enabling long eDRX cycle support in RRC_INACTIVE state

The following aspects are concluded as principles for the normative work:

-	It is agreed to support MT data/signalling handling in CN when the UE is unreachable due to long extended DRX in RRC inactive.

-	The eNB determines if and when extended CN buffering is needed.

-	For the MT data/signalling handling in CN during unreachable time approach, the gNB sends an indication to CN and provides unreachability information (e.g., eDRX values negotiated between UE and gNB for RRC_INACTIVE state) to CN (when UE is entering RRC_INACTIVE state with long eDRX or when receiving MT traffic targeted for the UE). If CN handles the MT data, CN applies the HLCOM functions based on gNB provided unreachability information (as described in solution #6).

-	Based on the deployment scenario (UPF collocated with NG-RAN, CU-UP split, etc.), the traffic model, mobility patterns, the gNB may send the request for MT data/signalling handling in CN with unreachability information, when the UE enters RRC_INACTIVE state, or when the gNB receives MT data/signalling, or when the gNB cannot buffer any (further) MT data.

NOTE 1: Extended RAN buffering capability enhancement to support long eDRX cycle is not supported.

NOTE 2: Data forwarded over N3 while the UE is in RRC-Inactive might be discarded by the RAN node and not delivered to the UE.

NOTE 3: HLCOM functionality is not supported until the request for MT data/signalling handling in CN is received and acknowledged by the CN.

-	If the eNB has indicated UE unreachability to the CN, the gNB notifies also CN with RRC State transition to RRC_CONNECTED from MT data/signalling delivery.

- 	RAN paging triggered by CN in RRC_INACTIVE state may be escalated to entire Registration Area if RAN paging in the RNA failed.

NOTE 4: further synch with RAN WGs is needed.

* End of changes *