3GPP TSG-SA WG2 Meeting #152e	S2-2206898r07

# Aug 17 – 26, 2022

# Title:	[draft]LS on XR and Media Services

# Response to:	LS on UE Power Saving for XR and Media Services (R1-2205531/S2-2205421)

# Release:	Rel-18

# Work Item:	FS_XRM, FS_NR_XR_enh

Source:	3GPP TSG SA WG2

To:	RAN WG1, RAN WG2

Cc:

Contact Person:

Name:	Xiaowan Ke

Tel. Number:

E-mail Address:	xiaowan(dot)ke(at)vivo(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

SA2 would like to thank RAN1’s LS reply (R1-2205531/S2-2205421) on UE power saving for XR and media services. So far, SA2 starts to evaluate and conclude solutions for key issues for the study of XR and Media Services and would like to coordinate with RAN WG about the following aspects

- KI#3 studies what information is useful and can be exposed by 5GS to the server. Wherein, SA2 is considering RAN node congestion and per QoS flow congestion which are exposed by RAN. SA2 understands the granularity of RAN scheduling is DRB/LCH based and a DRB can be used for multiple QoS flows, hence SA2 would like to request RAN2 to feedback

- whether it is feasible for RAN to detect the per QoS flow or DRB congestion and per RAN node congestion.

- KI#4&5 is about the PDU Set handling, mainly including PDU Set identification and PDU Set level QoS. SA2 will focus on the how to support the PDU Set handling for downlink XR traffic. The uplink is closely related to the interaction between the UE and RAN, hence SA2 would like to request RAN to decide whether and how to support the PDU Set handling for uplink XR traffic. SA2 will align with RAN’s progress and decision for uplink, if any.

- KI#8, i.e. power saving and CDRX enhancement, SA2 is working the conclusion. Before that,

- SA2 would like to clarifies he main usage of following information raised in (R1-2205531/S2-2205421) is about PDU Set QoS and PDU Set integrated handing: “PDU set level QoS parameters including priority and delay budget of a PDU set, PDU set identity and relationship information among PDUs within the same PDU set, PDU set end indication or indication of the last PDU in a PDU set, PDU set size (number of bits) or number of PDUs in a PDU set.” Whether the above information is concluded is left to PDU Set QoS and PDU Set integrated handing. If they were concluded, whether and how to use them into power saving and CDRX enhancement will be left to RAN WG’s decision. SA2 would like to inform RAN1 that relying on above listed PDU Set information alone may not result in an optimal CDRX configuration. Traffic over one QoS flow may be made up of multiple PDU sets and these PDU sets may belong to a single traffic pattern or may belong to multiple traffic patterns. As a result, a PDU set end indication does not necessarily mark the end of a Data Burst. It may be important for RAN to detect the end of data bursts.

From 3GPP TR 23.700-60 V0.3.0:

PDU Set: A PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. a frame or video slice for XRM Services, as used in TR 26.926). In some implementations all PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information. In other implementations, the application layer can still recover parts all or of the information unit, when some PDUs are missing.

Data Burst: A set of multiple PDUs generated and sent by the application in a short period of time.

NOTE: A Data Burst can be composed by one or multiple PDU Sets.

Furthermore SA2 would like to point out that a QoS flow may be characterized by multiple PDU set periodicities (i.e., one for each traffic pattern). Thus, a single / static CDRX configuration may be inefficient for traffic from such a QoS flow. There may be scenarios where multiple flows with different periodicities are mapped to the same QoS Flow.

Consequently, SA2 has concluded that:

- NG-RAN will be provided with assistance information to help NG-RAN configure CDRX and to detect when to change CDRX configurations.

  - Assistance information is provided to the NG-RAN via the SMF, in an N2 SM message.

- For DL data, an End-of-Burst (EOB) indication is provided to the NG-RAN via the UPF, in a GTP-U header.

- Whether and how the UE may be configured with multiple CDRX configurations (depending on the traffic pattern) and that the UE may request to move from one CDRX configuration to another CDRX configuration will be determined by RAN WGs. How the UE indicates what CDRX configuration it intends to use or a change of CDRX parameters (e.g. RRC message MAC CE, or PHY layer control signalling) will be determined by RAN WGs.

SA2 would like to request that RAN1 and RAN2 take these conclusions into account when discussing power saving enhancements for XR applications.

2. Actions:

To RAN WG1, RAN WG2:

ACTION: 	SA2 kindly asks RAN1, RAN2 to take the above information into account and reply.

3. Date of Next TSG SA WG2 Meetings:

SA WG2 Meeting #153     	10 – 14 October August 2022			E-Meeting

SA WG2 Meeting #154      	14 – 18 November 2022			Canada, CA