SA WG2 Meeting #S2-152E	S2-2206900r03

17 - 26 August, 2022, Electronic meeting

Title:	[DRAFT] Reply LS on FS_eEDGEAPP Solution for Support of NAT deployed within the edge

Response to:	S2-2205458 / S6-221953

Release:	Release 18

Work Item:	FS_eEDGEAPP

Source:	SA2

To:	SA6

Cc:	SA3

Contact Person:

Name:	Sašo Stojanovski

E-mail Address:	saso stojanovski intel com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 thanks SA6 on their LS (S2-2205458 / S6-221953) and provides the following feedback on the SA6 questions.

- As mentioned in KI#16 about AF (e.g. EES) with a NATted UE IP address, can SA2 support taking NATted UE IP address as input and then exposing the UE External ID to EES?

Currently (Rel-17) there is no specification support for NAT devices controlled by 5GS, as indicated in the following NOTE in TS 23.501 clause 5.6.10.1:

NOTE 2:	An operator can deploy NAT functionality in the network; the support of NAT is not specified in this release of the specification.

as well as in the following NOTE in TS 23.502 clause 4.15.10:

NOTE 3:	The case where UE IP address provided by the AF to the NEF corresponds to an IP address that has been NATed (Network and Port Address Translation) is not supported in this release.

Although there is currently no specification support for exposure of UE External ID based on the NATted UE IP address, SA2 is working on the related work to support the UE public/private IP address mapping by 5GC in the ongoing Rel-18 study (FS_UPEAS), and there will be a conclusion in 2022 Q3 to decide whether the solution will be included in the normative phase. This solution will support the NAT deployed scenario by 5GC.

- In solution #23, EES (as AF) invokes Nnef_UEID API with UE private IP address allocated by 3GPP CN. In certain deployment when multiple UEs are allocated with the same private IP address, can SA2 support addressing IPv4 address overlap issue?

It is SA2’s understanding that the use of overlapping IPv4 addresses is mainly related to the use of NAT devices, when devices using overlapping IPv4 addresses need to communicate with a common data network (e.g. the Internet). In line with the previous reply, there will be no issue on overlapping IPv4 addresses if the 5GC can support the public/private IP address mapping.

2. Actions:

To SA6 group.

ACTION: 	SA2 respectfully asks SA6 to take into account the feedback above.

3. Date of Next TSG-SA WG2 Meetings:

3GPP SA2#153E			10 - 14 October 2022			e-meeting