SA WG2 Meeting #S2-152E	S2-2206902

17 – 26 August , 2022, E-meeting

Source:	Xiaomi

Title:	evaluation for key issue#3

Document for:	Approval

Agenda Item:	9.4

Work Item / Release:	FS_eNPN_Ph2/Rel-18

Abstract of the contribution: This paper proposes the evaluation for key issue #3.

*** Start of changes ***

7	Evaluation

Editor's note:	This clause provides an evaluation of the solutions.

7.3 Evaluation for KI#3 Enabling NPN as hosting network for providing access to localized services

| Solution#7 High level flow for localized service support | The service provider (can be network operators or 3rd party application providers) of the localized service needs to establish localized service agreement with the operator of a hosting network. A service agreement between hosting network operator and UE's home network operator is needed to enable, e.g.: UE to receive and use configuration provided by a 3rd party service provider, etc. |
| --- | --- |
| Solution#12 Discovering services offered by SNPN while camping in a serving network | Current serving network (PLMN or SNPN) may assist UE in discovering hosting network for localized services in specific conditions, such as when serving network determines that UE moves into area where localized services are available. Some level of co-operation between current serving network and hosting network is needed. |
| Solution#22 Hosting network to provide localized service based on default credentials | It is assumed that UE has default credentials that can be used in the hosting network. Hosting network broadcasts a dedicated Hosting networking service indication to show that this NPN can provide hosting networking for localized services. - In case there is some service agreement between hosting network and home network, so that the UE can be successfully authenticated by the hosting network by using credentials of home network - In case there is no service agreement between hosting network and home network, the hosting network performs authentication by using the UE’s default credentials. The localized services information are pre-configured in AMF of hosting network. |

*** End of changes ***