SA WG2 Meeting #S2-152E	S2-2206906

17 – 26 August , 2022, E-meeting

Source:	Xiaomi

Title:	conclusion for Key Issue#3

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	5G_PIN /Rel-18

Abstract of the contribution: This paper proposes the conclusion for key Issue #3.  This paper merged S2-2206015, S2-2206483, S2-2206906 and S2-2205743

*** Start of changes ***

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.3 conclusion for KI#3 Management of PIN and PIN Elements

- PEMC or UDM or centralized AF is responsible for PINE management (e.g., add/remove the PINEs, etc.) and PIN management (e.g. create/modify/delete/activate/deactivate a PIN, etc.).

- UDM supports for allocating PIN ID for a PIN

- AF or PEMC or UDR stores all the information for PIN management and PINE management (e.g., PIN ID/name, PIN Element list, profile of PINE, PIN configuration).

- UE’s subscription data should contain part of PIN and PINE information when it registers to network as PEMC.

NOTE: The specific PIN or PINE information will be decided in normative work.

- AF may provide necessary information (e.g., PINE ID, IP address/prefix, port number, default QoS associated with a PINE) to 5GC to enable the PEGC UE to route PINE traffic (i.e. URSP) and for traffic differentiation and QoS control for specific PINE, leveraging exposed 5G capabilities for PIN.

- Information for Management of PIN and PIN Elements (e.g. PIN creation/deletion/modification/activation/deactivation/joining PIN etc.) is exchanged in user plane and based on application layer implementation, which is not specified by SA2.

- PINE/PIN management among PINE(s), PEGC(s) and PEMC(s) is enforced via application layer or implementation way.

- A PEMC or a PEGC shall link with a trust member group, which includes at least one PEMC and one or more PEGCs.

- To support management of multiple PEMCs for a PIN, a trust member group may include multiple PEMCs.

- In order to make sure PEMCs and PEGCs using same PINMF, or a PEMC using same PINMF after recovery, the PINMF registers itself into UDR via UDM or NEF for a trust member group.

*** End of changes ***