SA WG2 Meeting #S2-152E	S2-2206906r12

17 – 26 August , 2022, E-meeting

Source:	Xiaomi

Title:	conclusion for Key Issue#3

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	5G_PIN /Rel-18

Abstract of the contribution: This paper proposes the conclusion for key Issue #3.  This paper merged S2-2206015, S2-2206483, S2-2206906 and S2-2205743

*** Start of changes ***

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.3	Conclusion on Key Issue #3

The following principles are concluded for Key Issue #3 "Management of PIN and PIN Elements":

1)	PEMC is responsible for PIN Element management (e.g., add/remove the PINE/PEGCs, etc.) and PIN management (e.g. create/modify/delete/activate/deactivate a PIN, etc.).

4)	UE’s subscription data should contain authorization information when it needs to act as PEGC

.

Editor’s note: If there is other possible  PIN related information needed will be decided on the final conclusion..

Editor’s note: whether UE’s subscription data should contain authorization information, and how to use this information by 5GC needs further evaluation and discussion to decide keep or remove.

9)	When communication between a PEMC and a PINE, who is behind a PEGC, via 5GC and the PEGC is needed, or when communication between PINEs via multiple PEGCs and 5GC is needed, the existing functionalities in 5GS for routing the traffic of the communications via UPF(s) can be applied if available.

Editor’s note: whether AF information needs to be stored in UDR for the case, e.g., when UE change PEMC UE, or user’s PEMC UE is recovered, etc., is FFS.

Editor’s note: whether AF or 5GC NF will be involved in PIN/PINE management depends on the conclusion of KI#1 (whether the architecture of PIN contains the PIN AF or existing 5GC NF)

*** End of changes ***