3GPP SA WG2 Meeting #152E	S2-2205932

e-meeting, 17-26 August 2022

Source:	Nokia, Nokia Shanghai Bell, NTT DOCOMO, Ericsson, ZTE, Huawei, HiSilicon

Title:	Conclusions for KI#3

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract of the contribution: This paper proposes conclusions and way forward for key issue #3 “Support for controlling 5G time synchronization service based on subscription” to the TR 23.700-25.

# Discussion

There are two solutions, Solutions #18 and #19, in the TR that address KI#3. The only aspect of solution #19 that is not covered by solution 18 is the notification to 5GC if the time synchronization status of a UE changes. The notification of time synchronization status of a UE is not in the scope of KI#3 but more relevant to KI#1. Therefore, it is proposed that solution #18 should be used as the basis for normative work.

- In order to provide continuous time synchronization service for targeted UEs, 5GC should be informed if the time synchronization status of those UEs change. The time status changes can be resulted from some unexpected cases, e.g. time sources failure or UE mobility.

Proposal 1: Solution #18 is selected for normative work.

# Proposal

It is proposed to add the following solution to TR 23.700-25.

*** Start of changes ***

## 8.X	Conclusion for KI#3: Support for controlling 5G time synchronization service based on subscription

The following principles summarize the solution baseline :

-	UE subscription data types stored in the UDM are extended to include  a new subscription data "Time Synchronization Subscription data" related to time synchronization service:

Editor’s note: it is FFS Whether UE time synchronization subscription data is reflected by option

- existing subscription data is updated + new subscription data type, or

- only in new subscription data type

-	“Access Stratum Time Synchronization Service Authorization” which indicate whether the UE is provisioned to receive RTI reception.

-	(Optionally) the Uu time synchronization error budget.

-	one or more “Subscribed time synchronization service ID(s)”, each containing the DNN/S-NSSAI and a reference to a PTP instance configuration pre-configured at the TSCTSF (e.g. PTP profile, PTP domain, etc).

Editor’s note: it is FFS whether the PTP instance configuration referenced by "Time Synchronization Subscription data" for a UE is pre-configured at TSCTSF or not.

-	(Optionally) Uu time synchronization error budget.

-	The AMF uses the " Time Synchronization Subscription data " in the following way:

-	The AMF retrieves the " Time Synchronization Subscription data " stored at the UDM during the registration procedure.

-	If the AMF receives Time Synchronization Subscription data Service Authorization from UDM, it provides 5G access stratum time distribution indication and Uu time synchronization error budget (if part of the Access Stratum Time Synchronization Service Authorization received from the UDM) to NG-RAN.

-	The TSCTSF uses the Time Synchronization Subscription data in the following way:

-	The TSCTSF retrieves the Time Synchronization Subscription data from the UDM when the TSCTSF receives an AF request for the time synchronization service (either ASTI or (g)PTP). According to in the Subscription data, the TSCTSF proceeds as specified in TS 23.502 [3].

-	The TSCTSF retrieves the Time Synchronization Subscription data from the UDM when the TSCTSF receives notification from the PCF that a UE has established a PDU Session that is potentially impacted by (g)PTP-based time synchronization service. The TSCTSF retrieves the PTP instance configurations referenced from the Subscribed time synchronization service ID(s).

-	The TSCTSF determines if one or more of the PTP instance configurations match with the DNN/S-NSSAI of the given PDU Session. If no PTP instance exists for the given PTP instance configuration, the TSCTSF initializes the PTP instance in 5GS as described in clause K.2.2 of TS 23.501 [2]. The TSCTSF configures a PTP port in DS-TT and adds it to the corresponding PTP instance in NW-TT as described in clause K.2.2 of TS 23.501 [2].

-

*** End of changes ***