SA WG2 Meeting #SA2-152E 	S2-2207750

17 – 26 August, 2022 Electronic meeting

Source:	China Mobile

Title:	Solution update for Sol#40

Document for:	Approval

Agenda Item: 	9.11

Work Item / Release:	FS_EDGE_Ph2

Abstract of the contribution: This document proposes to update the solution 40

- Discussion

- Proposal

It is proposed to capture the following aspects in TR 23.700-48.

# * * * * Start of change * * * *

6.40	Solution 40 (KI#5): EAS discovery for shared EHE

6.40.1	Introduction

This solution addresses Key Issue #5 and allows the discovery of an Edge Application Server (EAS) within a set of Operators sharing the EHE. It is assumed that the EAS is hosted by a different PLMN than the PLMN that is serving the UE, and the two PLMNs have IP connection at edge area.

6.40.2	Functional description

In this solution description it is assumed that the EAS is deployed in MNO2’s PLMN2 and that the UE is connected to MNO1’s PLMN1 (see Figure 6.40.2-1). The solution is based on the following two steps:

Step 1: Configuration Phase.

The MNO2 AF provides the DNS query filtering information for PLMN2 EHE as well as the EASDF Deployment information for PLMN2 described in Table 6.40.2-1 to EASDF1 in PLMN1 before the PDU Session establishment.

Step 2: After PDU session establishment.

When a DNS query from UE intents for a specific application at edge, but EASDF1 in PLMN1 finds that the application cannot be found in current PLMN1's edge platform but can be found in PLMN2's  DNS query filteringinformation, the EASDF1 will send (UE IP address, DNS handling rule) created by SMF1 to the selected EASDF2 through SEPP through control plane. Then EASDF1 forwards the DNS Query message to EASDF2 through user plane. EASDF2 then send the DNS query to DNS server in PLMN2 for EAS discovery.

DNS query filtering information includes the PLMN2’s supported EAS FQDN(s).

The EASDF2 is selected by EASDF1 based on UE location, EASDF2 serving area, EDI information for PLMN2 EHE, and the N6 Transmission requirement.

EASDF1 sends DNS query to EASDF2 through user plane. The DNS query is transmitted in the path of EASDF1->UPF1->UPF2->EASDF2. The UPF1 and UPF2 are local configured to support the DNS message transmission between two PLMNs.

The indication of supporting shared EAS discovery will be included in UE SM subscription data. SMF1 gets this information from UDM1 during the PDU session establishment. SMF1 may select the proper UPF(s) to support the DNS message transmission via two PLMNs. The UPF(s) in PLMN1which support access to PLMN2 domain may support to add DSCP value in IP layer to guarantee the transmission between two PLMNs.

MNO2 AF may create an External Group ID to show that these UE(s) supports the interaction between different PLMNs.

Figure 6.40.2-1: Architecture with Operator Platform

EASDF Deployment Information

EASDF1 finds EASDF2 based on EASDF Deloyment Information information including UE location, serving are of EASDF2 in PLMN2, N6 transmission requirement of PLMN2.

Table 6.40.2-1: EASDF Deployment Information

| Parameters | Description |
| --- | --- |
| PLMN ID(s) | Used to identify in which PLMN the EAS is located |
| EASDF FQDN(s) | FQDN(s) of EASDF in other PLMN. |
| Serving area(s) | Serving area where the EASDF can provides service. |
| External Group Identifier/Internal Group Identifier | Group ID for the EAS Deployment information. [optional] NOTE: The AF may provide External Group Identifier, and NEF can map the External Group Identifier into Internal Group Identifier according to information received from UDM. The PLMN2 provides external group identifier to PLMN1 thus PLMN1 can identify that the UE is allowed to access the EAS in PLMN2. |
| N6 Transmission requirement (e.g. DSCP) | Transmission requirements when packet transmit between two PLMNs. This parameter will be added by UPF of PLMN1 in the IP-layer when packet is sent out from PLMN1 to PLMN2. [optional] |

6.40.3	Procedures

UE

UPF

L-PSA

EASDF1

UPF1

EASDF2

2.DNS query

SMF

UPF PSA

UPF

UL CL/BP

UPF2

DNS server

1.step 1-6 described in TS 23.548 clause 6.2.3.2.2

3.Neasdf_DNSContext_Notify Request

4.Neasdf_DNSContext_Notify Response

5.Neasdf_DNSContext_Create Request

6.Neasdf_DNSContext_Create Response

7.DNS query

8.DNS query

9.DNS response

10.DNS response

11.Neasdf_DNSContext_Notify Request

12.Neasdf_DNSContext_Notify Response

13.UL CL/BP insertion, PSA relocation

14.Neasdf_DNSContext_Update Request

15.Neasdf_DNSContext_Update Response

16.DNS response

Figure 6.40.3-1: EAS discovery in shared EAS scenario

1. Step 1-6 is the same as EAS discovery procedure in clause 6.2.3.2.2 of TS 23.548 [3].

2. 	UE sends DNS query to EASDF1.

3. 	If EASDF1 finds the DNS Query message cannot be resolved by PLMN1's DNS server, but matches a PLMN2's FQDN(s) information, EASDF1 sends the DNS Query Message to SMF with providing indication that EASDF2 in PLMN2 should be selected.

NOTE:	When EASDF receives multiple DNS Query messages from one UE, if EASDF detects that these DNS Query messages are the same, EASDF will only send the Neasdf_DNSContext_Notify Request once to SMF for these DNS Query messages.

4.	The SMF responds with Neasdf_DNSContext_Notify Response.

5.	EASDF1 invokes Neasdf_DNSContext_Create Request (UE IP address, PLMN1 ID, notification endpoint, (DNS message handling rules)) to the selected EASDF2 in PLMN2.

The EASDF2 creates a DNS context for the PDU Session and stores the UE IP address, PLMN1 ID, notification endpoint, (DNS message handling rules) into the context.

The DNS handling rule is created by SMF and redirected to EASDF1 and EASDF1 sends this to EASDF2. In this handling rule, option A is used to forward the DNS message with the EDNS Client Subnet option that is updated to UE location information. Option B is not supported since the EASDF1 do not have DNS address in PLMN2.

6.	The EASDF2 invokes the service operation Neasdf_DNSContext_Create Response.

7.	EASDF1 sends DNS query to EASDF2 through user plane. The DNS query may pass though UPF1 which is able to access UPF2 in PLMN2. The DSCP valued can be added by UPF1 in DNS query IP layer to satisfy transmission requirement with low latency between two PLMNs.

8-9. the same as steps 12-13 in clause 6.2.3.2.2 in TS 23.548.

10.

EASDF2 returns DNS response (EAS IP address or FQDN) to EASDF1.

11-12. the same as steps 14-15 in clause 6.2.3.2.2 in TS 23.548.

13. The SMF may perform UL CL/BP and Local PSA selection and insert UL CL/BP and Local PSA.

14-16. the same as steps 17-19 in clause 6.2.3.2.2 in TS 23.548.

During PDU Session Release procedure, the SMF triggers the removing of the DNS context by invoking Neasdf_DNSContext_Delete service from EASDF1 to EASDF2.

6.40.4	Impacts on existing entities and interfaces

EASDF:

-	enhance to support to communicate with EASDFs in other PLMN to transform DNS handling rule and related information;

-	need to be configured with EASDF Deployment information to discovery other PLMN’s EASDF.

UDM:

-

UE authorization for EAS discovery via other PLMN’s EASDF

UPF:

- Enhance to support for add N6 Transmission requirement (e.g. DSCP) into IP-layer to ensure low latency.

AF:

-	can configure other PLMN's DNS query filtering information and EASDF deployment information in serving PLMN.

# * * * *end of change * * * *