3GPP TSG-SA WG2 Meeting #153E (e-meeting)	S2-2208171

# October 10th – 17th, 2022, Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | Draft | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Interworking with TSN network deployed in the transport network |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5TRS_URLLC |  |  |  |  |  | Date: |  |  | 2022-09-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Add the enhancements for interworking with TSN deployed in the transport network as concluded for KI#5 in TR 23.700-25. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 4.4.8.x provides architecture to support interworking with TSN deployed in transport network. Clause 5.27.x.1 describes the general principles to support interworking with TSN deployed in transport network. Clause 5.27.x.2 describes the Mapping of the parameters between 5GS and TSN UNI. Clause 5.27.x.3 describes the Transfer of TL-Container between SMF/CUC and AN-TL and CN-TL. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interworking with TSN deployed in the transport network is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.4.8.1, 4.4.8.x (new), 5.27.0, 5.27.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.203: "Policies and Charging control architecture; Stage 2".

[5]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS); Stage 2".

[6]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface: Stage 3".

[7]	IETF RFC 7157: "IPv6 Multihoming without Network Address Translation".

[8]	IETF RFC 4191: "Default Router Preferences and More-Specific Routes".

[9]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[10]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[11]	ITU-T Recommendation I.130: "Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN".

[12]	ITU-T Recommendation Q.65: "The unified functional methodology for the characterization of services and network capabilities".

[13]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS): Stage 3".

[14]	IETF RFC 3736: "Stateless DHCP Service for IPv6".

[15]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[16]	3GPP TS 22.173: "IMS Multimedia Telephony Service and supplementary services; Stage 1".

[17]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[18]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[19]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[20]	IETF RFC 7542: "The Network Access Identifier".

[21]	3GPP TS 23.002: "Network Architecture".

[22]	3GPP TS 23.335: "User Data Convergence (UDC); Technical realization and information flows; Stage 2".

[23]	3GPP TS 23.221: "Architectural requirements".

[24]	3GPP TS 22.153: "Multimedia priority service".

[25]	3GPP TS 22.011: "Service Accessibility".

[26]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[27]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

[28]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[29]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[30]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[31]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[32]	3GPP TS 23.214: "Architecture enhancements for control and user plane separation of EPC nodes; Stage 2".

[33]	3GPP TS 22.101: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Service aspects; Service principles".

[34]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[35]	3GPP TS 33.126: "Lawful Interception Requirements".

[36]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[37]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[38]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[39]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[40]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[41]	3GPP TS 32.240: "Charging management; Charging architecture and principles".

[42]	3GPP TS 38.401: "NG-RAN Architecture description".

[43]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[44]	IETF RFC 4960: "Stream Control Transmission Protocol".

[45]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[46]	3GPP TS 23.041: "Public Warning System".

[47]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[48]	3GPP TS 24.502: "Access to the 5G System (5GS) via non-3GPP access networks; Stage 3".

[49]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[50]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[51]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[52]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[53]	Void.

[54]	IETF RFC 4861: "Neighbor Discovery for IP version 6 (IPv6)".

[55]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[56]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[57]	IETF RFC 4555: "IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

[58]	3GPP TS 29.510: "5G System: Network function repository services; Stage 3".

[59]	3GPP TS 29.502: "5G System: Session Management Services: Stage 3".

[60]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2) ".

[61]	3GPP TS 23.380: "IMS Restoration Procedures".

[62]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[63]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services; Stage 2".

[64]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs".

[65]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[66]	3GPP TS 32.421: "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements".

[67]	3GPP TS 32.290: "5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[68]	3GPP TS 32.255: "5G Data connectivity domain charging; Stage 2".

[69]	3GPP TS 38.306: "NR; User Equipment -UE) radio access capabilities".

[70]	3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access -E-UTRA); User Equipment -UE) radio access capabilities".

[71]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[72]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[73]	IETF RFC 2865: "Remote Authentication Dial In User Service (RADIUS)".

[74]	IETF RFC 3162: "RADIUS and IPv6".

[75]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[76]	3GPP TS 26.238: "Uplink streaming".

[77]	3GPP TR 26.939: "Guidelines on the Framework for Live Uplink Streaming (FLUS)".

[78]	International Telecommunication Union (ITU), Standardization Bureau (TSB): "Operational Bulletin No. 1156"; http://handle.itu.int/11.1002/pub/810cad63-en (retrieved October 5, 2018).

[79]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[80]	3GPP TS 24.250: "Protocol for Reliable Data Service; Stage 3".

[81]	IETF RFC 8684: "TCP Extensions for Multipath Operation with Multiple Addresses".

[82]	IETF RFC 8803: "0-RTT TCP Convert Protocol".

[83]	IEEE Std 802.1CB-2017: "IEEE Standard for Local and metropolitan area networks-Frame Replication and Elimination for Reliability".

[84]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[85]	WiFi Alliance Technical Committee, Hotspot 2.0 Technical Task Group: "Hotspot 2.0 (Release 2) Technical Specification".

[86]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[87]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[88]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[89]	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

[90]	BBF TR-124 issue 5: "Functional Requirements for Broadband Residential Gateway Devices".

[91]	BBF TR-101 issue 2: "Migration to Ethernet-Based Broadband Aggregation".

[92]	BBF TR-178 issue 1: "Multi-service Broadband Network Architecture and Nodal Requirements".

[93]	BBF TR-456 issue 2: "AGF Functional Requirements".

[94]	BBF WT-457: "FMIF Functional Requirements".

Editor's note:	The reference to BBF WT-457 will be revised when finalized by BBF.

[95]	IEEE Std 802.1Qcc-2018: "IEEE Standard for Local and metropolitan area networks - Bridges and Bridged Networks - Amendment: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements".

[96]	Void.

[97]	IEEE Std 802.1AB-2016: "IEEE Standard for Local and metropolitan area networks -- Station and Media Access Control Connectivity Discovery".

[98]	IEEE Std 802.1Q-2018: "IEEE Standard for Local and metropolitan area networks--Bridges and Bridged Networks".

[99]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[100]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[101]	3GPP TS 29.274: "Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[102]	3GPP TS 23.632: "User Data Interworking, Coexistence and Migration; stage 2".

[103]	3GPP TS 29.563: "5G System (5GS); HSS services for interworking with UDM; Stage 3".

[104]	IEEE Std 802.1AS-2020: "IEEE Standard for Local and metropolitan area networks--Timing and Synchronization for Time-Sensitive Applications".

[105]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[106]	IEEE Std 802.11-2012: "IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[107]	IEEE Std 1588-2008: "IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems".

[108]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[109]	3GPP TS 24.193: "Access Traffic Steering, Switching and Splitting; Stage 3".

[110]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

[111]	3GPP TS 22.186: "Enhancement of 3GPP support for V2X scenarios; Stage 1".

[112]	3GPP TR 38.824: "Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)".

[113]	IEEE: "Guidelines for Use of Extended Unique Identifier (EUI), Organizationally Unique Identifier (OUI), and Company ID (CID)", https://standards.ieee.org/content/dam/ieee-standards/standards/web/documents/tutorials/eui.pdf.

[114]	3GPP TS 32.256: "Charging Management; 5G connection and mobility domain charging; Stage 2".

[115]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

[116]	3GPP TS 38.415: "PDU Session User Plane Protocol".

[117]	3GPP TS 24.535: "Device-side Time-Sensitive Networking (TSN) Translator (DS-TT) to network-side TSN Translator (NW-TT) protocol aspects; Stage 3".

[118]	3GPP TS 32.274: "Charging Management; Short Message Service (SMS) charging".

[119]	3GPP TS 23.008: "Organization of subscriber data".

[120]	3GPP TS 38.314: "NR; Layer 2 measurements".

[121]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[122]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[123]	3GPP TS 32.254: "Charging management; Exposure function Northbound Application Program Interfaces (APIs) charging".

[124]	3GPP TS 33.535: "Authentication and Key Management for Applications based on 3GPP credentials in the 5G System (5GS)".

[125]	3GPP TS 38.410: "NG-RAN; NG general aspects and principles".

[126]	IEEE Std 1588: "IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems", Edition 2019.

[127]	ST 2059-2:2015: "SMPTE Standard - SMPTE Profile for Use of IEEE-1588 Precision Time Protocol in Professional Broadcast Applications".

[128]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[129]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

[130]	3GPP TS 23.548 "5G System Enhancements for Edge Computing; Stage 2".

[131]	IEEE Std 802.3: "Ethernet".

[132]	3GPP TS 29.561: "5G System; Interworking between 5G Network and external Data Networks; Stage 3".

[133]	3GPP TS 29.513: "Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

[134]	3GPP TS 23.558: "Architecture for enabling Edge Applications (EA)".

[135]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[136]	3GPP TS 23.256 "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[137]	GSMA NG.116: "Generic Network Slice Template".

[138]	IETF RFC 3948: "UDP Encapsulation of IPsec ESP Packets".

[139]	3GPP TS 24.539: "5G System (5GS); Network to TSN translator (TT) protocol aspects; Stage 3".

[140]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic bootstrapping architecture".

[141]	3GPP TS 33.223: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA) Push function".

[142]	3GPP TS 23.540: "Technical realization of Service Based Short Message Service; Stage 2".

[143]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[144]	3GPP TS 29.525: "5G System; UE Policy Control Service; Stage 3".

[145]	3GPP TS 29.505: "5G System; Usage of the Unified Data Repository Services for Subscription Data; Stage 3".

[x]	IEEE P802.1Qdj-d0.3: "IEEE Draft Standard for Local and metropolitan area networks - Bridges and Bridged Networks - Amendment XX: Configuration Enhancements for Time-Sensitive Networking".

*** NEXT CHANGE ***

#### 4.4.8.1	General

The 5G System can be extended to support the following:

a)	Integration as a bridge in an IEEE 802.1 Time Sensitive Networking (TSN). The 5GS bridge supports the Time sensitive communication as defined in IEEE 802.1 Time Sensitive Networking (TSN) standards. The architecture is described in clause 4.4.8.2.

This Release supports of the specification, integration of the 5G System with IEEE 802.1 TSN networks that apply the fully centralized configuration model as defined in IEEE Std 802.1Qcc [95]. IEEE TSN is a set of standards to define mechanisms for the time-sensitive (i.e. deterministic) transmission of data over Ethernet networks.

b)	Enablers for AF requested support of Time Synchronization and/or some aspects of Time Sensitive Communication. The architecture is described in clause 4.4.8.3.

c)	Enablers for interworking with TSN network deployed in the transport network. This option can be used simultaneously with either option a) or b). The architecture is described in clause 4.4.8.x. The interworking is applicable when the transport network deploys the fully centralized configuration model as defined in IEEE Std 802.1Qcc [95].

*** NEXT CHANGE ***

#### 4.4.8.x	Architecture to support interworking with IEEE Time Sensitive Networking deployed in the transport network

This clause describes the architecture to enable the 5GS to control the IEEE TSN features deployed in the transport network (TN) for N3 tunnels. The architecture can be used when the underlay transport network for N3 tunnels supports IEEE TSN and deploys a CNC. Figure 4.4.8.x-1 shows the architecture to support interworking with IEEE TSN in transport network.

Figure 4.4.8.x-1: Architecture to support interworking with IEEE TSN in transport network

NOTE 1:	The TN CNC in transport network (TN) is independent from the CNC in the DN.

NOTE 2: The rest of the architecture follows either clause 4.4.8.2 or 4.4.8.3, depending on whether option a) or b) in clause 4.4.8.1 is used.

NOTE 3: When option b) in clause 4.4.8.1 is used, both Ethernet and IP type PDU Sessions are supported.

The 5G System is integrated with the transport network and is acting as a TSN CUC towards the CNC in transport network (TN CNC). The CUC implements the UNI as defined in IEEE P802.1Qcc [95]. The CUC is collocated with the SMF(s) that are serving the corresponding PDU Session(s). The SMF/CUC is responsible to derive the 5GS parameters that are mapped from/to the parameters in UNI in IEEE P802.1Qcc [95] as enhanced by IEEE P802.1Qdj [x]. The mapping of 5GS parameters and parameters in UNI in IEEE P802.1Qdj [x] is described in clause 5.27.x.

NG-RAN may include AN-TL function and UPF may include CN-TL function to support acting as a TSN Talker and Listener (End Station) as defined in IEEE P802.1Qcc [95] for the N3 tunnel interface. The SMF/CUC communicates with the AN-TL and CN-TL via TL-Container that conveys the IEEE Std 802.1Qcc [6] data sets in N2 interface (via N11 interface) and in N4 interface, respectively.

It is assumed that NG-RAN, 5GC and transport network are time synchronized with each other using the 5GS internal system clock.

*** NEXT CHANGE ***

### 5.27.0	General

This clause describes 5G System features that can be used independently or in combination to enable time-sensitive communication and time synchronization:

-	Delay-critical GBR;

-	A hold and forward mechanism to schedule traffic as defined in IEEE Std 802.1Q-2018 [98] for Ethernet PDU Sessions in DS-TT and NW-TT (see clause 5.27.4) to de-jitter flows that have traversed the 5G System if the 5G System is to participate transparently as a bridge in a TSN network;

-	TSC Assistance Information: describes TSC flow traffic characteristics as described in clause 5.27.2 that may be provided optionally for use by the gNB, to allow more efficiently schedule radio resources for periodic traffic and applies to PDU Session type Ethernet and IP.

-	Time Synchronization: describes how 5GS can operate as a PTP Relay (IEEE Std 802.1AS [104]), as a Boundary Clock or as Transparent Clock (IEEE Std 1588 [126]) for PDU Session type Ethernet and IP.

-	Interworking with IEEE TSN deployed in the transport network: describes how 5GS can operate as a CUC (IEEE Std 802.1Qcc [95]) towards the CNC in the transport network.

The 5G System integration as a bridge in an IEEE 802.1 TSN network as described in clause 5.28 can make use of all features listed above.

To support any of the above features to enable time-sensitive communication and time synchronization, during the PDU Session establishment, the UE shall request to establish a PDU Session as an always-on PDU Session, and the PDU Sessions are established as Always-on PDU session as described in clause 5.6.13. In this release of the specification, to use any of the above features to enable time-sensitive communication and time synchronization:

-	Home Routed PDU Sessions are not supported;

-	PDU Sessions are supported only for SSC mode 1;

-	Service continuity is not supported when the UE moves from 5GS to EPS .i.e. interworking with EPS is not supported for a PDU Session for time synchronization or TSC.

*** NEXT CHANGE ***

### 5.27.x	Interworking with IEEE TSN deployed in the transport network

#### 5.27.x.1	General

When the 5GS supports interworking with IEEE TSN deployed in the transport network, the CUC that is collocated with SMF interworks with the CNC in the transport network (TN CNC). The SMF/CUC provides the merged stream requirements on QoS Flow basis (i.e. translated Talker group and Listener group information) as specified in clause 46.2 of IEEE P802.1Qcc [95] via the User/Network-Interface (UNI) to the TN CNC. The TN CNC uses the merged stream requirements as input to select respective path(s) and calculate schedules in TN. Based on the results, the TN CNC provides a Status group that contains the merged end station communication-configuration back to the SMF/CUC.

If the NG-RAN and UPF support the TSN Talker and Listener functionality (i.e. implement the AN-TL and CN-TL, respectively), the SMF/CUC can communicate with the AN-TL and CN-TL via TL-Container. The TL-Container conveys the data sets defined in IEEE P802.1Qdj [x] between the SMF/CUC and AN-TL and CN-TL.

The AN-TL and CN-TL enable the following functions:

a)	hold and buffer functionality in a case when the TSCAI contains a BAT in UL and/or DL direction.

b)	support of stream transformation as described in clause 46.1.4 in IEEE Std 802.1Qcc [95].

c)	for SMF/CUC to retrieve the InterfaceCapabilities and/or EndStationInterfaces from the AN-TL or CN-TL.

#### 5.27.x.2	Mapping of the parameters between 5GS and TSN UNI

The details of the parameters in the TSN UNI are described in IEEE Std 802.1Qcc [95]) as enhanced by IEEE Std 802.1Qdj [x].

The SMF/CUC derives the End Station related information for the merged stream requirements towards the TN CNC for the QoS Flow as follows:

a) For the Talker group:

‐	StreamID: set based on pre-configuration, shall be set to a unique value per a QoS Flow. The SMF/CUC can derive the value based on the PDU Session ID and QFI.

-	StreamRank: set to zero for ARP priority values 1-8; set to one for other ARP values.

‐	EndStationInterfaces: If the AN-TL and CN-TL are supported, the SMF/CUC receives the EndStationInterface (MacAddress, InterfaceName) from the AN-TL and CN-TL via TL-Container. If the AN-TL and CN-TL are not supported the SMF/CUC sets the information based on pre-configuration.

-	DataFrameSpecification (optional): If the AN-TL and CN-TL are supported, the SMF/CUC does not include the DataFrameSpecification to the TN CNC. 

If the AN-TL and CN-TL are not supported, the SMF/CUC derives the DataFrameSpecification based on the N3 tunnel end point addresses/ports that are used for the QoS Flow. The SMF/CUC instruct the UPF and NG-RAN to assign a separate N3 tunnel end point address for each QoS Flow. This ensures the TN can distinguish the QoS Flows based on the N3 tunnel destination IP addresses.

NOTE 1: IPv6 can be used in the N3 tunnel end point addresses to provide sufficient address space.

If the DataFrameSpecification is provided to the TN CNC, the TN CNC configures the edge bridge to perform the stream transformation based on the provided the DataFrameSpecification. In this case the TN CNC provides the configuration of the interfaces for the Talker and Listener in the InterfaceConfiguration in the Status group.

Editor's Note: It is FFS if the interface between the SMF/CUC and TN CNC could allow the SMF/CUC to indicate the TEID and QFI of the given QoS Flow to the TN CNC.

‐	In TrafficSpecification elements:

‐	Interval: derived from the Periodicity of the traffic as indicated in the TSCAI.

‐	MaxFramesPerInterval: set to 1.

‐	MaxFrameSize: derived from the Burst Size of the QoS Flow. The PCF transfers the Burst Size to the SMF/CUC. The SMF/CUC considers the framing bits which is not used for transferring in 5GS, (e.g. CRC), and the GTP-U tunnel overhead, when setting the MaxFrameSize.

-	TransmissionSelection: is set to zero.

‐	TSpecTimeAware group (optional, present only if the traffic in the QoS Flow is time-synchronized):

‐	EarliestTransmitOffset: the earliest offset within the Interval.

For uplink, EarliestTransmitOffset should be set based on the following formula:

TSCAC BAT in UL direction (presented in TAI time and corrected for clock drifting as specified in TS 23.501) + UE-DS-TT Residence Time + 5G-AN PDB - M x Interval, where M is the largest integer for which the relation:

TSCAC BAT in UL direction + UE-DS-TT Residence Time + 5G-AN PDB > M x Interval duration.

would be true.

For downlink, EarliestTransmitOffset should be set based on the following formula:

TSCAC BAT in DL direction (presented in TAI time and corrected for clock drifting as specified in TS 23.501) + UPF Residence Time - M x Interval, where M is the largest integer for which the relation:

TSCAC BAT in DL direction + UPF Residence Time > M x Interval duration.

would be true.

‐	LatestTransmitOffset: the last chance within an interval should leave enough time to transfer a packet with MaxFrameSize. Derived from the EarliestTransmitOffset plus the interval, subtracted by the sum of jitter and the time to transfer a packet with MaxFrameSize.

‐	Jitter: derived based on local configuration.

‐	UserToNetworkRequirements:

‐	NumSeamlessTrees: set to one (no redundancy) or other value (if redundancy is required).

‐	MaxLatency: set to CN PDB subtracted by UPF Residence Time.

-	InterfaceCapabilities (optional): If the AN-TL and CN-TL are supported, the SMF/CUC collects InterfaceCapabilities from AN-TL and CN-TL via TL-Container. If the AN-TL and CN-TL are not supported, the SMF/CUC leaves the InterfaceCapabilities empty.

b) For the Listener group:

-	Stream ID: derived as with the corresponding information for the Talker group.

-	EndStationInterfaces: derived as with the corresponding information for the Talker group.

- 	UserToNetworkRequirements:

‐	NumSeamlessTrees: set to one.

‐	MaxLatency: derived as with the corresponding information for the Talker group.

-	InterfaceCapabilities: derived as with the corresponding information for the Talker group.

c) For the Status group: The Status group contains the merged end station communication-configuration provided by TN CNC to the SMF/CUC:

‐	Stream ID.

‐	StatusInfo.

‐	AccumulatedLatency: If the AccumulatedLatency is included from TN CNC to SMF/CUC for a stream in DL direction, the SMF/CUC should use the AccumulatedLatency to update the TSCAI BAT to the NG-RAN; the SMF sets the TSCAI Burst Arrival Time in downlink direction as the sum of the TSCAC BAT value in downlink direction and AccumulatedLatency and UPF Residence Time.

‐	InterfaceConfiguration (optional):

‐	MAC Address (optional, present only if stream transformation is performed).

‐	VLAN Tag (optional, present only if stream transformation is performed).

‐	IPv4/IPv6 Tuples (optional, present only if stream transformation is performed).

‐	TimeAwareOffset (optional, present only if the traffic is time-synchronized).

If the InterfaceConfiguration is included from TN CNC to SMF/CUC, and if the AN-TL/CN-TL are supported, and AL-TL/CN-TL support the Stream Transformation as described in IEEE 8021.Qdj [10], the SMF/CUC can instruct the UPF and NG-RAN to assign for each QoS Flow an individual TSN Transport address by providing the InterfaceConfiguration to the AN-TL/CN-TL via TL-Container. The Talker in AN-TL/CN-TL shall use the indicated InterfaceConfiguration, e.g. source MAC address, multicast destination MAC address, VLAN ID, or IP-tuples as assigned by the TN CNC for the data stream in a QoS Flow. The TN can identify the streams based on the Stream Transformation that is applied in the Talker in the AN-TL/CN-TL. This allows to use a single GTP-U tunnel as defined for non-TSN Transport networks.

If the TimeAwareOffset is included from TN CNC to SMF/CUC, the SMF/CUC should configure the Talker in the AN-TL (for streams in UL direction) and in the CN-TL (for streams in the DL direction) based on the TimeAwareOffset. The SMF/CUC adjusts the TimeAwareOffset to be relative from the start of the BAT before sending it to the AN-TL or CN-TL. The AN-TL and CN-TL should set the sending time of the traffic (e.g. AdminBaseTime and OperBaseTime in 802.1Qbv data sets) in the transport layer accordingly. The Talker in the NG-RAN or UPF buffers the data burst until the time indicated in the TimeAwareOffset is reached.

If the SMF/CUC receives a TimeAwareOffset from TN CNC for a downlink stream (i.e. for a Talker in the UPF/CN-TL), the SMF/CUC adds the received TimeAwareOffset value to the TSCAI BAT in the DL direction in the TSCAI and updates the NG-RAN for the new TSCAI.

‐	FailedInterfaces (optional).

NOTE 2:	It is assumed that the merged end station communication-configuration will contain at least the same information as defined for the status.

#### 5.27.x.3	Transfer of TL-Container between SMF/CUC and AN-TL and CN-TL

The SMF/CUC may use the TL-Container to send a:

1)	get-request.

2) 	set-request: submits the following elements of the Status group to the AN-TL or CN-TL:

-	list of InterfaceConfiguration(s) (one InterfaceConfiguration is associated with each QFI in the N3 tunnel)

The AN-TL or CN-TL may use the TL-Container to send a:

1)	get-response: indicates the following elements of the Talker or Listener group from the AN-TL or CN-TL:

-	EndStationInterfaces: list of InterfaceIDs, one InterfaceID is associated with each Local F-TEID

-	InterfaceCapabilities

2) 	set-response: acknowledges the reception of the Status group to the SMF/CUC.

The SMF may request the UPF to report the support for AN-TL or CN-TL by including a TL-Container with a get-request to the AN-TL or CN-TN, respectively. The get-request is sent to AN-TL in the N2 SM information and to CN-TL in the N4 Session Establishment or Modification request as described in clause 4.3.2.2 in TS 23.502 [3].

If the NG-RAN/UPF supports AN-TL/CN-TL, the NG-RAN/AN-TL or UPF/CN-TL responds with a TL-Container including the elements defined for the get-response as applicable. The elements are associated with the corresponding N3 Tunnel Info (Local F-TEID) in the NG-RAN or UPF.

The SMF may submit TL-Container including a set-request the elements defined for the set-request to NG-RAN/AN-TL or UPF/CN-TL. The set-request is sent to AN-TL in the N2 SM information and to CN-TL in the N4 Session Establishment or Modification request as described in clause 4.3.2.2 in TS 23.502 [3]. The InterfaceConfiguration is associated with the corresponding QFI in the N3 tunnel in the NG-RAN or UPF, respectively. The AN-TL or CN-TL uses the provided configuration for the traffic in the QoS Flow of the given QFI as described in clause 5.27.x.2.

*** END OF CHANGES ***