3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208317

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#4, Update conclusion on KI#4

Document for:	Approval

Agenda Item:	9.4.1

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract: The conclusion on KI#4 of FS_eNPN_Ph2 about how to enable UE to discover, select and access NPN as hosting network and receive localized services.

1. Introduction/Discussion

This contribution would like to further update the conclusion on KI#4 of FS_eNPN_Ph2 to provide principles for solving how UE discover, select and access to hosting network and receive localized services according to the service conditions. Since the mechanism of network selection can be divided into automatically network selection and manual network selection, the following principles are provided corresponding to these two mechanisms. In addition, the service conditions can be location and/or time. Therefore, the principles to ensure the UE accesses to the localized services based on service conditions are related to location and time respectively.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-08.

# * * * * First change * * * *

8.4	Key Issue #4: Enabling UE to discover, select and access NPN as hosting network and receive localized services

The mechanism to enable UE to discover, select and access NPN as hosting network shall follow the principles listed below:

-	For automatic network selection: the hosting network selection information shall be provided by UE’s home network via SoR procedure.

-	The hosting network selection information (e.g. hosting network identifier or human readable name of hosting network) provided by localized service provider via user plane shall only be used for manual network selection.

The mechanism to ensure the localized services are accessed by UE according to the validity conditions shall follow the principles listed below:

-	If validity conditions include location restriction, AMF determines the location of UE based on the validity conditions (cell or TAI) and report it to SMF;

-	If validity conditions include time restriction, SMF instructs UPF to stop transmitting the localized services to the UE when outside the time when localized services are allowed.

# * * * * End of changes * * * *