3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208389

Elbonia, October 10 – 17, 2022

Source:	Huawei, HiSilicon

Title:	KI#7:Evaluation and Conclusion

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR / Rel-18

Abstract: Evaluation and Conclusion for KI#7: Control of UE's access to 5GS via a mobile base station relay.

1. Discussion

This contribution proposes the evaluation and conclusion for KI#7.

2. Text Proposal

It is proposed to capture the following changes into TR 23.700-05 V1.0.0.

# * * * * First change * * * *

7	Evaluation

Editor's note:	This clause provides the evaluations of the solutions of clause 6.

7.X  Evaluation for KI#7

The Solution#20 mapping to KI#7 is targeting at using the existing CAG, Closed Access Group, concept as a base to manage the UE access control via MBSR. In addition, if the MBSR is pre-configured with the PLMN list in which the MBSR is allowed to operate as an IAB node, the corresponding CAG Identifier per PLMN is also configured in the MBSR. The UE CAG configuration per PLMN follows the legacy mechanism as defined in the TS 23.501[2].

RAN and CN supports the UE access control based on the CAG identifier associated with the cell and the allowed CAG identifiers for the UE. This is as legacy mechanism defined in the TS 23.501[2]. The legacy UE(s) also shall support to access the MBSR cell based on the CAG identifier.

# * * * * Second change * * * *

8	Conclusions

Editor's note:	This clause provides the conclusions for the study.

8.Y  Conclusion for KI#7

For Key Issue #7 (Control of UE's access to 5GS via a MBSR), the followings can be taken as initial conclusion:

- When the MBSR is allowed to be operated as an IAB node for a PLMN, the MBSR is configured with CAG identifier which is unique within the scope of this PLMN. If the MBSR is pre-configured with the PLMN list in which the MBSR is allowed to operate as an IAB node, the corresponding CAG Identifier per PLMN is also configured in the MBSR.

- The UE CAG configuration per PLMN follows the legacy mechanism as defined in the TS 23.501 [2].

- RAN and CN supports the UE access control based on the CAG identifier associated with the cell and the allowed CAG identifiers for the UE.

- The legacy UE(s) shall also support to access the MBSR cell based on the CAG identifier.

# * * * * End of changes * * * *