3GPP TSG-SA WG2#153E e-meeting	S2-2208401

Elbonia, 10–17 October 2022

Source:	Ericsson

Title:	KI#2, Sol#7: Evaluation of the deferred 5GC-MT-LR-based approach

Document for:	Approval

Agenda Item:	9.7.1

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract of the contribution: This paper discusses the latest solution updates that propose to rely on the “Deferred 5GC-MT-LR” procedure to solve KI#2 in TR 23.700-25.

Propose to include in the TR:

# (All new text)

# 7.x.	Evaluation: UE’s presence in Requested Coverage Area obtained via the Deferred 5GC-MT-LR procedure triggered by TSCTSF

An alternative approach on how UE’s presence in the Requested Coverage Area can be determined was agreed after conclusion reached.

The approach relies on the TSCTSF triggering the deferred 5GC-MT-LR procedure towards the GMLC for the type of events assigned to an Area. Specifically, the Area is set to the Requested Coverage Area as requested by the AF at the invocation of a time synchronization service. The GMLC, after receiving a Ngmlc_Location_ProvideLocation request from the TSCTSF, invokes the deferred 5GC-MT-LR procedure as described in clause 6.3.1 of TS 23.273 [16] to determine and report location events, indicating the UE moving IN or OUT of the Request Coverage Area. However, the deferred 5GC-MT-LR procedure imposes certain capabilities/functionalities to be supported by AMFs and by UEs, which is not always the case:

- AMF needs to support a deferred location request (“If the AMF supports a deferred location request, the AMF returns an acknowledgment to the external LCS client, via the (H)GMLC and, in the case of roaming, …”see steps 6-8 in clause 6.3.1 of TS 23.273 [Ref. 3])

- UE needs to support the periodic and triggered location request (“If the UE cannot support the periodic and triggered location request, the service operation returned to the AMF shall include a suitable error cause,” see step 18 in clause 6.3.1 of TS 23.273 [Ref.3]).

Additionally, although it is specified that the procedure can be used to report events when a UE changes a cell or a TA (“For area event reporting, the target geographical area is converted into a corresponding list of cell and/or tracking area identities.”), it introduces complexity and UE dependency, which is uncessary for the granularity on a cell/TA level.

Furthermore, the description of this approach itself contains flaws that may be ambiguous. For instance,

- clause 6.7.3.2a claims to provide the procedure for 5G Access Stratum based time synchronization services, however, the request flows as well as their descriptions use (g)PTP service operations;

- higher signaling load as the procedure requires the signaling towards GMLC and LMF as well as extra requests towards UDM compared to the procedure exploiting “UE mobility event notification” instead of the deferred 5GC-MT-LR procedure; there is no reference point between TSCTSF and GMLC defined in Rel-17 specifications;

- Figure 6.7.3.1a-1 and Figure 6.7.3.2a-1 reference Steps 2-20 in Figure 6.7.3.1-1 of TS 23.273 [16] v17.5.0 , but there is no such figure, and even clause 6.7.3.1 does not exist in that specification;

- Other issues, e.g., a claim that time synchronization services are provided by TSN AF.

Considering all the above, the solution alternative relying on the deferred 5GC-MT-LR procedure shall not be selected as a way forward for the solution of this KI and, therefore, this alternative shall not be reflected in the conclusion.