SA WG2 Meeting #S2-153E	S2-2208444

10 – 17 October. 2022, Electronic, Elbonia	(revision of S2-220xxxx)

Source:	MediaTek Inc.

Title:	KI#4: Evaluation and interim conclusions on automatic Hosting network selection

Document for:	Approval

Agenda Item: 	9.4.1

Work Item / Release:	FS_eNPN_Ph2/Rel-18

Abstract of the contribution: This paper proposes the evaluation and interim conclusions on KI#4

1	Proposal

It is proposed to adopt the following proposal into TR 23.700-08.

* * * * 1st Change * * * *

7	Evaluation

7.4	Key Issue #4: Enabling UE to discover, select and access NPN as hosting network and receive localized services

Editor's note:	The evaluation in this clause has to be updated to consider the solutions agreed during the SA2#152 meeting.

7.4.1	General

The evaluation for KI #4 solutions is made for each component listed below:

-	The content of the localized service information that is used by UE to perform discovery/selection/access to the hosting NW for localized services.

-	From where and how UE obtains such information.

-	How the localized service information is used by UE.

Furthermore, since Hosting Network based on the architecture assumption and requirements in clause 4 as below

-	Hosting network can be NPN, i.e. SNPN, PNI-NPN

-	Home network can be NPN or PLMN

If the UE accesses the Hosting network using the subscription/credentials from the UE Home network, there are only two scenarios considered in such case:

-	If the Home network of the UE is PLMN, the Hosting network could be PNI-NPN or SNPN.

-	If the Home network of the UE is SNPN, the Hosting network could be only SNPN.

The case the Home network of the UE is SNPN and the Hosting network is PNI-NPN is not existed according to below SA1 requirement in clause 6.41.2 of TS 22.261 [2]:

-	Only subscribers of a public network can roam into a PLMN.

Furthermore, if the UE has multiple hosting subscriptions/credentials, that which subscription/credential to use for automatic Hosting network selection is determined by implementation specific prior to network selection as same logic specified in clause 5.30.2.4 of TS 23.501 [3]

7.4.2	Evaluation for the content of the localized service information

Construct the information on localized service basis (Sol #11, #12, #13, #15, #24, #27, #31, #32, #34).

-	The information includes an element to distinguish different localized service (e.g. identifier, name of the localized service), validity conditions (time/location), associated hosting networks' information (e.g. hosting network identifier, temporary credential or credential type for accessing hosting network, CAG IDs, QoS, cost, information transmitted in the Hosting network's SIB, etc).

-	The localized service information may include either (a) a list of one or more Hosting Network Identifiers, or (2) information which is broadcasted by the Hosting Network (e.g. CAG IDs, localized service ID) used by the UE to select the Hosting network.

NOTE:	Credential provisioning mechanisms need to be evaluated by SA WG3.

-	The benefit of this principle is to provide UE/end user with relationship between localized services and hosting networks, so that it enables the possibility for UE/end user to make request on specific localized service and for network to inform UE what are the available localized services and over which hosting networks these localized services are provided.

Construct the information based on priority list for Hosting network selection (Sol #10, #14, #18, #25, #29, #30, #33, #36).

-	The information is a new priority list for Hosting network selection. Each entry on the list includes hosting network identifiers, validity conditions (time/location), etc.

-	This principle requires other solution components (e.g. Sol #12, #13, #24, #30) to complement, since the information does not include reference to the localized services available over the different hosting networks included in the priority list and therefore it assumes that either UE or the network providing this information knows beforehand which localized service end user intends to access or what are the localized services supported by the hosting network on the priority list.

The localized service information in Sol #22 and Sol #46 provided by hosting network to UE, is not used for selecting hosting networks (instead the UE has registered to a hosting network based on a prioritized list of Hosting network), but rather for UE to know how to access a service in the hosting network which can be addressed by existing functionality, such as URSP rules.

7.4.3	Evaluation for from where and how UE obtains the localized service information

UE can obtain the localized service information from application server, serving network, home network, or hosting network.

UE fetches the information from an application server (Sol #14, #28, #31, #40).

-	When a full set of new subscription/credential is provided to UE via application server, this principle has less system impact on 5GS. But on the other hand, home network subscription/credential is utilized for the UE to establish PDU connection with the application server and the UE needs to handle co-existence of multiple subscriptions/credentials on device.

-	When the information includes a list of hosting networks and an indication that home network credential is to be used, there will be impact on network selection. Application server obtaining such information via network exposure is not necessary, since it is covered as part of the SLA between localized service provider and hosting network operator.

UE fetches the information from home network (Sol #10, #13, #14, #15, #18, #23, #24, #29, #30, #31, #33, #36, #43).

-	This principle assumes that home network is capable of managing the localized service information data.

-	The impact is about how home network is provisioned with the localized service information data, and how UE is provisioned by the home network.

-	Most of the solutions following this principle propose that the home network provides the localized service information to UE via SoR procedure in combination with the principle that the localized service information is constructed based on a priority list for (hosting) network selection as described in clause 7.4.2.

-	Some solutions assumed that the SoR-AF within the home network should be involved as the data source and the mechanism regarding how SoR-AF obtains such data should be out of 3GPP scope.

-	Since SoR procedure is mainly used for network discovery and selection, it is not suitable to be extended to provision UE with data more than required for the UE's actual network discovery and selection.

-	Sol #24 proposes another variant for UE to fetch the information from home network, that is via home network UDR and new UE policy. UE policy has the potential to include necessary data of the localized service, and possibility to be extended in the future. The new UE policy can be delivered to UE by PCF in home network, or PCF in serving network using N24.

UE fetches the information from serving network (Sol #11, #12, #24, #27, #32, #42, #44).

-	This principle assumes serving network (either home network or another network which UE is accessing using home network credential) is capable of storing the localized service information data.

-	The impact is about how serving network is provisioned with the localized service information data, and how UE is provisioned by the serving network.

-	Some solutions following this principle proposes to use AMF/SMF in serving network to store the localized service information data. In this case, the provisioning to serving network is based on configuration which has less impact, but it requires the UE is served by the correct AMF/SMF configured with such information (e.g. UE is located in the overlapping area where the hosting network is available) or requires all the AMF/SMF in the serving network are configured with such information. The data is provisioned to UE via MM NAS or SM NAS message (it is TBD which procedures/messages are impacted).

-	Other solutions following this principle, proposes to use UDR in the serving network to store the localized service information data as application data for any UE registered to the serving network. This enables the flexibility that if the serving network is not the home network, UE can obtain localized service information data from either the serving network, or from the home network, or from both. The impact is to extend service specific parameter provisioning for the serving network to receive the data (see TS 23.502 [4] clause 4.15.6.7) and new UE policy to provide the data to the UE.

UE fetches the information from hosting network (Sol #22, #26, #34, #41).

-	This principle assumes a hosting network is selected prior to a localized service is selected, since the localized services information arrives later from the hosting network. It can happen that the fetched information is not interesting for the end user.

-	The impact is to extend broadcast with capability for localized service, new on-demand SIB, or code for URL. Another impact is to deliver localized service information via registration reject/accept message.

NOTE:	Privacy aspect needs to be evaluated by WG SA3.

-	This principle implies that in some scenarios the UE needs to suspend the connection with serving network when getting information from hosting network, which can cause service interruption.

There are different triggers to provision the information to UE:

-	UE request as trigger (Sol #11, #13, #15, #24, #27, #32, #34).

-	UE location as trigger (Sol #11, #12, #27, #32).

-	UE subscription data change via external parameter provisioning as trigger (Sol #14, #23).

-	UE Steering of Roaming information update (SoR based solutions).

UE request as trigger assumes UE/end user is aware of the desired localized services. UE location as trigger requires the serving network is aware of which hosting network is available in which serving network tracking areas, and the information sent to UE is unsolicited. UE subscription data change and SoR based solutions restrict the information source to home network only.

7.4.4	Evaluation for how the localized service information is used by UE

New network selection mode is proposed (Sol #10, #25, #30, #33), where UE selects only hosting network when it intends for localized services and bypasses or temporary stops existing network selection procedure. This is needed in case UE is using home network credential to access hosting network, since home network and hosting network could be collocated in the same area and UE will always prioritize home network based on existing network selection mechanism. The new network selection mode can be requested by UE and authorized by home network (Sol #25).

The time and location validity condition along with other localized service information as described in clause 7.4.2. provided to UE can be used as a criteria for the UE to determine when and where to enter the new network selection mode. Furthermore, by SA1 requirement in Clause 6.41.2.2 of TS 22.261 [2] specifies:

-	Subject to regulatory requirements and localized service agreements, the 5G system shall allow a Home Network operator to automatically negotiate policies with the Hosting Network for allowing the Home Network's subscribers to connect at a specific occasion, e.g. time and location, for their Home Network services.

the UE is *allowed* by Home Network to select a Hosting Network, i.e. the UE is *not enforced* to select a Hosting Network. Therefore, the UE can decide (e.g. UE implementation) whether it stays in current serving network or it selects a Hosting Network.

Except the validity conditions, other methods for UE to determine whether to enter hosting network selection mode are also proposed, e.g. serving network to indicate to UE the presence of hosting network via either NAS or broadcast (Sol #12, #29, #30), , which requires extra system impacts.

8	Conclusion

8.4	Interim Conclusion on KI#4

8.4.1	General

When UE accesses the Hosting network using the subscription/credentials of its Home network, only two cases are considered:

-	If Home network is PLMN, the Hosting network can be PNI-NPN or SNPN.

-	If Home network is SNPN, the Hosting network can be only SNPN

If the UE accesses the Hosting network using the other credentials rather than the subscription/credentials from the UE Home network, the determination of the subscription used to access the Hosting network is by implementation specific prior to automatic network selection as described in NOTE 1 of clause 5.30.2.4.2 of TS 23.501 [3].

8.4.2	The content of the localized service information

8.4.2.1	The content of the localized service information for automatic Hosting network selection

If the Hosting network is an SNPN, a new prioritized list is considered and includes:

A new prioritized list for Hosting network selection is considered.

-	SNPN Hosting network identifier (i.e., PLMN ID+NID)

-	validity information (time and/or location)

If Hosting network is PNI-NPN, the allowed CAG list additionally contain the validity information (time and/or location)

8.4.3	The provisioning of the localized service information

8.4.3.1	The provisioning for automatic Hosting network selection

The localized service information used by the UE for automatic network selection can be only provisioned by UE Home network using SoR mechanism. The content of this type of the localized service information can be referred to clause 8.4.2.1.

8.4.4	The usage of the localized service information

8.4.4.1	The usage for automatic Hosting network selection

If the Hosting network is SNPN, when the validity information is met:

-	The UE may initiate the automatic network selection for Hosting network based on the list in clause 8.4.2.1. As long as the validity information is met, the UE can switch between the normal automatic selection (i.e. PLMN selection or SNPN selection) and Hosting network automatic selection. How the UE switches among the automatic network selections is up to UE implementation.

If the Hosting network is PNI-NPN, when the validity information is met:

-	The UE applies the existing mechanism with further considering those CAG cells in which the validity information is met. This may potentially initiate a new registration procedure to a PLMN

If the validity information is no longer met, the UE is not allowed to perform automatic Hosting network selection, i.e. the UE is only allowed to perform normal automatic network selection (i.e. PLMN selection or SNPN selection)

* * * * End of Changes * * * *