SA WG2 Meeting #153e	S2-220xxxx

# October 10th – October 17th, Elbonia               		     (Revision of S2-220xxxx)

Source: 	China Mobile

Title: 	KI #1, Sol #1: Update to add input of UPF registration in NRF

Document for: 	Approval

Agenda Item: 	9.25.1

Work Item / Release:	FS_UPEAS / Rel-18

Abstract of the contribution: Update to add input of UPF registration in NRF.

1 Discussion

Solution #20 proposed in SA2#152e includes UPF NF profile information registered in NRF. By adding Public IP pool information per IP Domain in NRF, AF/NEF can find UPF via NRF by providing NATed addressing information (including a public IP address and a port number).

2 Proposal

It is proposed to update TR 23.700-62 v1.0.0 as follows.

First change

6.1.2.1	UPF Event Exposure service Registration

The following Figure 6.1.2.1-1 depicts the UPF Event Exposure service Registration procedure.

Figure 6.1.2.1-1: UPF Event Exposure service Registration procedure

1.	The UPF sends the Nnrf_NFManagement_NFRegister Request message to NRF to inform the NRF of its NF profile when the NF service consumer becomes operative for the first time. The existing UPF NF profile parameters include e.g. S-NSSAI(s) and the associated NSI ID(s), DNN(s), IP range, information about the location of the UPF (operator specific information, e.g. geographical location, data centre), UPF Service Area (TAI List), DNAI, Public IP address pool information (i.e. Public IP address range, additionally ports number range), as described in TS 29.510 [6]. In addition, to support UPF Event Exposure Service, also Event Exposure Service Name, Supported Event ID(s) are provided with the UPF NF profile.

2.	The NRF stores the UPF profile and marks the UPF Event Exposure service as available.

3.	The NRF acknowledge UPF Registration is accepted via Nnrf_NFManagement_NFRegister response.

Second change

6.1.2.4	UPF Event Exposure service Discovery

The following Figure 6.1.2.4-1 depicts the UPF Event Exposure service Discovery procedure.

Figure 6.1.2.4-1: UPF Event Exposure service Discovery procedure

1.	Service consumer NF that requires UPF services invokes Nnrf_NFDiscovery_Request message to NRF with the intent to discover UPF. The input may include e.g. UPF service name, Event ID(s), TAI, NF type (i.e. UPF), S-NSSAI, DNN, DNAI, Public IP address pool information (i.e. Public IP address range, additionally ports number range), as described in TS 29.510 [6].

2.	The NRF authorizes the Nnrf_NFDiscovery_Request and based on the UPF profile the NRF determines if the service consumer of the UPF is allowed to discover the UPF.

3.	If allowed, the NRF determines a set of UPFs matching the input parameters included in the Nnrf_NFDiscovery_Request to the service consumer of the UPF via Nnrf_NFDiscovery_Request_Response. The output includes one or more UPF instances, and for each UPF instance it includes UPF NF profile.

End of change