3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208580

Elbonia, October 10 – 14, 2022

Source:	China Mobile

Title:	Conclusion update for KI#5 of EAS discovery

Document for:	Approval

Agenda Item: 	9.11

Work Item / Release:	FS_EDGE_Ph2

Abstract: conclusion for KI #5: This contribution proposes conclusion update of EAS discovery.

1. Discussion

2. Text Proposal

It is proposed to capture the following aspects in TR 23.700-48.

# * * * * First change * * * *

8.5	Interim conclusion for KI#5: GSMA OPG impacts and improvements for EHE operated by separate party

To enable the discovery of an EAS deployed in another PLMN (i.e. the hosting PLMN of the EHE) than the one serving the UE (e.g. an EAS deployed in a shared EHE).

EAS Deployment Provision

The serving PLMN is provided with the information needed to support UE's to discover and connect to the shared EHE.

-	The information may include PLMN ID, FQDN(s), DNS Server information, location information of EAS or DNS (e.g. DNAI respect to serving PLMN).

-	The above information can be local configured in serving SMF, or provided by AF, or provided by hosting PLMN.

-	The serving EASDF can get the above information from serving SMF with using R17 method.

-	The DNS server can be selected by EASDF with considering UE location, the same as R17 method.

NOTE 1: The final list of parameters that can be exchanged among PLMNs will be determined in the normative phase based on feedback from GSMA OPG

NOTE 2: The information mapping between serving PLMN and hosting PLMN maybe needed, and should be decided in normative phase.

EAS Discovery Procedure

-	EAS discovery procedure defined in TS 23.548 [3] will be reused.

NOTE 3: Whether all the methods defined in TS 23.548 [3] will be reused in this GSMA OPG scenario and/or whether extensions to the methods can be determined in normative phase.

Traffic transmission between two PLMNs

-	In order to support low latency transmission between two PLMNs, the serving SMF sends the N6 transmission requirement information to the UPF.

Charging

- 	Charing should be supported in sharing EHE scenario. And how to support charging in EHE sharing scenario should be specified with considering GSMA OPG's feedback. Some cooperation with SA5 may be needed.

Edge Relocation

Editor's note:	It is FFS how to support the Edge Relocation in GSMA OPG scenario.

Editor's note:	It is FFS how to support edge relocation between an EAS deployed by a source EHE provider to another EHE deployed by a target EHE provider, even in scenarios when EHEs are operated by different service providers.

Editor's note:	It is FFS how to support edge relocation between PLMNs.

# * * * * End of changes * * * *