3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-228585

Elbonia, Oct 10 – 17, 2022		(revision of S2-2206366)

Source:	vivo

Title:	KI#7, Sol#42, Update: EN removal about mobility restriction

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract: This paper proposes to update Sol#42 for EN removing about mobility restriction.

1. Introduction/Discussion

It can be understood that Relay UE and Remote UE taking the special roles of the normal UE still need to follow the basic principles on the emergency services handlings under the Mobility Restrictions and Access Restrictions. These basic principles can be referred to clause 5.16.4.3 in TS23.501 as following:

#### 5.16.4.3 Mobility Restrictions and Access Restrictions for Emergency Services When Emergency Services are supported and local regulation requires IMS Emergency Sessions to be provided regardless of Mobility Restrictions or Access Restrictions, the Mobility Restrictions or Access Restrictions (see clause 5.3.4.1) should not be applied to UEs receiving Emergency Services. Additionally, due to Mobility Restrictions or Access Restrictions (e.g. CAG restrictions) for normally registered UEs, that have established both non-emergency PDU Sessions and emergency PDU Sessions, the AMF indicates to the SMF to perform a local release of all non-emergency PDU Session via PDU Session Release procedure as specified in clause 4.3.4 of TS 23.502 [3]. The UE locally releases non-emergency PDU Sessions. The AMF and the UE behave as if the UE is emergency registered as described in TS 24.501 [47].

Thus, the current mobility restriction handlings on relay UE and remote UE needs to be override as described in clause 5.4.3 of TS 23.304 with following principles:

When Emergency Services are supported, the Mobility Restrictions or Access Restrictions (see clause 5.4.3 of TS 23.304 [X]) should not be applied to 5G ProSe enabled UEs receiving Emergency Services. The handlings on Mobility Restrictions for 5G ProSe UE-to-Network Relaying follow the principles as specified in clause 5.16.4.3 of TS 23.501 [4]. In particular, the 5G ProSe UE-to-Network Relay UE and Remote UE are allowed to perform the Relay operation including relay discovery and connection setup (see clause 6.3.2.3 and clause 6.5 of TS 23.304 [X]) when under the Mobility Restrictions or Access Restrictions.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-33.

# * * * * First change * * * *

6.42.2	Procedures

This clause captures the changes to the following (existing) procedures in TS 23.304 [3]:

-	clause 5.1.4.1	Policy/Parameter provisioning for 5G ProSe UE-to-Network Relay

-	Add description that how to provision emergency service support.

Editor's note:	The details of the description are FFS.

-	clause 5.4.3	Mobility Restrictions for 5G ProSe UE-to-Network Relaying

-	When Emergency Services are supported, the Mobility Restrictions or Access Restrictions (see clause 5.4.3 of TS 23.304 [3]) should not be applied to 5G ProSe enabled UEs receiving Emergency Services. The handlings on Mobility Restrictions for 5G ProSe UE-to-Network Relaying follow the principles as specified in clause 5.16.4.3 of TS 23.501 [4]. In particular, the 5G ProSe UE-to-Network Relay UE and Remote UE are allowed to perform the Relay operation including relay discovery and connection setup (see clause 6.3.2.3 and clause 6.5 of TS 23.304 [3]) when under the Mobility Restrictions or Access Restrictions.

Editor's note:	The details of the description are FFS.

-	clause 6.3.2.3	5G ProSe UE-to-Network Relay Discovery

-	Add description about discovery for emergency service.

Editor's note:	The details of the description are FFS.

-	clause 6.5.1.1	5G ProSe Communication via 5G ProSe Layer-3 UE-to-Network Relay without N3IWF

-	Add description of emergency service support, i.e., the 5G ProSe Layer-3 Relay UE establish emergency PDU Session based on the request from the 5G ProSe Layer-3 Remote UE.

Editor's note:	The details of the description are FFS.

-	clause 6.5.1.2	5G ProSe Communication via 5G ProSe Layer-3 UE-to-Network Relay with N3IWF support

-	Add description of emergency service support.

Editor's note:	Whether the 5G ProSe Layer-3 UE-to-Network Relay's PDU Session needs to be prioritized for 5G ProSe Layer-3 UE-to-Network Remote UE's emergency service is FFS.

Editor's note:	The details of the description are FFS.

-	clause 6.5.2	5G ProSe Communication via 5G ProSe Layer-2 UE-to-Network Relay

-	The following addition is to be added:

-	If the 5G ProSe UE-to-Network Relay UE's state is in RRC_IDLE, then the Relay UE set RRC establishment cause to "emergency".

-	If the 5G ProSe UE-to-Network Relay UE's state is in RRC_CONNECTED, the 5G ProSe UE-to-Network Relay needs to inform its CN over NAS that the UE is involved in emergency service for a 5G ProSe UE-to-Network Remote UE, so that the 5G ProSe UE-to-Network Relay UE can be exempted from e.g., overload control.

Editor's note:	When the 5G ProSe UE-to-Network Relay's state is in RRC-CONNECTED without emergency treatment, how the 5G ProSe UE-to-Network Relay get emergency treatment in NG-RAN is to be determined.

6.42.3	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

# * * * * End of changes * * * *