3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208610

Elbonia, Oct 10th – 17th, 2022	(revision of S2-220xxxx)

Source:	Samsung

Title:	Conclusion for FS_5TRS_URLLC KI#3

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract: Providing Conclusion for KI#3.

1. Introduction/Discussion

Based on the discussions during last meeting, it is proposed to update the conclusion for KI#3.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-25.

# * * * * First change (all new) * * * *

8.X	Conclusion for KI#3: Support for controlling 5G time synchronization service based on subscription

The following principles summarize the solution baseline :

-	UE subscription data types stored in the UDM are extended to include a new subscription data "Time Synchronization Subscription data" related to time synchronization service:

existing subscription data is updated + new subscription data type

-	“Access Stratum Time Synchronization Service Authorization” which indicate whether the UE is provisioned to receive RTI reception.

-	(Optionally) the Uu time synchronization error budget.

-	one or more “Subscribed time synchronization service ID(s)”, each containing the DNN/S-NSSAI and a reference to a PTP instance configuration may be pre-configured at the TSCTSF (e.g. PTP profile, PTP domain, etc).

-	(Optionally) Uu time synchronization error budget.

-	The AMF uses the " Time Synchronization Subscription data " in the following way:

-	The AMF retrieves the " Time Synchronization Subscription data " stored at the UDM during the registration procedure.

-	If the AMF receives Time Synchronization Subscription data Service Authorization from UDM, it provides 5G access stratum time distribution indication and Uu time synchronization error budget (if part of the Access Stratum Time Synchronization Service Authorization received from the UDM) to NG-RAN.

-	The TSCTSF uses the Time Synchronization Subscription data in the following way:

-	The TSCTSF retrieves the Time Synchronization Subscription data from the UDM when the TSCTSF receives an AF request for the time synchronization service (either ASTI or (g)PTP). According to in the Subscription data, the TSCTSF proceeds as specified in TS 23.502 [3].

-	The TSCTSF retrieves the Time Synchronization Subscription data from the UDM when the TSCTSF receives notification from the PCF that a UE has established a PDU Session that is potentially impacted by (g)PTP-based time synchronization service. The TSCTSF retrieves the PTP instance configurations referenced from the Subscribed time synchronization service ID(s). -	The TSCTSF determines if one or more of the PTP instance configurations match with the DNN/S-NSSAI of the given PDU Session. If no PTP instance exists for the given PTP instance configuration, the TSCTSF initializes the PTP instance in 5GS as described in clause K.2.2 of TS 23.501 [2]. The TSCTSF configures a PTP port in DS-TT and adds it to the corresponding PTP instance in NW-TT as described in clause K.2.2 of TS 23.501 [2].

# * * * * End of changes * * * *