3GPP SA WG2 Meeting #153E  			   	   S2-2208676

Elbonia, October 10-17, 2022

Source:	Samsung

Title:	KI#7: Proposal for Conclusion for KI#7

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys

Abstract of the contribution: This contribution provides a conclusion proposal to KI#7.

# 1.	Discussion

This contribution provides a conclusion proposal to KI#7.

# 2 Proposal

It is proposed to include the below changes to TR 23.700-80.

FIRST CHANGE

## 8.7	Key Issue #7: 5GS Assistance to Federated Learning Operation

Based on the evaluation in cl. 7.7, the following architecture principles and specific mechanisms below proposed by KI#7 solutions are considered for the normative work on 5GS assistance to FL operation.

-  Key principles proposed for 5GC assistance to application FL operation are as follows.

-  A new NF within the 5GC is recommended with the purpose of assisting the FL operation controlled by the AI/ML AF.

-  The new NF provides services specific to the FL operation. The new NF shall handle information exposure to communicate directly with the AI/ML AF when the latter is trusted, and via NEF when the AI/ML AF is untrusted. Other details of the new services are to be determined during normative work.

-  The new NF shall provide assistance for the AF to perform UE member selection for FL operation.

-  New and existing/updated analytics provided by NWDAF are to be used by the AI/ML AF to assist the FL operation.

-  Specific mechanisms proposed for 5GC assistance to application FL operation are as follows.

-  New QoS mechanisms for groups of UEs are required to assist FL operation:

-  The new QoS parameter Group-MBR and Group QoS monitoring are to be supported.

-  AF session QoS service is enhanced to provide QoS requirements for a group of UEs.

-  Time dependent QoS relying on AF model information mapping performed by new NF is supported.

-  New services and events are required to support exposure towards the AF of FL configuration and group performance.

-  Support for user consent exposure from UDM.

-  A new identifier is supported for the session utilized by the group participating in the FL operation.

-  Support for PCF group policy authorization when multiple UEs are served by the same PCF.

-  BSF discovery service enhancement to provide multiple serving PCF addresses for a given list of UE IDs.

END OF CHANGES