3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208708

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Update the conclusion for KI#3

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract: This contribution proposes to update the conclusion for KI#3.

1. Introduction/Discussion

Please find the discussion part in S2-2208706.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-81.

# * * * * First change * * * *

8.3	Key Issue #3: Data and analytics exchange in roaming case

For KI#3, the following architectural principles are agreed as interim conclusions:

-	An NWDAF or DCCF is enhanced as entry point to exchange analytics in roaming scenario between HPLMN and VPLMN. It authorizes the request according to operator policies and user consent and it filters the information exposed in reply to the request.

Editor's note:	The selection of the NWDAF or DCCF is FFS.

Editor's note:	It is FFS whether to define new services or reuse existing services.

Editor's note:	For security aspects, coordination with SA WG3 is required.

Editor's note:	Whether to provide raw data from VPLMN to HPLMN, or vice versa is FFS, and whether to provide analytics from VPLMN to HPLMN is FFS.

The interim conclusions for Key Issue #3 "Data and analytics exchange in roaming case" are as follows:

The following use cases are supported:

-	VPLMN may consume analytics information generated by HPLMN:

-	In home routed roaming scenarios, HPLMN analytics information (i.e. slice load level analytics, NF load analytics, etc.) can be leveraged by the AMF in the VPLMN for Network Slice selection and SMF selection for PDU Session management.

Editor's note:	It is FFS if the analytics information is analytics, analytics profiles or both.

-	UE-related analytics information provided by the HPLMN (e.g.. service experience analytics, etc.) can include statistics or predictions for outbound roaming UEs

Editor's note:	It is FFS whether the HPLMN can consume UE related analytics generated by the VPLMN. It needs to be studied whether the VPLMN can provide any meaningful analytics related to a user, in particular if the VPLMN can have analytics models for a user and whether it has access to raw data from the HPLMN for analytics and predictions. Otherwise the HPLMN will need access to raw data from the VPLMN for analytics statistics or predictions for outbound roaming UEs.

-	In home routed roaming scenarios,  analytics information with statistics or predictions for outbound roaming UEs can be leveraged by the H-PCF for QoS control of the PDU Session.

-	Analytics information (i.e. service experience analytics, slice load level analytics, etc.) can be leveraged by the H-PCF for decision of NSSP in URSP rules provisioned to the UE roaming in the VPLMN.

Editor's note:	The above is FFS and requires coordination with FS_UEPO, key issue 1.

# * * * * End of changes * * * *