3GPP TSG-WG SA2 Meeting #153E e-meeting  	S2-2208723

# Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0139 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on MTK or MSK processing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | LS from SA3 (S3-222391) provides the feedback on MSK/MTK update issues. The response from SA3 can be highlighted as follows: 1. Both the MSK and MTK are sent in the same messages or at least using the same signalling channel to reduce the complexity. In other words, during UE join processing, the MSK and MTK will be sent to the UE via PDU session modification command message from SMF to the UE. 2. MB-SMF can request the MSK, if the MSK is not available (e.g. the old MSK expired). In other words, there should be some services provided by MB-SMF or MBSF to enable such MSK provisioning procedure (e.g., PUSH or PULL-like provisioning procedure). 3. Authorization information refers to whether the UE is authorized to access the multicast service identified by the multicast session ID. MB-SMF subscribes the authorization information change event to UDM and acquire the information afterwards. In fact, it should be SMF that is notified by UDM but it is already mentioned in current spec. No further modification is needed. 4. SA3 agrees with the suggestion from SA2 that the impacts on MBS session creation can be described as well, and SA3 will make clearer references to related messages in the MBS session management procedures in clause 7.1.1 of TS 23.247, and reuse the existing messages in those procedures. In other words, SA2 can update the spec to reflect SA3 specification, e.g., Session join/establish/update/creation procedures. 5. SA3 agrees that MBSF is aware of the session status (i.e., active/inactive), and then MBSF can defer the MTK distribution until the session is active. For MSK distribution, it should be timely provided regardless of the session status. In other words, SA2 can update the spec accordingly reflect SA3 specification. Regarding the way to document in SA2, it is proposed to keep the impact to a small range, given that the Rel-17 is already frozen, and forward compatibility issue in case SA3 later update the changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Update Session join/establish/update/creation procedures, to reflect the content of the LS from SA3. 2. Update the associating services to reflect the LS from SA3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SA2 spec does not support MSK/MTK related processing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.13, 9.1.3.3, 9.1.3.6, 9.1.3.7, 9.1.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.13	MBS Security function

Security function may be needed to protect MBS related signalling/data. Detailed descriptions of security requirements, procedures and handling for 5G Multicast/Broadcast Service (MBS) are provided in TS 33.501 [20].

MBS security function is implemented in the MBSF/MBSTF so that it may apply only when MBSF/MBSTF are used (i.e. Configuration option 2 and 3). For configuration option 1 how to support MBS security is out of 3GPP scope.

The following additions for control plane procedure as defined in TS 33.501 [20] might be used to support the security function for multicast:

-	The multicast session security context, as defined in TS 33.501 [20], is used to protect MBS traffic.

-	MBSF distributes the multicast session security context to the MB-SMF via the Nmbsmf_MBSSession_Create Request message.

-	The SMF interacts with the MB-SMF to obtain the multicast session security context.

-	If the UE is authorized to join the Multicast MBS session, the SMF shall provide the multicast session security context to the UE in N1 SM container if received from the MB-SMF. The involved UEs are notified and use the received multicast session security context, to process the protected MBS traffic.

-	When the multicast session security context needs to be updated, MBSF shall send the updated information of multicast session security context to the MB-SMF, and then the MB-SMF shall trigger the session update as specified in clause 7.2.6. The involved UEs are also to be updated.

NOTE 1:	Interaction between MBSF and MBSTF will be defined in TS 33.501 [20] and 26.502 [18].

NOTE 2:	MBSF subscribes multicast session state from MB-SMF to efficiently update multicast session security context.

NOTE 3:	 Clause 9.1 defines the services for supporting security function for control plane procedure.

The additions for user plane procedure to support the security function for multicast and broadcast might be used as defined in TS 33.501 [20].

# * * * * Second change * * * *

9.1.3.3	Nmbsmf_MBSSession_ContextStatusSubscribe service operation

Service operation name: Nmbsmf_MBSSession_ContextStatusSubscribe

Description: Service Consumer NF can use this service operation to request information (e.g. QoS information) about a multicast session and to subscribe to notification of events about the multicast session context.

Inputs, Required: MBS Session ID, Notification Target Address, Events ID(s).

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: Event information (e.g. QoS information for multicast session, multicast MBS session state (Active, Inactive), multicast session service area for local multicast service), Start time of multicast MBS session, multicast DL tunnel info, if consumer is SMF: indication that the multicast MBS session allows any UE to join, multicast session security context.

9.1.3.6	Nmbsmf_MBSSession_Create service operation

Service operation name: Nmbsmf_MBSSession_Create

Description: Create a new multicast session or broadcast session, or for a location dependent MBS session, the part of the MBS session within a service area. Optionally subscribe to notifications for this MBS session.

Input, Required: MBS Session ID (SSM or TMGI) or TMGI allocation request, MBS Service Type (multicast or broadcast).

Input, Optional: DNN, S-NSSAI, MBS service area,  MBS Service Information (as defined in clause 6.14), Input Transport Address Request, indication that the PCF has to be contacted. For a multicast session, indication that any UE may join, multicast session security context. For a broadcast session, MBS start time, MBS termination time, MBS FSA ID(s). For subscription to notifications event ID(s), Notification Target Address, Request for location dependent MBS session.

Output, Required: Result Indication.

Output, Optional: TMGI, Expiry Time of the TMGI, Cause, MB-UPF tunnel info, MBS FSA ID(s), Area Session ID.

9.1.3.7	Nmbsmf_MBSSession_Update service operation

Service operation name: Nmbsmf_MBSSession_Update

Description: Update the established multicast session or broadcast session, or for a location dependent MBS session, the part of the MBS session within a service area, e.g. QoS update.

Input, Required: MBS Session ID.

Input, Optional: MBS Service Information (as defined in clause 6.14), MBS service area, for a broadcast session, MBS FSA ID(s), Area Session ID, indication that the PCF has to be contacted. For multicast, session state (Active/Inactive), multicast session security context.

Output, Required: Result Indication.

Output, Optional: Cause, MBS FSA ID(s).

9.1.3.9	Nmbsmf_MBSSession_StatusNotify service operation

Service operation name: Nmbsmf_MBSSession_StatusNotify

Description: This service operation is used by the MB-SMF to notify its consumers about the status change of the MBS session, or for a location dependent MBS session, of the part of the MBS session within a service area.

Input, Required: MBS Session ID, Event ID.

Input, Optional: Event information (e.g., expiration of the timer of multicast session security context), Area Session ID.

Output, Required: Result Indication.

# * * * * End of changes * * * *