3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208917

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#5 Conclusion Update for Sol#27

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract: This contribution proposes KI#5 conclusion for Sol#27 of FS_5G_ProSe_Ph2.

# 1. Introduction/Discussion

In SA2#152 e-meeting, solution 27 was updated to support two-path transmission via a Layer 3 UE to Network Relay without N3IWF or support in the application function. The solution describes how the PDU sessions over the direct and indirect paths are correlated in the core network so that UPF forwarding tunnels can be established to transfer data to and from the AF/UE using either of the available paths. This solution is transparent to the AF and does not require ATSSS.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-33.

# * * * * First change * * * *

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.5	Key Issue #5: Support of multi-path transmission for UE-to-Network Relay

For Key Issue #5 "Support of multi-path transmission for UE-to-Network Relay", the followings are taken as conclusions:

-	Policy authorization for multi-path transmission via direct Uu path and Layer 3 U2N Relay UE without N3IWF is needed, detailed URSP enhancement design and potential Registration procedure enhancement (e.g. in Sol #25) will be determined in normative phase.

NOTE 1:	Whether and how to authorize the multi-path transmission based on ProSe policy will be determined in normative phase.

Editor's note: Whether policy authorization for multi-path transmission via direct Uu path and Layer 3 U2N Relay UE with N3IWF is needed is FFS.

-	Multi-path transmission via direct Uu path and Layer 3 U2N Relay UE with N3IWF can rely on MA-PDU Session (introduced in clause 5.32 of TS 23.501 [7]) as described in Sol #28 and Sol #41, no system enhancement is required; in normative phase, how to use the MA PDU Session to support multi-path transmission via direct Uu path and Layer 3 U2N Relay UE with N3IWF will be described for information (e.g. as described in Sol #28 and Sol #41).

Editor's note: Redundancy and duplication for multipath transmission depends on R18 study ATSSS_Ph3 feature.

-  Multi-path traffic transmission via direct Uu path and Layer 3 U2N Relay UE without N3IWF indirect path can be achieved by creating forwarding tunnels in the UPFs terminating the PDU sessions as described in Sol #27. This method is transparent to the AF and does not rely on ATSSS.

-	For multi-path transmission via direct Uu path and Layer 2 U2N Relay UE:

-	Multi-path transmission via direct Uu path and Layer 2 U2N Relay will be defined by RAN WGs and alignment work can be made by SA2 based on the progress and outcome in RAN WGs.

# * * * * End of changes * * * *