3GPP TSG-WG SA2 Meeting #153E 	S2-2208940

# E-meeting, October 10th – 17th, 2022	(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | - | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: Support the enhancement of group attribute management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | GMEC |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Based on KI#1 conclusions and TR 23.700-74 KI#1 concluded that three basic solutions are proposed for enforcement of the service area. Two of them can be considered as an enhancement when re-using existing service area enforcement mechanisms. - TR 23.700-74 KI#1 concluded that the parameter provisioning service needs to be enhanced for 5G VN group management and UE related information to support the provisioning of the service area for a group. The service area is provisioned as part of the 5G VN group data. It is proposed to align this specification with the conclusion. - TR 23.700-74 KI#1 concluded that the parameter provisioning service needs to be enhanced for 5G VN group management to support the provisioning of QoS applicable to each UE within the group. The QoS for a group can be stored as part of the group subscription data. Also, the 5G VN group data is extended to include Group-MBR. It is proposed to align this specification with the conclusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Extend the service area mechanisms and apply service area information for each UE within the group. - Update the corresponding service to support provisioning of the service area and QoS applicable to each UE within the group, and store it as a part of the group subscription data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Normative work for reusing LADN mechanism use for group service area and the usage of service area and QoS for a group are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.29a (new), 5.29a.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.29a	Support for generic group management and exposure

5.29a.1	Group attribute management

A group may be a 5G VN group managed as defined in clause 5.29.2, as well as a group configured by OA&M. The attributes for a group includes:

-	Service area, the service area is consisted of Tracking Area identities or geographical information, it is applicable to each UE member within the group.

-	QoS, the QoS refers to QoS reference or individual QoS parameters as defined in clause 6.1.3.22 of TS 23.503 [45] and it is applicable to each UE member within the group.

-	Group-MBR, the Group-MBR limits the total bit rate that is expected to be provided across all PDU Sessions targeting for the group.

The 5G VN group management as defined in clause 5.29.2 is applicable to the provisioning of service area or QoS for the group with the following clarifications and enhancements:

-	The 5G VN group data additionally contains the service area or QoS. The procedures described in clause 4.15.6.2 of TS 23.502 [3] apply for provisioning service area and QoS as part of the 5G VN group data.

-	If the AF provides the service area in the form of geographical information, the NEF maps the geographical information to a list of TAs before sending the service area as part of 5G VN group data to the UDM.

-	The UDM receives service area as part of the 5G VN group data, UDM stores service area as part of the 5G VN group data at the UDR, provides service area as part of the 5G VN group data to AMF, and updates Session Management Subscription data for each UE member within the group to contain the DNN and S-NSSAI that associates with the group.

-	The UDM stores the QoS as default QoS in 5G VN group data or sets QoS as default QoS in each member’s SM subscription data. The SMF obtains such default QoS from UDM and sends it to PCF for policy decisions as described in clause 5.7.2.7 and TS 23.503 [45].

-	The DNN of the group may be a LADN DNN. If the DNN of the group is a LADN DNN, then network configuration restricts that the LADN DNN associated with the group is uniquely linked to an S-NSSAI.

If the DNN provisioned for the group is a LADN DNN, LADN as defined in clause 5.6.5 is applicable for enforcement of service area of a group with the following clarifications and enhancements:

-	The AMF receives the service area as part of the 5G VN group data and determines the DNN of the group is LADN DNN, then the AMF provides the UE with the LADN Information. In addition to the configured LADN service area for the DNN (if any) and registration area of the UE, the AMF additionally takes the service area received as part of the 5G VN group data into account to determine the LADN Information.

If the DNN provisioned for the group is a DNN other than LADN DNN, the SMF enforces the service area of a group with the following clarifications:

-	During PDU Session establishment procedure, the SMF receives the service area as part of group subscription data from UDM.

-	The SMF determines that the DNN and S-NSSAI of the PDU Session are associated with the group, but the DNN of the group is not LADN DNN, then the SMF subscribes to "UE mobility event notification" to the AMF for reporting UE presence in Area of Interest or UE location as described in clause 5.6.11. Upon reception of a notification from AMF, the SMF knows the UE presence in service area of the group is IN or OUT, then the SMF determines how to deal with the PDU Session according to operator's policy.

# * * * * End of changes * * * *