3GPP TSG-WG SA2 Meeting #153e	S2-2208986

E-meeting, October 10-17, 2022	(revision of S2-220xxxx)

Source:	Samsung

Title:	KI #1, conclusion update to reflect the SA2#152e decision.

Document for:	Approval

Agenda Item:	9.22

Work Item / Release:	FS_eUEPO / Rel-18

Abstract: This contribution proposes to update the conclusion on Key Issue #1.

1. Background

During SA2#152E meeting, there was a decision using a show of hands regarding whether V-PCF can generate VPLMN URSP rule. Based on a show of hand decision, it is agreed to exclude V-PCF generating VPLMN specific URSP rules.

2. Proposal

It is proposed to update the following changes in TR 23.700-85.

# * * * * 1st change * * * *

8.1	Conclusions on KI#1

8.1.1	General

The following definitions will be included as part of the normative work on URSP Rules in roaming:

VPLMN specific URSP Rules: A VPLMN specific URSP Rule is applicable when the UE is registered in the VPLMN only. VPLMN specific URSP rules are provided from the HPLMN and contains, based on agreements with VPLMN, the Network Slice Selection, DNN Selection, Time and Location criteria.. It is provided to the UE to route traffic on a PDU Session to a SMF and UPF in the VPLMN.

Editor's note:	It is FFS whether the definition of VPLMN specific URSP Rules needs to be updates to allow for the case where the VPLMN specific URSP Rule includes RSDs that apply in both the HPLMN and VPLMN.

HPLMN specific URSP Rules: A HPLMN specific URSP Rule is applicable either when the UE is registered in VPLMN or HPLMN. It contains HPLMN specific values on the Network Slice Selection, DNN Selection, Time and Location criteria. It is provided to the UE to route traffic on a PDU Session to a SMF and UPF in the HPLMN.

8.1.2	Conclusions on the how to identify PLMN specific URSP Rules

To enable the PCF to provide and the UE to identify the PLMN specific URSP Rules, the following conclusion principles apply:

-	VPLMN ID is provided to UE along the URSP.

Editor's note:	How to provide VPLMN ID to UE along URSP is FFS.

8.1.3	Conclusions on which PLMN determines the VPLMN specific URSP Rules

The scenario where the network provides the UE with URSP rules applicable in the VPLMN It is proposed to adopt the following interim conclusion principles:

-	The H-PCF provides VPLMN specific URSP Rules to the UE.

-	The H-PCF generate VPLMN specific URSP rules by taking Service Parameters either from V-PCF or the V-AF in.

Editor's note:	It is FFS whether a new re-evaluation trigger (e.g., change of PLMN/Route Selection Validation Criteria or an indication in the URSP Rule) is needed when VPLMN specific URSP Rules or VPLMN specific RSDs are used.

Editor's note:	It is FFS which solution is taken for normative specification, H-PCF generating VPLMN specific URSP rules based on service parameters provided by V-PCF or based on URSP guidance provided by the AF from VPLMN for the case when the H-PLMN generates the VPLMN specific URSP rules.

NOTE: The solutions of VPLMN providing URSP directly to a roaming UE are excluded since the HPLMN’s subscriber and VPLMN do not have a direct business relationship without HPLMN involvement.

# * * * * End of changes * * * *