3GPP TSG-SA WG2 Meeting #153E (e-meeting)	S2-2209025

# Elbonia, 10 – 17 October, 2022												 (revision of S2-xxxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.700-68 | CR | 0002 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove editor’s note in Conclusion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_REDCAP_Ph2 |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In 23.700-68 clause 8.1, which describes the conclusion of the study phase, there is still and editor’s note: Editor's note: Details on possible triggers for gNB to send indication are FFS. However, there are multiple issues if the gNB does not notify the CN when it moves the UE to RRC_INACTIVE with eDRX longer tan 10.24 seconds: - Data may be lost if the UE moves away from RAN notification area before the PTW. - Network initiated procedures failure rate increases with longer eDRX. NAS retransmissions are shorter than 10.24s, and that was one of the reasons RRC Inactive was only increased to 10.24 seconds in Rel-17. - Services provided to AF via NEF, like UE reachability, downlink delivery status, and UE availablity after failure, are broken. The CN cannot know if the UE is actually reachable, may indicate to AF that the UE when it isn’t, may indicate downlink delivery succesful, when the data is actually still buffered in the RAN or even discarded. The NG-RAN also does not know if the AF is using this feature in the CN, so it cannot use this information to decide whether to indicate or not edRX>10.24s in RRC Inactive to CN. All the above issues need to be resolved before it is acceptable for RAN to not indicate anything to CN when the UE is transitioned to RRC Inactive with eDRX>10.24s. In fact, the above issues are in part the reason why SA2 needed a study phase in the first place. Therefore, it is proposed to conclude that NG-RAN always indicates to CN that UE has entered eDRX>10.24s with RRC inactive. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove editor’s note. Conclude that NG-RAN always notifies the CN when the ue enters RRC Inactive >10.24s. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear RAN behavior may lead to loss of data and control plane procedure failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

## 8.1	Conclusions for Key Issue #1: Enabling long eDRX cycle support in RRC_INACTIVE state

The following aspects are concluded as principles for the normative work:

-	It is agreed to support MT data signalling handling in CN when the UE is unreachable due to long extended DRX in RRC inactive.

-	When the gNB sends an indication to the CN and provides unreachability information (e.g., eDRX values negotiated between UE and gNB for RRC_INACTIVE state), the CN applies the HLcom functions based on gNB provided unreachability information (as described in solution #6).

-	If the gNB has indicated the UE has entered RRC_INACTIVE to the CN, the gNB also notifies the CN about the RRC State transition back to RRC_CONNECTED (as described in solution #6).

-	The gNB provides the indication to the CN at the moment the UE is transitioned to RRC_INACTIVE with eDRX larger than 10.24 seconds.

NOTE 1:	If the indication of UE transition to RRC_INACTIVE is not sent (or sent after UE has entered RRC_INACTIVE) by the gNB then until CN receives it the CN cannot apply HLcom functionality and other NFs will not be aware of the UE reachability, and certain HLcom related services provided to the AF via NEF would not be available. Downlink data transmitted from the UPF to RAN might be discarded and not delivered to the UE.

NOTE 2:	Further coordination with RAN WGs may be needed during the normative phase.

.

****************** END CHANGES ***************