3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2209076

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

Title:	Update of key issue#3 conclusion

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3 / Rel-18

Abstract: this paper is to update key issue#3 conclusion.

1. Introduction/Discussion

Key issue #3 was concluded in the last meeting with a simple description that The GMLC provides LMF ID to AMF. The AMF uses the LMF ID to select the LMF, as described in solution#6.

The conclusion should be further clarified with more details. It is unclear how the GMLC can derive or provide the LMF ID to the AMF.

As proposed in several solutions, GMLC’s context for LCS client/AF data needs to be extended with more parameters, of which are used as mapping information for the GMLC to determine the LMF ID.

Another option is the UDM’s UE LCS subscriber data is enhanced by adding a correlating LMF ID, and GMLC fetches it during the location procedure.

It is proposed to add more details into key issue#3 conclusion.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-71.

# * * * * First change * * * *

8.3	Key Issue #3: Local Area Restriction for an LMF and GMLC

The following aspects are concluded for normative work:

Aspect 1: Provisioning of LMF ID

- LCS Data in the GMLC for an LCS Client/AF, maybe enhanced by adding the following parameters:

- LMF ID;

- Group ID and its correlating LMF ID;

When the GMLC receives a MT location request from LCS client/AF, GMLC determines the LMF ID based on the LCS Data context per LCS client/AF.

In case that group ID can be provided or derived from the LCS request, GMLC determines the LMF ID based on the provisioned Group ID.

- GMLC may be configured with an LMF ID, and irrelevant to any LCS client/AF. When the GMLC receives a MT location request from LCS client/AF, GMLC determines the LMF ID for all LCS client/AF.

- UDM may be enhanced in the UE LCS subscriber data by adding a correlating LMF ID, and GMLC fetches it when interacting with UDM.

- AMF maybe configured locally, a mapping table of UE identity e.g. MSISDN and LMF/GMLC address, When receiving a MO-LR, AMF determines LMF and GMLC based on local configuration.

Aspect 2: LMF selection

The GMLC provides LMF ID to AMF or AMF determines the LMF ID based on aspect 1. The AMF uses the LMF ID to select the LMF.

# * * * * End of changes * * * *