3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2209126

Elbonia, October 10 – 14, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Updates to KI #7 Evaluation and conclusion

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3/ Rel-18

Abstract: Updates to KI #7 Evaluation and conclusion.

# 1. Discussion

This paper proposes Updates to KI #7 Evaluation and conclusion

# 2. Text Proposal

It is proposed to capture the following in the TR 23.700-71.

# * * * * First change * * * *

### 7.7.1	Evaluation of solutions for PRU

There are four solutions related to Positioning Reference Units (PRU) of KI#7, i.e. solution#15, #16, #28, #29 . Solution#15 includes three options from PRU Information Acquisition perspective, which are referred as solution#15-AMF, solution#15-LMF and solution#15-UDM.

The comparison of solutions is shown in Table 7.7.1-1.

Table 7.7.1-1: Comparison of solutions to PRU of KI#7

|  | Sol#15-AMF | Sol#15-LMF | Sol#15-UDM | Sol#16 | Sol#28 | Sol#29 |
| --- | --- | --- | --- | --- | --- | --- |
| PRU Registration | UE sends Registration Request including PRU information to AMF | Including two options: Option#B.1: UE sends PRU Registration Request including PRU information to LMF via AMF. The message is a supplementary service operation. Option#B.2: UE sends PRU Registration Request including PRU information to LMF via LPP procedure. | PRU information included in the UE subscription data is pre-configured in UDM | When AMF receives PRU indication in Registration Request, it triggers positioning procedure after the Registration procedure. LMF obtains PRU location after the positioning procedure and stores the information locally. | Similar to sol#15-LMF, option#B.1. PRU may register to multiple LMFs. | UE sends Registration request with PRU info to AMF. PRU Info may include AMF ID, LMF ID or URI, mobile/static PRU, Position info |
|  | AMF invokes Nnrf_NFManagement_NFUpdate Request including PRU location and PRU existence indication to NRF. | LMF invokes Nnrf_NFManagement_NFUpdate Request including PRU location and PRU existence indication to NRF. |  | LMF invokes Nnrf_NFManagement_NFUpdate Request including PRU information to NRF |  | LMF notifies the GMLC the PRU information |
| How to obtain PRU information used to assist target UE positioning | Target UE LMF obtains PRU AMF from NRF. Target UE LMF obtains PRU information from PRU AMF. | Target UE LMF obtains PRU LMF from NRF. Target UE LMF obtains PRU information from PRU LMF. | Target UE AMF obtains PRU information from UDM and provides PRU information to Target UE LMF. | Same as sol#15-LMF | Target UE LMF stores information about available PRUs | Target UE LMF obtains PRU from GMLC. |
| Impacted 5GC NF | AMF, LMF, NRF | AMF, LMF, NRF | AMF, UDM, LMF | AMF, LMF, NRF | LMF | AMF, LMF, GMLC |
| Impacts to RAN/ RAN WG | PRU wake up: RAN receives PRU wake up indication from AMF and sends broadcast message in the serving cell of target UE. | PRU wake up: same as sol#15-AMF. In Option#B.2, PRU registration procedure is a new LPP procedure which is in RAN2 scope. | PRU wake up: same as sol#15-AMF | PRU wake up: same as sol#15-AMF | No | No |

Solution #15 supports multiple alternatives and impacts multiple entities besides PRUs (AMF, LMF, NRF, UDM). There are also separate procedures defined for supporting an MT-LR and MO-LR and for activating and deactivating PRUs which add more impact. Option A (PRU registration to AMF) impacts the AMF which needs to indicate a PRU to the NRF after which an LMF would need to discover PRUs from the NRF. However, PRUs are not NFs but more part of the NG-RAN making NRF support possibly unnecessary. Option B (PRU registration to LMF) is more direct and impacts the PRU, LMF and possibly NRF. Option C (PRUs information are maintained at the UDM) requires an AMF to query the UDM for available PRUs and then indicate these to an LMF. It is not clear how a UDM would know about PRU availability since a PRU could be powered off, out of coverage or undergoing maintenance. Of these options, option B looks simplest.

Solution #16 uses the NAS Registration procedure to indicate a PRU to an AMF which then indicates the PRU to an LMF via an Nlmf location request. There is no direct confirmation back to the PRU that the AMF was able to indicate the PRU to an LMF, so if the LMF rejects the Nlmf location request or if the AMF cannot find a suitable LMF, the PRU would not know. LMF utilization of a PRU to support location of a target UE (in clause 6.16.3.2) includes UE positioning in steps 2 and 6 for Figure 6.16.3.2-1. However, no details are provided concerning how the PRU would help position the target UE. Positioning of a target UE by a PRU is not so far defined by RAN, so it is unclear whether these steps are feasible and what might be the impacts to the LMF, target UE and PRU. The use of an NRF to discover other PRUs managed by another LMF to assist with the location of a target UE would add additional impacts to an LMF and NRF.

Solution #17 is based on use of sidelink positioning and ProSe. There seems to be no Uu (UL/DL) level positioning. This means the solution should be part of sidelink positioning and ranging in Release 18 and cannot be evaluated separately, e.g. in case it duplicates or conflicts with other sidelink positioning and ranging solutions for Release 18.

Solution #28 avoids any impact to an AMF or NRF. A PRU registers with one or more LMFs using a supplementary services protocol. The PRU can be used by an LMF to obtain measurements of RAN nodes in the same was as from a normal UE using LPP. These measurements and a location of the PRU can be used to determine correction terms (e.g. timings and internal delays) of RAN nodes, as defined in TS 38.305 [6]. Extension of the solution to support Reference UEs could be possible later.

# * * * * Second change * * * *

## 8.7	Key Issue #7: Support of Positioning Reference Units and Reference UEs

To support PRU, the interim conclusions are as follows:

-	For PRU Registration:

-	UE sends UL NAS Transport including PRU Registration Request to AMF.

-	The PRU Registration is an LCS supplementary service message.

-

-	PRU Registration Request includes PRU information

-

-	AMF stores PRU information to UDM/UDR.

-	The LMF of the target UE determines to use PRU and may obtain PRU information  from UDM/UDR.

-	A serving LMF of PRU obtains PRU location measurements as described in clause 5.4.5 in TS 38.305 [6] by triggering the existing procedures defined in clause 6.11 in TS 23.273 [5]:

-	To improve target UE positioning, the serving LMF of the target UE may obtain PRU location measurements from a serving LMF of PRU.

# * * * * End of changes * * * *