3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2209151

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Intel

Title:	TR 23.700-05: KI#7 Evaluation and conclusion

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR / Rel-18

Abstract:  Evaluation and conclusion of KI#7 Control of UE's access to 5GS via a mobile base station relay.

# 1. Introduction/Discussion

This paper is trying to evaluate and conclude the solutions of KI#7.

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-05.

# * * * * First change (all text new) * * * *

## 7.X	Key Issue #7: Control of UE's access to 5GS via a mobile base station relay

This clause evaluates the solutions related to KI#7. There is only one solution, #20, addressing KI#7, which proposes to use the existing CAG, Closed Access Group, concept as a base to manage the UE access control via MBSR. In this solution, a new MBSR CAG identifier (or mobile-IAB CAG Identifier) is assigned for the MBSR case, then 5GC and RAN perform CAG-based access control. UE (supporting CAG) performs cell selection based on existing CAG control logic and UE (legacy UE not supporting CAG) performs cell selection as normal.

# * * * * Next change (all text new) * * * *

## 8.X	Key Issue #7: Control of UE's access to 5GS via a mobile base station relay

For Key Issue #7: Control of UE's access to 5GS via a mobile base station relay, the followings are taken as conclusions:

-	the CAG mechanism defined in clause 5.30.3 of TS 23.501 [2] can be used for Control of UE's access to 5GS via MBSR

NOTE:	CAG identifiers value assignment for MBSR will be determined during normative phase, e.g. whether dedicated CAG identifiers will be used for MBSR access.

Editor's note: Whether additional control (e.g., time duration, specific geographic location) may be linked to the MBSR CAG identifier management is FFS.

# * * * * End of changes * * * *