SA WG2 Meeting #S2-153E	S2-2209180

10 – 14 Oct. 2022, E-meeting

Source:	Xiaomi

Title:	update conclusion#1 to remove the EN

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2/Rel-18

Abstract of the contribution: This paper proposes to update the conclusion#1 to remove the EN.

Discussion

For Model A discovery, in Solution #10, UE-to-UE Relay collects the list of User Info ID and Layer-2 ID of target UE based on previous discovery or based on existing or prior communication between the UE-to-UE Relay and the target UEs, but without considering whether the target UE would like to be discovered via UE-to-UE Relay. It may face privacy and security issue to expose the target UE information without clear confirmation from target UE.

6.10.2.1 Model A discovery Depicted in figure 6.10.2.1-1 is the procedure for UE-to-UE Relay discovery Model A - The UE-to-UE Relay decides that it can act as a UE-to-UE Relay based on configured policy, and the UE-to-UE Relay determines a list of User Info ID and Layer-2 ID of the target UEs it can announce based on the previous discovery or based on existing or prior communication between the UE-to-UE Relay and the target UEs. For example, the UE-to-UE Relay may perform the Group Member Discovery procedure (either Model A or Model B) or Direct Discovery procedure (either Model A or Model B) with the RSC it supports to discover target UEs it can announce. NOTE: In order to keep an up-to-date target UE list, UEs want to be discovered by other UEs via the UE-to-UE Relay periodically performs the Discovery procedure, or response to the UE-to-UE Relay Announcement as described in sol#9. The UE-to-UE Relay, by implementation, removes specific target UE info from the target UE list after its expiration.

Proposal: UE-to-UE Relay needs the clear confirmation from the target UE before adding the User Info ID and Layer-2 ID of target UE into the list.

*** Start of changes ***

8.1	Key Issue #1: Support of UE-to-UE Relay

For Key Issue #1 (Support of UE-to-UE Relay), the followings are taken as conclusions:

The following conclusions are common for both Layer-3 UE-to-UE Relay and Layer-2 UE-to-UE Relay:

-	For UE-to-UE Relay discovery, both Model A and Model B discovery are supported.

-  For Model A discovery, the Announcement message includes:

- Type of discovery message

- Relay Service Code

- User Info ID of UE-to-UE Relay

- A list of User Info ID and Layer-2 ID of target UE

UE-to-UE Relay needs the confirmation from the target UE before adding the User Info ID and Layer-2 ID of target UE into the list.

Editor's Note: It is FFS if discovery integrated into PC5 unicast link establishment procedure is supported.

-	For UE-to-UE Relay selection, the Source UE performs the UE-to-UE Relay selection for both Model A and Model B discovery.

Editor's Note: It is FFS if Target UE can also perform the UE-to-UE Relay selection for Model B discovery.

-	For UE-to-UE Relay reselection, the negotiated UE-to-UE Relay reselection between Source UE and Target UE and the UE-to-UE Relay selection procedure can be used under different conditions.

NOTE 1:	UE-to-UE Relay selection/reselection requires cooperation with RAN WGs during normative work.

-	IP, Ethernet and Unstructured traffic types are supported.

NOTE 2: 	Ethernet and Unstructured traffic types can be encapsulated in IP traffic type if supported by source and target UE.

NOTE 3: 	Conclusion on Service Authorization information related to UE-to-UE Relay operation sent by AMF to NG-RAN can be finalized based on feedback from RAN WGs.

The following conclusions are specific for Layer-3 UE-to-UE Relay:

- 	For UE-to-UE Relay Per-hop links setup (i.e. PC5 link establishment between Source UE and UE-to-UE Relay, as well as between UE-to-UE Relay and Target UE), Source UE initiates the PC5 link setup with UE-to-UE Relay, and UE-to-UE Relay initiates the PC5 link setup with the target UE.

- 	For QoS control of Layer-3 UE-to-UE Relay, the UE-to-UE Relay receives E2E QoS from Source UE and determines the per-hop QoS parameters to satisfy the E2E QoS.

The following conclusions are specific for Layer-2 UE-to-UE Relay:

-	Per-hop links (i.e. PC5 link between Source UE and UE-to-UE Relay, as well as between UE-to-UE Relay and Target UE) needs to be established before E2E PC5 link establishment is performed.

NOTE 4:	How the E2E PC5-S messages are forwarded by the UE-to-UE Relay is to be determined by RAN WGs.

NOTE 5:	For Layer-2 UE-to-UE Relay, RAN WGs will define how the E2E QoS will be handled and split over the PC5 links.

*** End of changes ***