SA WG2 Meeting #S2-153E	S2-2209181

10 – 14 Oct , 2022, E-meeting

Source:	Xiaomi

Title:	conclusion for key issue#2

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	5G_AIMLsys/Rel-18

Abstract of the contribution: This paper proposes the conclusion for key Issue#2

*** Start of changes ***

## 8.2	Key Issue #2: 5GC information exposure to UE

For UE requests or subscribes to network information:

- User plane is supported for UE to request or subscribe to NWDAF for collecting the network information via AF or DCAF on behalf of UE. No normative work needed.

- Control plane is supported for UE to request or subscribe to network information via NAS procedure (e.g., PDU session establishment, or registration). New indication is introduced in the PDU session establishment/modification to indicate SMF to request/subscribe to NWDAF on behalf of UE.

For 5GC exposes the network information to UE:

- User plane is supported for network information exposing from NWDAF to UE via AF. No normative work needed.

- Control plane is supported for network information exposing from NWDAF to UE via SMF/AMF.

Authorization and network consent is needed before UE requesting or subscribing, collecting the network information exposed by NWDAF.

*** End of changes ***