3GPP TSG-SA WG2 Meeting #153 e-meeting	S2-2209276

# Elbonia, 10th – 17th Oct, 2022												         (was S2-2208866r01)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3586 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Aerial Subscription Update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The scenario of updating the Aerial subscription info to UAV when it was registered as normal UE was discussed in S2-2107322 (in #147e) and S2-2108457 (in #148e) but SA2 could not get any consensus and the issue remain in the system. But in recent CT1 137#e meeting this scenario was discussed in C1-225456 and majority agreed that AMF should inform the UE about this subscription change in UE Configuration Update procedure. Hence we are submitting this proposal again for SA2 to consider that there is an issue in the system and addressed it with the proposed change. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following change is proposed: - After the Aerial subscription gets enabled for the UAV it is proposed that AMF will inform the UE through UPU and UAV may initiate Mobility Registratio Update procedure to request for UAS service |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UAV will not be able to request for UAS service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.20.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR xxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * start of 1st change * * * *

### 4.20.1	General

The purpose of the control plane solution for update of UE parameters is to allow the HPLMN, SNPN, or CH to update the UE with a specific set of parameters, generated and stored in the UDM, by delivering protected UDM Update Data via NAS signalling. The HPLMN, SNPN, or CH updates such parameters based on the operator policies.

The UDM Update Data that the UDM delivers to the UE may contain:

-	one or more UE parameters including:

-	the updated Default Configured NSSAI (final consumer of the parameter is the ME);

-	the updated Routing Indicator Data (final consumer of the parameter is the USIM when the related credential is stored in the USIM, i.e. for PLMN or SNPN credentials; or final consumer of the parameter is the ME when the related credential is stored in the ME, i.e. for SNPN credentials);

-	indication of whether disaster roaming is enabled in the UE if UE MINT support indicator is received or UE is registered for Disaster Roaming service currently; and

-	indication of 'applicability of "lists of PLMN(s) to be used in disaster condition" provided by a VPLMN' if UE MINT support indicator is received or UE is registered for Disaster Roaming service currently.

-    indication of AerialUESubscriptionInfo is enabled for the UAV.

-	a "UE acknowledgement requested" indication.

-	a "re-registration requested" indication.

* * * * End of changes * * * *