SA WG2 Meeting #S2-153E (e-meeting)	S2-2209907

10 - 17 October 2022, Electronic Meeting	(revision of S2-220xxxx)

Source:	Ericsson, Deutsche Telekom, Meta, Apple, Qualcomm Incorporated

Title:	KI#1 and KI#2, Solutions evaluation and conclusion

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: This contribution provides evaluation and conclusions of the solutions for Key Issue #1 and Key Issue #2

# 1. Introduction

This contribution provides evaluations and conclusions of the solutions documented in the FS_XRM TR 23.700-60 for Key Issue #1 and Key Issue #2

# 2. Proposal

This paper proposes the following updates to TR 23.700-60 clause 8 (all new text), with respect to the evaluation and conclusions of solutions for Key Issue #2.  During meeting the KI#1 and 2 conclusion was handled in two separate documents.

* Start of changes all changes are new*

* First change all new*

* End of first change *

* Second change all new*

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.x Conclusions for Key Issue#2

The following aspects are concluded as principles for the normative work:

- For multi-modality flows transmitted by multiple UEs, the AF manages the flows in the following ways:

- The AF procedure is extended with QoS requirements for individual service flows and treatment requirements and policies according to the conclusions for Key Issue #1.

- In case multiple UEs are involved, AF provides the information related to all UEs to NEF either using a single procedure or using multiple UE specific procedures.

- In either case, the AF may indicate that specific service data flows belong to the same multimodal service by associating them with a common ID.

- PCF uses the individual service data flow QoS requirements and requirements and policies in the same way as concluded for Key Issue #1.

Editor’s Note: Whether multiple PCFs may be involved in the procedures for multiple UEs and how the coordination across PCFs works in that case is determined in the normative phase.

* End of second change *

* End of changes *