3GPP TSG-SA WG2#153E	S2-2208492

Elbonia, 10 - 17 October 2022	(was S2-22xxxxx)

Source:	CATT

Title:	KI#4: Remove RAN dependency ENs in KI#4 Conclusions

Document for:	Approval

Agenda Item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2/Rel-18

Abstract of the contribution: This contribution proposes to remove ENs in KI#4 Conclusions.

- Introduction

Path switching between direct path and indirect path for Layer-2 Relay is mainly in RAN scope and will be defined by RAN WGs, so the ENs in KI#4 conclusions can be removed and alignment work can be made by SA2 based on the progress and outcome in RAN WGs.

- Proposal

It is proposed to capture the following texts into TR 23.700-33.

>>>>First Change<<<<

8.4	Key Issue #4: Support of path switching between direct network communication path and indirect network communication path for Layer-2 UE-to-Network Relay with session continuity consideration

For Key Issue #4 "Support of path switching between direct network communication path and indirect network communication path for Layer-2 UE-to-Network Relay with session continuity consideration", the followings are taken as conclusions:

-	Xn based (as defined in clause 4.9.1.2 of TS 23.502 [8]) and N2 based (as defined in clause 4.9.1.3 of TS 23.502 [8]) HO procedure is applied for inter-gNB indirect-to-direct and inter-gNB direct-to-indirect path switching for Layer-2 Remote UE in CM-CONNECTED state.

NOTE:	Path switching between direct path and indirect path for Layer-2 Relay (e.g. whether the source or target gNB selects a target Relay UE or direct Uu route, and whether and what information to be taken into account by NG-RAN for path switching) will be defined by RAN WGs and alignment work (if any) can be made by SA2 based on RAN WGs conclusions in normative phase.

>>>>End Changes<<<<