3GPP TSG-SA WG2 Meeting #153e	S2-2208209r06

# Oct 10 – 14, 2022

Title:	[draft]LS on XR and Media Services

Response to:	LS on UE Power Saving for XR and Media Services (R1-2205531/S2-2208105)

Release:	Rel-18

Work Item:	FS_XRM, FS_NR_XR_enh

Source:	3GPP TSG SA WG2

To:	RAN WG1, RAN WG2, RAN WG3

Cc:

Contact Person:

Name:	Xiaowan Ke

Tel. Number:

E-mail Address:	xiaowan(dot)ke(at)vivo(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 would like to thank RAN1’s LS reply (R1-2205531/S2-2208105) on UE power saving for XR and media services. So far, SA2 has started to evaluate and conclude solutions for key issues for the study of XR and Media Services and has reached conclusions in chapter 8 of TR 23.700-60. SA2 would like to coordinate with RAN WGs about the following aspects:

- In KI#3, SA2 has been studying what information is useful and can be exposed by 5GS to the server. In the mentioned KI, SA2 clarifies supports for L4S and for exposure of congestion level are pending RAN WG's feedback on the feasibility of RAN judgment and/or exposure of the corresponding info (e.g. per QoS flow congestion level). Two variants of L4S marking are considered: (1) L4S marking in the NG-RAN node and (2) L4S marking by the PSA UPF based on information provided by NG-RAN. SA2 would like to ask RAN2 and RAN3 following questions:

  - Q1: whether it is feasible for RAN to detect the per QoS flow congestion level, per DRB congestion level and per RAN node congestion level in, downlink and uplink direction.

  - Q2: whether it is feasible for RAN to detect the per QoS flow UL congestion level, per DRB UL congestion level without UE impact.

  - Q3: if the answer to Q1 is yes, for the case of variant (1), is it feasible for NG-RAN to perform L4S marking on a given QoS Flow, DRB or RAN node?

  - Q4: if the answer to Q1 is yes, for the case of variant (2) is it feasible for NG-RAN to indicate in GTP-U to UPF the congestion level event for L4S marking on a given QoS Flow, DRB or RAN node?

  - SA2 is also considering RAN node congestion and per QoS flow congestion information exposure on N2 as well as on N3.

Hence SA2 would also like RAN2 and RAN3 to feedback regarding RAN support for reporting via N2 signalling and N3 (i.e. GTP-U to UPF) of following information:

-	Congestion event and congestion level information for a given QoS flow, DRB or for a RAN node;

-	Data rate for a given QoS flow.

- In KI#4&5, SA2 has been discussing the extension of the 5GS QoS framework to support the efficient handling of PDU Set, mainly including PDU Set identification and PDU Set level QoS. SA2 understands the uplink PDU Set handling is closely related to the interaction between the UE and RAN. Hence, SA2 decides to focus on the downlink PDU Set handling and to align with RAN’s progress and decision for uplink, if any.

In RAN1’s LS reply (R1-2205531/S2-2208105), SA2 notes that RAN1 indicates that it may be helpful for the core network to provide RAN with the following pieces of information: PDU set periodicity and start time, PDU set end indication, PDU set level QoS parameters, PDU set size (number of bits) or number of PDUs in a PDU set, PDU set identity and relationship information among PDUs within the same PDU set, and Jitter information. SA2 would like to inform RAN1 that relying on above listed PDU Set information alone may not result in an optimal CDRX configuration. Traffic over one QoS flow may be made up of multiple PDU sets and these PDU sets may belong to a single traffic pattern or may belong to multiple traffic patterns. As a result, a PDU set end indication does not necessarily mark the end of a Data Burst. It may be important for RAN to detect the end of data bursts. Furthermore, SA2 would like to point out that a QoS flow may be characterized by multiple PDU set periodicities (i.e., one for each traffic pattern). Thus, a single / static CDRX configuration may be inefficient for traffic from such a QoS flow. There may be scenarios where multiple flows with different periodicities are mapped to the same QoS Flow.

Editor's note:	If the other KIs also need to send LS to RAN to coordinate open questions during the meeting, they can be added in the LS based on consensus.

2. Actions:

To RAN WG1, RAN WG2, RAN WG3:

ACTION: 	SA2 kindly asks RAN WG1, RAN2 and RAN3 to take the above information into account and reply.

3. Date of Next TSG SA WG2 Meetings:

SA WG2 Meeting #154      	14 – 18 November 2022			Toulouse, France

SA WG2 Meeting #155     	20 – 24 February 2023			   	EU, EU