SA WG2 Meeting #153e	S2-2208226r04

# October 10th – 14th, 2022; Elbonia              		(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	update to conclusions for KI2

Document for: 	Approval

Agenda Item: 	9.13.1

Work Item / Release:	FS_5WWC_Ph2 / Rel-18

Abstract of the contribution: update to conclusions for KI2

# 1	Discussion

The conclusion in clause 8.2.2 is backed by the description in the second change

# 2 Proposal

It is proposed to update TR 23700-17 as follows

FIRST CHANGE

### 8.2.2	How to select a TNGF that supports the S-NSSAI(s) needed by the UE

Selection of a TNGF that supports the S-NSSAIs needed by the UE is enabled based on:

-	UE based solutions where UE(s) may use WLAN SP policy to select a SSID allowing to access to a TNGF that supports the slices the UE are willing to use and then leverage this information to try registering onto 5GC via TNGF.

-	The WLAN SP policy is extended with the indication of the set of slices associated to a SSID.

-	 (the AMF receives from the TNGF over N2 information on the set of slices associated to / supported by a SSID that the TNGF supports)

-	The AMF may determine the UE used a wrong SSID based on information received over N2 from the TNGF

-	The AMF may trigger the UE Policy Association Establishment procedure to provide the UE with updated WLANSP if the selected SSID (TNGF) does not support the slices requested by UE. The AMF requests the PCF to receive a notification when the PCF has completed the WLANSP update; once the AMF has received this notification from the PCF, the AMF can issue a registration reject

-	AMF may provide the target TNAN information (e.g. such as SSID) associated with the Requested NSSAI to UE within Registration Reject message.

Editor’s Note: it is FFS whether TNGF IDis provided as part of TNAN information and the Ue needs to use it to build the target NAI

-	UE selects an appropriate SSID associated with the Requested NSSAI and build the realm of NAI.

No solution using the AMF redirection of the UE with a registration accept will be pursued in normative specifications.

SECOND CHANGE all text is new