3GPP TSG-SA WG2 Meeting #153E	S2-2208295R04

Elbonia, Oct 10 - 17, 2022	(revision of S2-220xxxx)

# Title:	[Draft] Reply LS on UAV authorization container

# Response to:	(S2-2208104/C4-223513) LS on UAV authorization container

# Release:	Rel-17

# Work Item:	ID_UAS

Source:	[Nokia will be] SA2

To:	CT4

Cc:	CT1, CT3

Contact Person:

Name:	Pallab Gupta

Tel. Number:

E-mail Address:	pallab.gupta@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: TS 23.256 CR xxxx.

1. Overall Description:

SA2 thanks CT4 for their LS on UAV authorization container.

SA2 would ask CT4 to note the following answers for the corresponding questions raised by CT4 and would request CT4 to define/update stage 3 specification accordingly.

- Does the SMF need to identify the type of payload (C2- or UUAA-Aviation Payload) to perform the procedures from and towards the UE?

Answer: No. The SMF does not need to identify the payload type. From an SMF point of view, it is transparent whether the authorization is for UAV or for C2 connectivity or for both. For uplink direction, UE provides the Payload and Payload Type in the NAS container as per current stage 3 specifications. The SMF invokes Nnef_Authentication_AuthenticateAuthorize service operation including the Service Level Device Identity and the Payload and Payload Type provided by the UE. The UAS NF may need to identify the payload type, as the UAS NF checks the stored UUAA context before forwarding the request from SMF to the USS/UTM. For downlink direction, SA2 understand it is stage 3 to decide whether UAS NF provides the Payload and the Payload Type which is transparent to the SMF and the SMF forwards them to the UE.

- If yes, how is this identification performed, and what are the impacted procedures?

Answer: See answer for question 1.

2. Actions:

To CT4

ACTION: 	SA2 kindly asks CT4 to take the above into account and update stage 3 specification accordingly if necessary.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #154		Nov 14 - 18, 2022				Toulouse, France