3GPP TSG-SA2 Meeting # 153E (e-meeting)	 S2-2208344r01

# Elbonia, Oct 10 - 17, 2022																		(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3557 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RFSP index in use at 5GS to EPS mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AMP |  |  |  |  |  | Date: |  |  | 2020-09-29 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the agreed WID “5G AM Policy” in S2-2207871/SP-220807, this CR implements the conclusion in TR 23.700-89 for 5GS to EPS mobility with N26: ### 8.1.2 For deployment with N26-based interworking The following conclusion are agreed: - When a UE moves from 5GC to EPC, the MME sets the "RFSP Index in use" value the same as the value in the UE context received from AMF via N26 if the UE context in AMF also contains a validity period. - When the validity period expires, the MME re-evaluates the RFSP Index value according to current specification (see clause 4.3.6 of TS 23.401 [5]). - The validity period, i.e. "RFSPinUseExpiryTime" is selected by PCF for a UE. When the PCF for a UE provides the Authorized RFSP Index indicating the UE to move to 4G, it may also provide the value of "RFSPinUseExpiryTime". If the AMF selects the RFSP Index in use identical to the authorized RFSP Index, it should store the received "RFSPinUseExpiryTime" in UE context in AMF and further also send the received "RFSPinUseExpiryTime" to MME in UE context along with the "RFSP Index in use" when N26 interface applied. - When mobility happens between MMEs before the "RFSPinUseExpiryTime" expires, the remaining value of the "RFSPinUseExpiryTime" should also be sent to the new MME. The new MME handles RFSP Index in the same way as for mobility from the AMF to the MME as described above. - When a UE is served by the EPC, there is no use case or requirement to provide updated RFSP Index from 5GC to the EPC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description based on the TR conclusion. Note that RFSPinUseExpiryTime is changed to RFSPinUseValidityTime. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Potential ping-pong issue (i.e. 5GS keeps sending the UE to EPS, and the EPS keeps sending the UE back to 5G immediately) and risk of UE not being able to return to 5GS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.2.2, 4.11.1.5.3, 4.11.1.5.5, 4.11.1.5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR3710, TS 23.503 CR0739 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 4.16.2.2	AM Policy Association Modification initiated by the PCF

The AM Policy Association modification procedure may be initiated by an internal PCF event or by PCF obtaining pertinent analytics information from an NWDAF.

The following procedure is applicable to AM Policy Association modification due to Case B.

Figure 4.16.2.2-1: AM Policy Association Modification initiated by the PCF

The procedure driven by a PCF internal event applies to both roaming and non-roaming scenarios and when driven by NWDAF, applies only to non-roaming scenarios.

An AM Policy Association is established, with the V-PCF in case of roaming or with the PCF in a non-roaming case as described in clause 4.16.1.2 before this procedure is triggered.

In the non-roaming case the role of the V-PCF is performed by the PCF. For the roaming scenarios, the V-PCF interacts with the AMF.

NOTE:	The V-PCF/PCF stores the access and mobility related policy information provided to the AMF.

1.	[Conditional] The PCF determines internally that the new status of the UE context requires new policies, potentially triggered by an AF as described in clause 4.15.6.9 or by a notification from the UDR. This may be triggered by obtaining pertinent analytics information from an NWDAF as described in clause 6.1.1.3 of TS 23.503 [20].

2.	The (V-)PCF in case of roaming and PCF in a non-roaming case makes a policy decision. The PCF may also decide to subscribe to a new Analytics ID from NWDAF as described in clause 6.1.1.3 of TS 23.503 [20].

3.	The (V-)PCF in the roaming case and the PCF in a non-roaming case sends Npcf_AMPolicyControl_UpdateNotify including AM Policy Association ID associated with the SUPI defined in TS 29.507 [32]. The policy update may include Service Area Restrictions, UE-AMBR, RFSP index value and optionally the validity time for the RFSP index value, access stratum time distribution indication and Uu time synchronization error budget. If an AF has previously subscribed to event request for allocation of service area coverage outcome in step 1, the (V-)PCF checks if the allocated service area coverage was changed and sends a respective notification to the AF using Npcf_AMPolicyAuthorization_Notify as defined in clause 6.3.1.18 in TS 23.503 [20].

NOTE x: The validity time indicates the time by which the RFSP Index in use will be used in the MME after 5GS to EPS mobility as described in clause 5.17.2.2 of TS 23.501 [2].

4.	The AMF deploys and stores the updated access and mobility related policy information, which includes storing the Service Area Restrictions and Policy Control Request Trigger of AM Policy Association, provisioning of the Service Area Restrictions to the UE, provisioning the RFSP index, UE-AMBR, Service Area Restrictions to the NG-RAN, optionally the access stratum time distribution indication and Uu time synchronization error budget to the NG-RAN and request for notification of SM Policy association establishment and termination to a list of (DNN, S-NSSAI)(s) together with PCF for the UE binding information.

* * * Next Changes * * *

##### 4.11.1.5.3	Tracking Area Update

The following changes are applied to clause 5.3.3.1 (Tracking area update procedure with Serving GW change) in TS 23.401 [13]:

-	Step 2: The UE shall in Access Stratum signalling include GUMMEI that is mapped from 5G-GUTI following the mapping rules specified in TS 23.501 [2] and the UE indicates it as a native GUMMEI and should in addition indicate it as "Mapped from 5G-GUTI". The UE shall, in the TAU request message, include EPS GUTI that is mapped from 5G-GUTI following the mapping rules specified in TS 23.501 [2]. The UE indicates that it is moving from 5GC. The UE integrity protects the TAU request message using the 5G security context. If the UE supports any of the CIoT 5GS Optimisations included in 5GC Preferred Network Behaviour, then the UE shall include its 5GC Preferred Network Behaviour if it included its EPC Preferred Network Behaviour in the TAU request.

MME may steer the UE from EPC by rejecting the TAU request with an appropriate cause value. The MME should take into account availability of 5GC to the UE and the Preferred and Supported Network Behaviour (see clause 5.31.2 of TS 23.501 [2]) before steering the UE from EPC.

-	Step 5 and message Context Response may include new information Return preferred.

Return preferred is an indication by the AMF of a preferred return of the UE to the last used 5GS PLMN at a later access change to a 5GS shared network.

RFSP in Use Validity Time is provided by the AMF to the MME if the AMF selects the RFSP Index in use identical to the authorized RFSP Index as specified in clause 5.4.3.4 of TS 23.501 [2] and validity time is received from PCF as specified in clause 4.16.2.2 and in clause 6.1.2.1 of TS 23.503 [20]. The MME handles RFSP Index as specified in clause 4.11.1.5.x.

The MME may store the last used 5GS PLMN ID in UE's MM Context.

The MME may provide E-UTRAN with a Handover Restriction List taking into account the last used 5GS PLMN ID and the Return Preferred indication. The Handover Restriction List contains a list of PLMN IDs as specified by TS 23.251 [35].

-	Step 9a IP-CAN Session Modification procedure:

It is replaced by SM Policy Association Modification as specified in clause 4.16.5.

-	Step 13 and HSS use of Cancel Location

The HSS/UDM de-registers any old AMF node by sending an Nudm_UECM_DeregistrationNotification service operation to the registered AMF for 3GPP access. The registered AMF for 3GPP access initiates AM Policy Association Termination procedure as defined in clause 4.16.3.2 and UE Policy Association Termination procedure as defined in clause 4.16.13.1.

-	Step 17: If the DNN and SMF+PGW-C FQDN for S5/S8 interface association exist, the HSS/UDM sends APN mapped form DNN and SMF+PGW-C FQDN for S5/S8 to UE.

-	Step 20 and MME processing of the partial Tracking Area Update (TAU) procedure.

The MME may use an indication Return preferred from Context Response at step 6 when deciding the PLMN list content.

The MME may provide the eNodeB with a PLMN list. The Handover Restriction List contains a list of PLMN IDs as specified by TS 23.501 [2].

* * * Next Changes * * *

##### 4.11.1.5.5	5GS to EPS handover using N26 interface

In step 3 of clause 4.11.1.2.1, the Forward Relocation Request may include new information Return Preferred.

Return Preferred is an indication by the AMF of a preferred return of the UE to the last used 5GS PLMN at a later access change to a 5GS shared network.

RFSP in Use ValidityTime is provided by the AMF to the MME if the AMF selects the RFSP Index in use identical to the authorized RFSP Index as specified in clause 5.4.3.4 of TS 23.501 [2] and validity time is received from PCF as specified in clause 4.16.2.2 and in clause 6.1.2.1 of TS 23.503 [20]. The MME handles RFSP Index as specified in clause 4.11.1.5.x.

The MME may store the last used 5GS PLMN ID in UE's MM Context.

The MME may provide E-UTRAN with a Handover Restriction List taking into account the last used 5GS PLMN ID and the Return Preferred indication. The Handover Restriction List contains a list of PLMN IDs as specified by TS 23.251 [35].

* * * Next Changes * * *

##### 4.11.1.5.X	Radio Resource Management functions

The following changes are applied to clause 4.3.6 (Radio Resource Management functions) in TS 23.401 [13]:

- At 5GS to EPS mobility or during inter-MME mobility, if RFSP in Use ValidityTime is received from the AMF or the old MME, the new MME uses the received RFSP Index in use by the time indicated in RFSP in Use ValidityTime. When RFSP in Use ValidityTime expires, the MME re-evaluates the RFSP Index in use as in clause 4.3.6 of TS 23.401 [13].

NOTE: RFSP in Use ValidityTime is the validity time that is sent by the AMF to the MME as specified in clause 5.17.2.2 of TS 23.501 [2].

* * End of Changes * * *