3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208411

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, Ericsson

Title:	Evaluation and conclusion for KI#5

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: This contribution proposes the evaluation and conclusion of KI#5 Improved support of RAs including TAs supporting Rejected S-NSSAIs.

# 1. Introduction/Discussion

KI#5 studies whether and how to allow the UE to initiate a registration for an S-NSSAI which was rejected for the RA when the UE enters a TA that is part of the RA and the TA supports this S-NSSAI.

Currently, 9 solutions are proposed for this key issue mainly in the following two categories:

- Based on allowed S-NSSAI, where AMF provides conditional allowed S-NSSAI with its support TA to UE (solution 11, 23, 25, 26)

- Based on rejected S-NSSAI, where AMF provides rejected S-NSSAI with its reject/support TA or new cause to UE (solution 25, 27, 28, 29, 30, 31)

|  | Category 1: | Category 2: |
| --- | --- | --- |
| Pros | - Does not require UE to perform registration update for using a requested S-NSSAI not supported in the previous TA - PDU sessions need not be re-established and disconnected (see solution 29) | - Reuse most of the current procedures. (only Registration procedure is impacted) - Aligned with the current slice management features, where the network determines and updates UE on the Allowed NSSAI, rejected S-NSSAIs, target NSSAI and so on. |
| Cons | - Features related to slice management such as NSAC will be impacted. (e.g. Cannot be used together) - Procedures related to PDU Session management will be impacted and the details are not fully discussed. - Have impact on Homogenous slice support in RA. AMF will allow the UE to access an S-NSSAI which is only supported in part of the TAs. May cause non-backward compatibility. - The AMF in connected mode can detect when the connectivity of a slice is available or not, but in idle mode the MT services cannot assume where in the RA the UE is so the AMF would page anyhow. In MO case the UE can avoid causing MO access attempts for the slices that are not working in current TA. - There can be RAN impact (e.g. Solution #29) to treat the PDU Sessions. | - Require UE to perform registration update for using a requested S-NSSAI not supported in the previous TA. - Requires to release and re-establish sessions at mobility in an out the areas where the slice is supported. - When entering the area of support of S-NSSAI, the RA size may need to be reduced. |

Therefore, it is proposed to take Category 2 solution as the baseline for KI#5.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-41.

# * * * * First change (all new) * * * *

# 7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

## 7.5	Evaluation for KI#5

The related solutions are #11, 23, 25, 26, 27, 28, 29, 30, 31.

Category 1 is based on allowed S-NSSAI, where AMF provides conditional allowed S-NSSAI with its support TA to UE (solution 11, 23, 25, 26, 29).

Category 2 is based on rejected S-NSSAI, where AMF provides rejected S-NSSAI with its reject/support TA or new cause to UE (solution 25, 27, 28, 29, 30, 31)

Table 7.X-1: Evaluation of KI#5 related principles

|  | Category 1: | Category 2: |
| --- | --- | --- |
| Pros | - Does not require UE to perform registration update for using a requested S-NSSAI not supported in the previous TA - PDU sessions need not be re-established and disconnected (see solution 29) | - Reuse most of the current procedures. (only Registration procedure is impacted) - Aligned with the current slice management features, where the network determines and updates UE on the Allowed NSSAI, rejected S-NSSAIs, target NSSAI and so on. |
| Cons | - Features related to slice management such as NSAC will be impacted. (e.g. Cannot be used together) - Procedures related to PDU Session management will be impacted and the details are not fully discussed. - Have impact on Homogenous slice support in RA. AMF will allow the UE to access an S-NSSAI which is only supported in part of the TAs. May cause non-backward compatibility. - The AMF in connected mode can detect when the connectivity of a slice is available or not, but in idle mode the MT services cannot assume where in the RA the UE is so the AMF would page anyhow. In MO case the UE can avoid causing MO access attempts for the slices that are not working in current TA. - There can be RAN impact (e.g. Solution #29) to treat the PDU Sessions. | - Require UE to perform registration update for using a requested S-NSSAI not supported in the previous TA. - Requires to release and re-establish sessions at mobility in an out the areas where the slice is supported. - When entering the area of support of S-NSSAI, the RA size may need to be reduced. |

# * * * * Second change (all new) * * * *

# 8	Conclusions

Editor's note: This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.X	Conclusions for Key Issue #5

The following principle is proposed to be the conclusion for normative work:

1.  AMF provides information related to the supported or not supported TA(s) regarding a rejected S-NSSAI in the RA to UE in the registration procedure and UE configuration procedure if the UE indicates it supports this feature . The UE shall be able to request a rejected S-NSSAI in a supported TA based on the supported/not supported TA information. A UE receiving this information shall trigger registration update when it requests a rejected S-NSSAI in a supported TA.

2.	When a UE provides one or more S-NSSAIs in a Requested NSSAI that is not available in a subset of the TAs of the RA, the AMF may register the S-NSSAIs that are available in a subset of the TAs of the RA, or the AMF may decide to not register the S-NSSAI e.g. due to S-NSSAI is subject for NSAC and the AMF is configured to limit to only register such S-NSSAIs when the UE is in a TA supporting the S-NSSAI, see conclusion for partially rejected S-NSSAIs.

2.	The AMF informs the UE which S-NSSAIs that have been registered and provides the UE with a list of TAs where the S-NSSAIs are available. The list of TAs can include or exclude the UE’s current TA. The AMF provides the S-NSSAIs that are registered in a subset of the RA to the NG-RAN e.g. in the Allowed NSSAI.

a.	The registered TA-specific network slices are treated as part of an extension to the Allowed NSSAI and the UE may request services from the network slices without the need of an MRU.

NOTE:	It is to be determined with RAN3 whether Allowed NSSAI or a new IE is to be used for the S-NSSAIs that are registered in a subset of the RA. The NG-RAN uses the NSSAI as for Allowed NSSAi i.e. for RRM purposes.

3.	The UE is not allowed to initiate Service Request, or any connection requests for User Plane data, or SM signalling (except for PS Data Off status change reporting) for an S-NSSAI when the UE is in a TA where the S-NSSAI is defined to not be available. The AMF/SMF can reject UE request if the AMF/SMF determines that the UE does not follow such logic.

4.	The IE names to be used is to be determined during the normative phase.

# * * * * End of changes * * * *